3GPP TSG-WG SA2 Meeting #154-AH e-meeting  	S2-2301776

Elbonia, January 16th – 20th, 2023	(revision of S2-2300555r04)

Source:	Huawei, HiSilicon

Title:	KI#6: Update of Solution 9

Document for:	Approval

Agenda Item:	9.5.1

Work Item / Release:	FS_Ranging_SL / Rel-18

Abstract: To have a clear conclusion, this contribution updates Solution 9.

1. Introduction

To minimize the impact to the LCS mechanism and also align with MT case (i.e. AMF acts as the GMLC consumer to get the Ranging/SL positioning result), this paper proposes to remove Option A from Sol#9.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-86.

# * * * * First change * * * *

6.9	Solution #9: Solution for KI#6 on Ranging/SL positioning service exposure to UE

6.9.1	General

The solutions address the ranging and sidelink positioning service exposure to UE for Key Issue #6.

The followings give two potential cases and the corresponding solutions:

Case 1:	Two UEs, the UE involved in Ranging/SL positioning consumes the Ranging/SL positioning service.

Case 2:	Three UEs, the SL Positioning Client UE consumes the Ranging/SL positioning service.

6.9.2	Functional descriptions

Ranging/SL positioning refers to the determination of the distance between two UEs or more UEs and/or the direction and/or relative positioning of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 interface.

The consumer can be the UE involved in the Ranging/SL positioning service, or a SL Positioning Client UE. The UE/SL Positioning Client UE may perform the Ranging/SL positioning operation directly via PC5 interface and use the result locally, without involvement of 5GC, or the SL Positioning Client UE (e.g. out of SL coverage of either UE subject to Ranging/SL positioning) requests Ranging/Sidelink positioning service with the involvement of 5GC (e.g. AMF, LMF, etc.).

6.9.3	Procedures

6.9.3.1	UE involved in SL ranging consumes the Ranging/SL positioning service

Figure 6.9.3.1-1: UE involved in SL ranging consumes the Ranging/SL positioning service

In this case, two UEs are involved in the Ranging/SL positioning operation. The UE triggers ranging service, performs the Ranging/SL positioning operation, and makes the calculation locally, i.e. Ranging/SL positioning service is exposed to one of the two UEs.

Figure 6.9.3.1-2: Procedure of ranging UE consumes the Ranging/SL positioning service

1.	The application layer of the UE1 triggers the request for Ranging/SL positioning.

2.	The UE1 initiates the Ranging/SL positioning operation with the UE2. The NG-RAN node is involved if UE is in network coverage and the scheduled resource allocation is used.

NOTE:	The role of Target UE and Reference UE is negotiated between UE1 and UE2 over PC5 interface in this step which can refer to the Solutions of KI#3&4.

NOTE:	The security and privacy consideration for the Ranging/SL positioning operation should be studied by SA WG3.

3.	The UE1 calculates the result and uses it locally.

6.9.3.2	SL Positioning Client UE consumes the Ranging/SL positioning service

6.9.3.2.1	General

Figure 6.9.3.2.1-1: SL Positioning Client UE consumes the Ranging/SL positioning service

In this case, three UEs are involved in the Ranging/SL positioning, in which a SL Positioning Client UE asks for the Ranging/sSL positioning for the other two UEs (i.e. UE1 and UE2). The SL Positioning Client UE may request via Uu interface to perform Ranging/SL positioning operation between UE1 and UE2 with the involvement of 5GC. If the SL Positioning Client UE is in the sidelink coverage with one of the requested UEs (e.g. UE1), the SL Positioning Client UE may alternatively request UE1 via PC5 interface to perform Ranging/SL positioning with UE2.

When 5GC is involved, there are network based Ranging/Sidelink positioning in which the Ranging/Sidelink positioning estimation result calculation is performed at LMF and UE based Ranging/Sidelink positioning in which the Ranging/Sidelink positioning estimation result calculation is performed at the UE.

6.9.3.2.2	Service exposure to the SL Positioning Client UE via PC5

Figure 6.9.3.2.2-1: Service exposure to the SL Positioning Client UE via PC5

1.	The application layer of the SL Positioning Client UE triggers the request for the Ranging/SL positioning, and the SL Positioning Client UE requests the UE1 to perform the ranging/sidelink positioning operation with UE2.

2.	The UE1 initiates the Ranging/SL positioning operation with the UE2. The NG-RAN node is involved if UE is in network coverage and the scheduled resource allocation is used.

NOTE:	The role of Target UE and Reference UE is negotiated between UE1 and UE2 over PC5 interface in this step which can refer to the Solutions of KI#3&4.

NOTE:	The security and privacy consideration for the Ranging/SL positioning operation should be studied by SA WG3.

3.	The UE1 makes the calculation locally.

4.	The UE2 returns the Ranging/SL positioning result to the SL Positioning Client UE via the PC5 interface.

6.9.3.2.3	Service exposure to the SL Positioning Client UE via 5GC/Uu (network based)

3rd party UE

NG-RAN

AMF3

LMF3

LMF2

2. UL NAS TRANSPORT (MO-LR Request (SL ranging requirement, UE1 ID, UE2 ID, initiator UE ID))

GMLC

UE1

UE2

10. DL NAS TRANSPORT (result)

1. UE Triggered Service Request

AMF2

UDM

NRF

3. Ngmlc_Location_ProvideLocation_Request (SL ranging requirement, UE1 ID, UE2 ID, initiator UE ID, 3rd UE ID)

4. Namf_Location_ProvidePositioningInfo request()

5. Nlmf_Location_DetermineLocation request()

7. Nlmf_Location_DetermineLocation response(result)

6. UE SL Ranging/Positioning

8. Namf_Location_ProvidePositioningInfo response(result)

9. Ngmlc_Location_ProvideLocation_Response (result)

Figure 6.9.3.2.3-1: Service exposure to the SL Positioning Client UE via 5GC/Uu (network based)

1.	The application layer of the SL Positioning Client UE triggers the Ranging/SL positioning, and then requests the network to perform the Ranging/SL positioning between the UE1 and UE2. If the SL Positioning Client UE in CM-IDLE state, it firstly triggers the service request procedure.

2.	The SL Positioning Client UE requests the network to perform the Ranging/SL positioning operation between the UE1 and UE2. The Ranging/SL positioning requirement may include the requirement regarding Ranging/SL positioning demand (i.e. the distance and/or the direction between two UEs or more UEs), delay, accuracy, result feedback mode (e.g. immediately, periodically or at a later time). The SL Positioning Client UE includes the Ranging/SL positioning requirement in the MO-LR Request toward the AMF3, then the AMF3 gets the Ranging/SL positioning requirement and sends it to GMLC. The SL Positioning Client UE may suggest the initiator UE (i.e. which is used to perform signalling exchange with the LMF) based on implementation and indicate which UE is the initiator UE to the AMF.

3.	The AMF may select a GMLC based on NRF query or configuration in AMF, and then acts as an LCS client to request the GMLC for Ranging/SL positioning result, the AMF further includes the SL Positioning Client UE ID in the request.

4. 	The GMLC determines the initiator UE (e.g. UE2) from UE1 and UE2 and sends the Namf_Location_ProvidePositioningInfo request to the UE2’s AMF2. GMLC can performs the authorization on the Ranging/SL positioning operation request from SL Positioning Client UE.

NOTE:	The security and privacy consideration for the Ranging/SL positioning operation should be studied by SA WG3.

5. The AMF2 selects the LMF (e.g. LMF2) and sends the Nlmf_Location_DetermineLocation request (UE1 ID, UE2 ID) to the LMF2 for the Ranging/SL positioning result between the UE1 and UE2.

6. The LMF2 triggers the UE2 to perform Ranging/SL positioning with the UE1 via AMF2. The measurement request message may include the Ranging/SL positioning demand, UE1 ID, UE2 ID, etc. The UE2 performs the Ranging/SL positioning operation with the UE1, and reports the Ranging/SL positioning measurements to the LMF for calculation.

7. The LMF2 makes calculation based on the Ranging/SL positioning measurements and returns the Ranging/SL positioning result to the AMF2.

8. The AMF2 returns the Ranging/SL positioning result to the GMLC.

9. Then the GMLC returns the result to the AMF3.

10.	The AMF3 returns the result to the SL Positioning Client UE.

6.9.4	Impacts on services, entities, and interfaces

UE:

-	sends Ranging/SL positioning requirement to another UE for Ranging/SL positioning measurement or to the LMF for Ranging/SL positioning result.

-	indicate which UE is the initiator UE to the LMF.

-	receives Ranging/SL positioning measurement from another UE or Ranging/SL positioning result from the LMF (via the AMF).

-	receives Ranging/SL positioning measurement request from the LMF, performs Ranging/SL positioning measurement, and reports the Ranging/SL positioning measurement to the LMF.

NG-RAN:

-	allocates PC5 radio resource to UE for Ranging/SL positioning operation.

AMF:

-	acts as the consumer of GMLC service.

-	selects LMF and routes the Ranging/SL positioning requirement to the LMF.

-	routes the signalling (e.g. LPP messages for SL ranging) between the UE and the LMF.

-	Support check the SL Positioning Client UE is authorized to request the Ranging/SL positioning information between the UE1 and UE2.

LMF:

-	requests and controls the UE to perform Ranging/SL positioning measurement.

-	receives the Ranging/SL positioning measurement from the UE and makes calculation.

-	returns the Ranging/SL positioning result to the AMF.

# * * * * End of changes * * * *