3GPP TSG-WG SA2 Meeting #154AH-E	S2-2301032

# Elbonia, January 16 – 20, 2023		(revision of S2-2210509)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0073 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS Re-discovery Procedure with EASDF in HR roaming scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is an Editor’s NOTE added in SA#154 meeting as below: “Editor’s Note: It is FFS how EAS (re)-discovery procedure for HR-SBO roaming scenario is perfomed.” This contribution proposes to add a new clause to describe EAS re-discovery procedure in TS 23.548. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause “6.7.3 EAS Re-discovery Procedure” and related sub-clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EAS Re-discovery Procedure for HR-SBO scenario is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.X(new), 6.7.X.1(new), 6.7.X.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR3619 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.7.X	EAS Re-discovery Procedure

#### 6.7.X.1	General

The EAS re-discovery in VPLMN can be triggered due to UE mobility

Editor's note:	AF triggered EAS Re-discovery is FFS.

#### 6.7.X.2	EAS Re-discovery

Figure 6.7.X.2-1 shows the procedure of EAS re-discovery in VPLMN for HR roaming.

Figure 6.7.X.2-1: EAS rediscovery procedure

1.	For UE mobility triggered EAS rediscovery, step 4 is skipped.

The procedure described in clause 4.23.7 (N2 based handover) or 4.23.11 (Xn based handover) of TS 23.502 [3] is performed by replacing I-SMF with V-SMF and SMF with H-SMF.

1a1.	(V-SMF insertion) The (target) V-SMF retrieves SM context from H-SMF.

1a2.	(V-SMF change) The target V-SMF retrieves SM context from the source V-SMF.

1b.	H-SMF (in case of V-SMF insertion) or source V-SMF (in case of V-SMF change) to (target) V-SMF: Nsmf_PDUSession_Context Response including SM context. The SM context includes Authorization Indication for HR-SBO and VPLMN Specific Offloading Policy.

The SM context also includes impact field corresponding to the old target DNAI if it has been inserted by the H-SMF or the source V-SMF.

In the case of V-SMF insertion or V-SMF change, when the AMF sends Nsmf_PDUSession_CreateSMContext Request to the target V-SMF, the AMF includes HR-SBO allowed indication if received from the UDM.

Step 1 is skipped if UE moves without V-SMF change.

Steps 1-6 are skipped if UE moves with V-SMF removal.

After the handover procedure, the (target) V-SMF may perform UL CL/BP and local PSA insertion/change/removal as described in clause 4.23.9 of TS 23.502 [3] by replacing I-SMF with V-SMF and SMF and H-SMF. Differences are shown as steps 2 to 5:OTE 1: When there are other V-UPF(s) between UL CL/BP and H-UPF, then the (target) V-SMF performs UL CL/BP and local PSA insertion/change/removal as described in clause 4.3.5.4 of TS 23.502 [3] by replacing SMF with V-SMF and PSA1 with V-UPF.

2.	The (target) V-SMF may (re-)select a new V-EASDF, e.g. based on the DNAI corresponding to the target UE location. The (target) V-SMF may invoke Neasdf_DNSContext_Create service to obtain the new V-EASDF address or the (target) V-SMF obtains the new V-EASDF address based on local configuration.

Editor’s Note: If a new V-EASDF is selected, the DNS context in the old V-EASDF is removed by the (source/target) V-SMF using Neasdf_DNSContext_Delete service.  it is FFS how the old VSMF knows  the old V-EASDF is  should be removed

3.	The (target) V-SMF invokes Nsmf_PDUSession_Update Request (EAS rediscovery indication, impact field if available, the new V-EASDF address if available) to the H-SMF.

For the case of local PSA removal, the request message is sent to the H-SMF without including any DNAI.

Editor’s Note: as The impact field is used to identify which EAS(s) information need to be refreshed, the following is FFS The impact field corresponds to:

-	the new target DNAI. For the case of local PSA insertion, impact field corresponding to the new target DNAI is sent from the V-SMF to the H-SMF.

-	the new target DNAI and old target DNAI. For the case of local PSA change, the (target) V-SMF determines the impact field based on the new target DNAI and the impact field corresponds to the old target DNAI received in step 1.

-	the old target DNAI. For the case of local PSA removal, only impact field corresponding to the old target DNAI received in step1 is sent from the V-SMF to the H-SMF.

4.	For AF deployed in VPLMN triggered EAS rediscovery, the AF may indicate the EAS rediscovery for the impacted applications, which are identified by Application Identifier(s), to the H-SMF via the H-PCF using the AF influence on traffic routing procedure as described in clause 4.3.6 of TS 23.502 [3].

Editor’s Note: bullet 4 is FFS as it is undecided how AF in VPLMN provides information to VSMF

5.	(V-SMF insertion of change case) The H-SMF initiates Nsmf_PDUSession_Update Request towards the (target) V-SMF including the PCO.

The PCO includes EAS rediscovery indication, optionally impact field and optionally V-EASDF address.

6.	(V-SMF insertion of change case) The V-SMF initiates PDU Session Modification command including the PCO to the UE.

7.	(V-SMF removal case) The H-SMF selects an EASDF in HPLMN and creates a contextfor the PDU session  in that EASDF and then initiates PDU Session Modification command including PCO to the UE.

The PCO includes EAS rediscovery indication and optionally the impact field. If the H-SMF selects an EASDF in HPLMN, the PCO also includes the new EASDF address.

# * * * * End of changes * * * *