3GPP TSG-SA2 Meeting #154-AH-e	S2-2300210

# Online, , 16th Jan 2023 - 20th Jan 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0611 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Key Issue #3: Data and analytics exchange in roaming case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Key issue #3 conclusions in TR 23.700-81 are the following: For KI#3, the following architectural principles are agreed as conclusion: - An NWDAF is used as entry point to exchange analytics in roaming scenario between HPLMN and VPLMN. It authorizes the request according to operator policies and user consent and it filters the information exposed in reply to the request. NOTE 1: How to discover the NWDAF for the roaming case is determined in normative phase. - The NWDAF offers new services to be accessed by the peer PLMN. This allows the NRF to be configured in such a manner that only access to those services is authorized from outside the PLMN. The new services will be defined with as much communality as possible to existing services to ease the implementation. - Exposure of input data for analytics is allowed from VPLMN to HPLMN and vice versa. It may be restricted based on operator policy and user consent. - The HPLMN may provide analytics to the VPLMN. The VPLMN may provide analytics to the HPLMN. NOTE 2: Analytics that rely on input data from the HPLMN are preferbly not provided from VPLMN to HPLMN but generated in the HPLMN. Analytics that rely on input data from the VPLMN are preferbly not provided from HPLMN to VPLMN but generated in the VPLMN. NOTE 3: Data and analytics exchange between HPLMN and VPLMN may be furter updated based on feedback from GSMA during the normative work. NOTE 4: The user consent check procedure and security aspects will align with SA WG3's conclusion during normative work. The following use cases are supported: - VPLMN may consume analytics generated by HPLMN or request input data and generate analytics on its own: NOTE 5: It will be determined during the normative work whether an analytics profile is required in addition to analytics. - In home routed roaming scenarios, HPLMN analytics (i.e. slice load level analytics, NF load analytics, etc.) can be leveraged by the AMF in the VPLMN for Network Slice selection and SMF selection for PDU Session management. - UE-related analytics provided by the HPLMN (e.g.. service experience analytics, etc.) can include statistics or predictions for outbound roaming UEs. - HPLMN may consume analytics generated by the VPLMN or request input data and generate analytics on its own. - In home routed roaming scenarios, analytics information with statistics or predictions for outbound roaming UEs can be leveraged by the H-PCF for QoS control of the PDU Session. - Analytics (i.e. service experience analytics, slice load level analytics, etc.) can be leveraged by the H-PCF for decision of NSSP in URSP rules provisioned to the UE roaming in the VPLMN. According to Note 5, the need for an analytics profile will be discussed in the normative Phase. An analytics profile is meant to contain basic information about inbound roamers determined from analytics which is deemed usefull in a VPLMN. In SA4#154 discussions, several operators voiced concerns that extensive exchange of analytics or input data over roaming interface may be unrealsistic. The analytics profile aims to address usch concerns as it is meant as a simplified mechanism to provide the most usefull information about inbound roaming users. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Procedures for data and analytics exchange in roaming case are provided based on key issue #3 conclusions in TR 23.700-81 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures for data and analytics exchange in roaming case are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, 5.2, 6.1.1.1, 6.1.1.2, new 6.1.1.A, new 6.1.1.B, 6.1.2.1, 6.1.2.2, new 6.1.2.X, new 6.1.2.Y, new 6.1.2.Z, new 6.2.A, new 6.2.B, 7.1, new 7.X, new 7.Y, new 7.Z, new 7.A, new 7.B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 4.3	Roaming architecture

The NWDAF of the H-PLMN may retrieve analytics from the NWDAF of the V-PLMN using the architecture shown in Figure 4.3-1.

Figure 4.3-1: Roaming Architecture to expose Network Data Analytics from V-PLMN to H-PLMN

The NWDAF of the V-PLMN may retrieve analytics or analytics profiles from the NWDAF of the H-PLMN using the architecture shown in Figure 4.3-2.

Figure 4.3-2: Roaming Architecture to expose Network Data Analytics or Analytics Profiles from H-PLMN to V-PLMN

The NWDAF of the H-PLMN may retrieve input data for analytics from the NWDAF of the V-PLMN using the architecture shown in Figure 4.3-3.

Figure 4.3-3: Roaming Architecture to expose Input data for analytics from V-PLMN to H-PLMN

The NWDAF of the V-PLMN may retrieve input data for analytics from the NWDAF of the H-PLMN using the architecture shown in Figure 4.3-4.

Figure 4.3-4: Roaming Architecture to expose Input data for analytics from H-PLMN to V-PLMN

2nd change

## 5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types, the consumer may query NRF providing the NF Set IDs or NF types in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	an NWDAF containing MTLF shall include the ML model provisioning services (i.e. Nnwdaf_MLModelProvision, Nnwdaf_MLModelInfo) as one of the supported services during the registration in NRF when trained ML models are available for one or more Analytics ID(s). The NWDAF containing MTLF may provide to the NRF a (list of) Analytics ID(s) corresponding to the trained ML models and possibly the ML Model Filter Information for the trained ML model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. For each Analytics ID, the NWDAF containing MTLF may also include, in the registration to the NRF, an ML Model Interoperability indicator.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML models requested by the NWDAFs from the vendors in the list.

Editor's note:	It is FFS if the ML Model Interoperability indicator is per Analytics ID.

NOTE 4:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (i.e. an NWDAF containing AnLF) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required and NF consumer information. The NRF returns one or more candidate for instances of NWDAF containing MTLF to the NF consumer and each candidate for instance of NWDAF containing MTLF includes the Analytics ID(s) and possibly the ML Model Filter Information for the available trained ML models, if available.

NOTE 5:	NF consumer information such as Vendor ID is defined in stage 3.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the ingress NWDAF to request analytics or input data is discovered via the NRF.

3rd change

#### 6.1.1.1	Analytics subscribe/unsubscribe by NWDAF service consumer

This procedure is used by any NWDAF service consumer (e.g. including NFs/OAM) to subscribe/unsubscribe at NWDAF to be notified on analytics information, using Nnwdaf_AnalyticsSubscription service defined in clause 7.2. This service is also used by an NWDAF service consumer to modify existing analytics subscription(s). Any entity can consume this service as defined in clause 7.2.

Figure 6.1.1.1-1: Network data analytics Subscribe/unsubscribe

1.	The NWDAF service consumer subscribes to or cancels subscription to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe/ Nnwdaf_AnalyticsSubscription_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.1.3.

When a subscription to analytics information is received, the NWDAF determines whether triggering new data collection is needed.

If the service invocation is for a subscription modification, the NF service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_AnalyticsSubscription_Subscribe.

If the subsctiption relates to outbound roaming users, the NWDAF in the H-PLMN may decide to retrieve, or to subscribe to input data or analytics from the related V-PLMN

If the subsctiption relates to inbound roaming users, the NWDAF in the V-PLMN may decide to retrieve, or to subscribe to input data, analytics, or an analytics profile, from the related H-PLMN

2.	If NWDAF service consumer is subscribed to analytics information, the NWDAF notifies the NWDAF service consumer with the analytics information by invoking Nnwdaf_AnalyticsSubscription_Notify service operation, based on the request from the NWDAF service consumer, e.g. Analytics Reporting Parameters. If the NWDAF provides a Termination Request, then the consumer cancels subscription to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Unsubscribe service operation.

4th change

#### 6.1.1.2	Analytics subscribe/unsubscribe by AFs via NEF

The analytics exposure to AFs may be performed via NEF by using analytics subscription to NWDAF.

Figure 6.1.1.2-1 illustrates the interaction between AF and NWDAF performed via the NEF.

Figure 6.1.1.2-1: Procedure for analytics subscribe/unsubscribe by AFs via NEF

0.	NEF controls the analytics exposure mapping among the AF identifier with allowed Analytics ID and associated inbound restrictions (i.e, applied to subscription of the Analytics ID for an AF) and/or outbound restrictions (i.e. applied to notification of Analytics ID to an AF).

In this Release, AF is configured with the appropriated NEF to subscribe to analytics information, the allowed Analytics ID(s) and with allowed inbound restrictions (i.e. parameters and/or parameter values) for subscription to each Analytics ID.

1.	The AF subscribes to or cancels subscription to analytics information via NEF by invoking the Nnef_AnalyticsExposure_Subscribe/ Nnef_AnalyticsExposure_Unsubscribe service operation defined in TS 23.502 [3]. If the AF wants to modify an existing analytics subscription at NEF, it includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnef_AnalyticsExposure_Subscribe. If the analytics information subscription is authorized by the NEF, the NEF proceeds with the steps below.

2.	Based on the request from the AF, the NEF subscribes to or cancels subscription to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe/ Nnwdaf_AnalyticsSubscription_Unsubscribe service operation.

If the parameters and/or parameters values of the AF request comply with the inbound restriction in the analytics exposure mapping, NEF forwards in the subscription to NWDAF service the Analytics ID, parameters and/or parameters values from the AF request.

If the request from AF does not comply with the restrictions in the analytics exposure mapping, NEF may apply restrictions to the subscription request to NWDAF (e.g. restrictions to parameters or parameter values of the Nnwdaf_AnalyticsSubscription_Subscribe service operations), based on operator configuration and/or may apply parameter mapping (e.g. geo coordinate mapping to TA(s)/Cell-id(s)).

The NEF records the association of the analytics request from the AF and the analytics request sent to the NWDAF.

The NEF selects an NWDAF that supports analytics information requested by the AF using the NWDAF discovery procedure defined in TS 23.501 [2].

If the subsctiption relates to outbound roaming users, the NWDAF in the H-PLMN may decide to retrieve, or to subscribe to, input data or analytics from the related V-PLMN

If the subsctiption relates to inbound roaming users, the NWDAF in the V-PLMN may decide to retrieve, or to subscribe to, input data, analytics, or an analytics profile, from the related H-PLMN

If the AF request is for a modification of the existing analytics subscription(s), the NEF invokes Nnwdaf_AnalyticsSubscription_Subscribe to modify the analytics subscription identified by an identifier (Subscription Correlation ID) associated with the AF.

3.	If the NEF has subscribed to analytics information, the NWDAF notifies the NEF with the analytics information or Termination Request by invoking Nnwdaf_AnalyticsSubscription_Notify service operation.

4.	If the NEF receives the notification from the NWDAF, the NEF notifies the AF with the analytics information or Termination Request by invoking Nnef_AnalyticsExposure_Notify service operation defined in TS 23.502 [3]. NEF may apply outbound restrictions to the notifications to AFs (e.g. restrictions to parameters or parameter values of the Nnef_AnalyticsExposure_Notify service operation) based on analytics exposure mapping and may apply parameter mapping for external usage (e.g. TA(s), Cell-id(s) to geo coordinate). The AF checks if a Termination Request is present in the Nnef_AnalyticsExposure_Notify as defined in step 2 in clause 6.1.1.1.

5th change

#### 6.1.1.A	Analytics subscription by H-NWDAF from V-NWDAF for inbound roaming users

This procedure may be used by the NWDAF in the H-PLMN as service consumer to subscribe/unsubscibe to notifications about analytics from the related V-PLMN for inbound roaming users (from the VPMN perspective)

Figure 6.1.1.A-1: Analytics subscription by H-NWDAF from V-NWDAF for inbound roaming users

1.	For subscription, the H-NWDAF checks the user consent of related users.

Editor´s note: User consent check needs to be alligned with SA3.

2.	The H-NWDAF of HPLMN discovers V-NWDAF of VPLMN that supports the Nwdaf_InboundRoamerAnalytics service using the NRF.

NOTE:	The access to the Nwdaf_AnalyticsSubscription service is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

3.	The H-NWDAF subsribes/unsubscribes to notifications about analytics information by invoking the Nnwdaf_InboundRoamerAnalytics_Subscribe/ Nnwdaf_InboundRoamerAnalytics_Unsubscribe service operation.

Editor´s note: Parameters for the Nnwdaf_InboundRoamerAnalytics Subscribe service operation are ffs.

4.	V-NWDAF checks if the HPLMN is authorised to request the analytics based on VPLMN operator polices (that may depend on the HPLMN and may indicate permissible or restricted analytics and related parameters).

5.	The V-NWDAF triggers new data collection if needed and monitores the requested analytics.

6.	The V-NWDAF may restrict the exposed analytics information based on VPLMN operator polices (that may depend on the HPLMN)

7.	The V-NWDAF notifies analytics information to the H-NWDAF.

6th change

#### 6.1.1.B	Analytics subscription by V-NWDAF from H-NWDAF for outbound roaming users.

This procedure may be used by an NWDAF in the V-PLMN to subscribe to notifications about analytics from the related H-PLMN for outbound roaming users (from the HPLMN perspective)

Figure 6.1.1.B-1: Analytics subscription by V-NWDAF from H-NWDAF for outbound roaming users

1.	The V-NWDAF of VPLMN discovers H-NWDAF of H-PLMN that supports the Nwdaf_OutboundRoamerAnalyticaInfo service using the NRF.

NOTE:	The access to the Nwdaf_AnalyticsSubscription service is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

2.	The V-NWDAF subscribes/unsubscribes to analytics information by invoking Nnwdaf_OutboundRoamerAnalytics_Subscribe/ Nnwdaf_OutboundRoamerAnalytics_Unsubscribe service operation.

Editor´s note: Parameters for the Nnwdaf_OutboundRoamerAnalytics_Subscribe service operation are ffs.

3.	H-NWDAF checks if the VPLMN is authorised to subscribe to the indicated analytics based on HPLMN operator polices (that may depend on the VPLMN and may indicate permissible or restricted analytics and related parameters) and user consent of related users.

Editor´s note: User consent check needs to be alligned with SA3.

4.	The H-NWDAF triggers new data collection if needed and monitors the requested analytics.

5.	The H-NWDAF may restrict the exposed analytics information based on HPLMN operator polices (that may depend on the VPLMN)

6.	The H-NWDAF notifies analytics information to the V-NWDAF.

7th change

#### 6.1.2.1	Analytics request by NWDAF service consumer

This procedure is used by the NWDAF service consumer (e.g. including NFs/OAM) to request and get from NWDAF analytics information, using Nnwdaf_AnalyticsInfo service defined in clause 7.3.

Figure 6.1.2.1-1: Network data analytics Request

1.	The NWDAF service consumer requests analytics information by invoking Nnwdaf_AnalyticsInfo_Request service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.1.3.

When a request for analytics information is received, the NWDAF determines whether triggering new data collection is needed.

If the analytics request relates to outbound roaming users, the NWDAF in the H-PLMN may decide to retrieve input data or analytics from the related V-PLMN

If the analytics request relates to inbound roaming users, the NWDAF in the V-PLMN may decide to retrieve input data, analytics, or an analytics profile, from the related H-PLMN

2.	The NWDAF responds with analytics information to the NWDAF service consumer. The NWDAF checks if a Termination Request is indicated as defined in step 2 in clause 6.1.1.1.

8th change

#### 6.1.2.2	Analytics request by AFs via NEF

The analytics exposure to AFs may be performed via NEF by using analytics request to NWDAF.

Figure 6.1.2.2-1 illustrates the interaction between AF and NWDAF performed via the NEF.

Figure 6.1.2.2-1: Procedure for analytics request by AFs via NEF

0.	NEF controls the analytics exposure mapping among the AF identifier with allowed Analytics ID(s) and associated inbound restrictions (i.e. applied to the Analytics ID requested by AF and/or outbound restrictions (i.e. applied to the response of Analytics ID to AF).

In this Release, AF is configured, e.g. via static OAM configuration, with the appropriated NEF to subscribe to analytics information, the allowed Analytics ID(s) and with allowed inbound restrictions (i.e. parameters and/or parameter values) for requesting each Analytics ID.

1.	The AF requests to receive analytics information via NEF by invoking the Nnef_AnalyticsExposure_Fetch service operation defined in TS 23.502 [3]. If the analytics information request is authorized by the NEF, the NEF proceeds with the steps below.

2.	Based on the request from the AF, the NEF requests analytics information by invoking the Nnwdaf_AnalyticsInfo_Request service operation.

If the parameters and/or parameters values of the AF request comply with the restriction in the analytics exposure mapping, NEF forwards in the subscription to NWDAF service the Analytics ID, parameters and/or parameters values from AF in the request to NWDAF.

If the request from AF does not comply with the restrictions in the analytics exposure mapping, NEF may apply restrictions to the request to NWDAF (e.g. restrictions to parameters or parameter values of the Nnwdaf_AnalyticsInfo_Request service operations) based on operator configuration and/or may apply parameter mapping (e.g. geo coordinate mapping to TA(s), Cell-id(s)).

The NEF records the association of the analytics request from the AF and the analytics request sent to the NWDAF.

The NEF selects an NWDAF that supports analytics information requested by the AF using the NWDAF discovery procedure defined in TS 23.501 [2].

If the analytics request relates to outbound roaming users, the NWDAF in the H-PLMN may decide to retrieve input data or analytics from the related V-PLMN

If the analytics request relates to inbound roaming users, the NWDAF in the V-PLMN may decide to retrieve input data, analytics, or an analytics profile, from the related H-PLMN

3.	The NWDAF responds with the analytics information to the NEF.

4.	The NEF responds with the analytics information to the AF. NEF may apply restrictions to the response to AFs (e.g. restrictions to parameters or parameter values of the Nnef_AnalyticsExposure_Fetch response service operation) based on operator configuration. The AF checks if a Termination Request is present and then follows as defined in step 2 in clause 6.1.1.1.

9th change

#### 6.1.2.X	Analytics request by H-NWDAF from V-NWDAF for inbound roaming users

This procedure may be used by the NWDAF in the H-PLMN as service consumer to retrieve analytics from the related V-PLMN for inbound roaming users (from the VPMN perspective)

Figure 6.1.2.X-1: Analytics request by H-NWDAF from V-NWDAF for inbound roaming users

1.	The H-NWDAF checks the user consent of related users.

Editor´s note: User consent check needs to be alligned with SA3.

2.	The H-NWDAF of HPLMN discovers V-NWDAF of VPLMN that supports the InboundRoamerAnalytics service

NOTE:	The access to the Nwdaf_AnalyticsInfo service is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

3.	The H-NWDAF requests analytics information by invoking Nnwdaf_InboundRoamerAnalytics_Request service operation.

Editor´s note: Parameters for the Nnwdaf_InboundRoamerAnalyticsInfo_Request service operation are ffs.

4.	V-NWDAF checks if the HPLMN is authorised to request the analytics based on VPLMN operator polices (that may depend on the HPLMN and may indicate permissible or restricted analytics and related parameters).

5.	The V-NWDAF triggers new data collection if needed and determines the requested analytics.

6.	The V-NWDAF may restrict the exposed analytics information based on VPLMN operator polices (that may depend on the HPLMN)

7.	The V-NWDAF responds with analytics information to the H-NWDAF.

10th change

#### 6.1.2.Y	Analytics request by V-NWDAF from H-NWDAF for outbound roaming users.

This procedure may be used by an NWDAF in the V-PLMN to retrieve analytics from the related H-PLMN for outbound roaming users (from the HPLMN perspective)

Figure 6.1.2.Y-1: Analytics request by V-NWDAF from H-NWDAF for outbound roaming users

1.	The V-NWDAF of VPLMN discovers H-NWDAF of H-PLMN that supports the OutboundRoamerAnalyticaInfo service

NOTE:	The access to the Nwdaf_AnalyticsInfo service is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

2.	The V-NWDAF requests analytics information by invoking Nnwdaf_OutboundRoamerAnalytics_Request service operation.

Editor´s note: Parameters for the Nnwdaf_OutboundRoamerAnalytics_Request service operation are ffs.

3.	H-NWDAF checks if the VPLMN is authorised to request the analytics based on HPLMN operator polices (that may depend on the VPLMN and may indicate permissible or restricted analytics and related parameters) and user consent of related users.

Editor´s note: User consent check needs to be alligned with SA3.

4.	The H-NWDAF triggers new data collection if needed and determines the requested analytics or retrieves a precomputed analytics profile.

5.	The H-NWDAF may restrict the exposed analytics information based on HPLMN operator polices (that may depend on the VPLMN)

6.	The H-NWDAF responds with analytics information and/or analytics profile to the V-NWDAF.

11th change

#### 6.1.2.Z	Analytics profile request by V-NWDAF from H-NWDAF for outbound roaming users.

This procedure may be used by an NWDAF in the V-PLMN to retrieve an analytics profile from the related H-PLMN for an outbound roaming user (from the HPLMN perspective)

The UE analytics profile contains precomputed information derived from analytics information about a UE:

-	UE Data usage patterns.

-	UE Mobility patterns (frequent locations of UE).

-	Optionally, expected UE behavioural parameters.

Editor´s note: Additional contents are FFS.

Figure 6.1.2.Z-1: Analytics profile request by V-NWDAF from H-NWDAF for outbound roaming users

0.	Prerequesit: The H-NWDAF computes analytics profes for UEs that are subscribed to the HPLMN and stores those profiles in the ADRF

1.	The V-NWDAF of VPLMN discovers H-NWDAF of H-PLMN that supports the OutboundRoamerAnalyticaInfo service

NOTE:	The access to the Nwdaf_AnalyticaInfo service is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

2.	The V-NWDAF an analytics profile by invoking Nnwdaf_OutboundRoamerAnalyticsProfile_Request service operation.

Editor´s note: Parameters for the Nnwdaf_OutboundRoamerAnalyticsProfile_Request service operation are ffs.

3.	H-NWDAF checks if the VPLMN is authorised to request the analytics profile based on HPLMN operator polices (that may depend on the VPLMN) and user consent of related users.

Editor´s note: User consent check needs to be alligned with SA3.

4.	The H-NWDAF retrieves a precomputed analytics profile.

5.	The H-NWDAF may restrict the exposed analytics profile based on HPLMN operator polices (that may depend on the VPLMN)

6.	The H-NWDAF responds with analytics profile to the V-NWDAF.

12th change

### 6.2.A	Data collection by H-NWDAF from V-NWDAF for inbound roaming users

This procedure may be used by the NWDAF in the H-PLMN as service consumer to subscribe/unsubscibe to notifications about input data from the related V-PLMN for inbound roaming users (from the VPMN perspective)

Figure 6.2.A-1: Analytics subscription by H-NWDAF from V-NWDAF for inbound roaming users

1.	For subscription, the H-NWDAF checks the user consent of related users.

Editor´s note: User consent check needs to be alligned with SA3.

2.	The H-NWDAF of HPLMN discovers V-NWDAF of VPLMN that supports the Nwdaf_InboundRoamerInputData service using the NRF.

NOTE:	The access to the Nnf_EventExposure services is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

3.	The H-NWDAF subsribes/unsubscribes to notifications about input data by invoking the Nnwdaf_InboundRoamerAnalytics_Subscribe/ Nnwdaf_InboundRoamerAnalytics_Unsubscribe service operation.

Editor´s note: Parameters for the Nnwdaf_InboundRoamerInputData Subscribe service operation are ffs.

4.	V-NWDAF checks if the HPLMN is authorised to request the input data based on VPLMN operator polices (that may depend on the HPLMN and may indicate permissible or restricted input data and related parameters).

5.	The V-NWDAF triggers new data collection if needed and monitores the requested input data using procedures as described in Clauses 6.2.1 to 6.2.8.

6.	The V-NWDAF may restrict the exposed input data based on VPLMN operator polices (that may depend on the HPLMN)

7.	The V-NWDAF notifies input data to the H-NWDAF.

13th change

### 6.2.B	Data collection by V-NWDAF from H-NWDAF for outbound roaming users

This procedure may be used by an NWDAF in the V-PLMN to subscribe/unsubscribe to notifications about input data from the related H-PLMN for outbound roaming users (from the HPLMN perspective)

Figure 6.2.B-1: Analytics subscription by V-NWDAF from H-NWDAF for outbound roaming users

1.	The V-NWDAF of VPLMN discovers H-NWDAF of H-PLMN that supports the Nwdaf_OutboundRoamerInputData service using the NRF.

NOTE:	The access to the Nnf_EventExposure services is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

2.	The V-NWDAF subscribes/unsubscribes to input data information by invoking Nnwdaf_OutboundRoamerInputData_Subscribe/ Nnwdaf_OutboundRoamerInputData_Unsubscribe service operation.

Editor´s note: Parameters for the Nnwdaf_OutboundRoamerInputData_Subscribe service operation are ffs.

3.	H-NWDAF checks if the VPLMN is authorised to subscribe to the indicated input data based on HPLMN operator polices (that may depend on the VPLMN and may indicate permissible or restricted input data and related parameters) and user consent of related users.

Editor´s note: User consent check needs to be alligned with SA3.

4.	The H-NWDAF triggers new data collection if needed and monitors the requested input data, using procedures as described in Clauses 6.2.1 to 6.2.8.

5.	The H-NWDAF may restrict the exposed input data based on HPLMN operator polices (that may depend on the VPLMN)

6.	The H-NWDAF notifies input data to the V-NWDAF.

14th change

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_InboundRoamerAnalytics | Subscribe | Subscribe / Notify | H-NWDAF |
|  | Unsubscribe |  | H-NWDAF |
|  | Notify |  | H-NWDAF |
|  | Request | Request / Response | H-NWDAF |
| Nnwdaf_OutboundRoamerAnalytics | Subscribe | Subscribe / Notify | V-NWDAF |
|  | Unsubscribe |  | V-NWDAF |
|  | Notify |  | V-NWDAF |
|  | Request | Request / Response | V-NWDAF |
| Nnwdaf_OutboundRoamerAnalyticsProfile | Request | Request / Response | V-NWDAF |
| Nnwdaf_InboundRoamerInputData | Subscribe | Subscribe / Notify | H-NWDAF |
|  | Unsubscribe |  | H-NWDAF |
|  | Notify |  | H-NWDAF |
| Nnwdaf_OutboundRoamerInputData | Subscribe | Subscribe / Notify | V-NWDAF |
|  | Unsubscribe |  | V-NWDAF |
|  | Notify |  | V-NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |

15th change

## 7.X	Nnwdaf_InboundRoamerAnalytics Service

### 7.X.1	General

Service Description: This service enables the consumer to request or to subscribe/unsubscribe for network data analytics related to inbound roaming UE.

### 7.X.2	Nnwdaf_InboundRoamerAnalytics_Subscribe service operation

Service operation name: Nnwdaf_InboundRoamerAnalytics_Subscribe.

Description: Subscribes to NWDAF analytics related to inbound roaming UE.

Inputs, Required:

-

Inputs, Optional:

-	.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

### 7.X.3	Nnwdaf_InboundRoamerAnalytics_Unsubscribe service operation

Service operation name: Nnwdaf_InboundRoamerAnalytics_Unsubscribe.

Description: Unsubscribe to NWDAF analytics related to inbound roaming UE.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

### 7.X.4	Nnwdaf_InboundRoamerAnalytics_Notify service operation

Service operation name: Nnwdaf_InboundRoamerAnalytics_Notify.

Description: NWDAF notifies the consumer instance of the analytics related to inbound roaming UEs that the consumer has subscribed to.

Inputs, Required: Notification Correlation Information: this parameter indicates the Notification Correlation Id that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

-	.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

### 7.X.5	Nnwdaf_InboundRoamerAnalytics_Request service operation

Service operation name: Nnwdaf_InboundRoamerAnalytics_Request.

Description: The consumer requests NWDAF operator specific analytics related to outbound roaming UEs.

Inputs, Required:

Inputs, Optional:.

Outputs, Required:.

Outputs, Optional:

Editor´s note: Parameters for the service operation are ffs.

16th change

## 7.Y	Nnwdaf_OutboundRoamerAnalytics Service

### 7.Y.1	General

Service Description: This service enables the consumer to request or to subscribe/unsubscribe for network data analytics related to outbound roaming UE.

### 7.Y.2	Nnwdaf_OutboundRoamerAnalytics_Subscribe service operation

Service operation name: Nnwdaf_OutboundRoamerAnalytics_Subscribe.

Description: Subscribes to NWDAF analytics related to outbound roaming UE.

Inputs, Required:

-

Inputs, Optional:

-	.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

### 7.Y.3	Nnwdaf_OutboundRoamerAnalytics_Unsubscribe service operation

Service operation name: Nnwdaf_OutboundRoamerAnalytics_Unsubscribe.

Description: Unsubscribe to NWDAF analytics related to outbound roaming UE.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

### 7.Y.4	Nnwdaf_OutboundRoamerAnalytics_Notify service operation

Service operation name: Nnwdaf_OutboundRoamerAnalytics_Notify.

Description: NWDAF notifies the consumer instance of the analytics related to InboundRoamer UEs that the consumer  has subscribed to.

Inputs, Required: Notification Correlation Information: this parameter indicates the Notification Correlation Id that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

-	.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

### 7.Y.5	Nnwdaf_OutboundRoamerAnalytics_Request service operation

Service operation name: Nnwdaf_OutboundRoamerAnalytics_Request.

Description: The consumer requests NWDAF operator specific analytics related to outbound roaming UEs.

Inputs, Required:

Inputs, Optional:.

Outputs, Required:.

Outputs, Optional:

Editor´s note: Parameters for the service operation are ffs.

17th change

## 7.Z	Nnwdaf_OutboundRoamerAnalyticsProfile Service

### 7.Z.1	General

Service Description: This service enables the consumer to request an analytics profile related to an outbound roaming UE

### 7.Z.2	Nnwdaf_OutboundRoamerAnalyticsProfile_Request service operation

Service operation name: Nnwdaf_OutboundRoamerAnalyticsProfile_Request.

Description: The consumer requests NWDAF operator specific analytics related to outbound roaming UEs.

Inputs, Required:

Inputs, Optional:.

Outputs, Required:.

Outputs, Optional:

Editor´s note: Parameters for the service operation are ffs.

18th change

## 7.A	Nnwdaf_InboundRoamerInputData Service

### 7.A.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe for input data related to inbound roaming UE for NWDAF analytics.

### 7.A.2	Nnwdaf_InboundRoamerInputData_Subscribe service operation

Service operation name: Nnwdaf_InboundRoamerInputData_Subscribe.

Description: Subscribe to input data related to inbound roaming UE.

Inputs, Required:

-

Inputs, Optional:

-	.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

### 7.A.3	Nnwdaf_InboundRoamerInputData_Unsubscribe service operation

Service operation name: Nnwdaf_InboundRoamerInputData_Unsubscribe.

Description: Unsubscribe to input data related to inbound roaming UE.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

### 7.A.4	Nnwdaf_InboundRoamerInputData_Notify service operation

Service operation name: Nnwdaf_InboundRoamerAnalytics_Notify.

Description: NWDAF notifies the consumer about input data related to inbound roaming UEs that the consumer has subscribed to.

Inputs, Required: Notification Correlation Information: this parameter indicates the Notification Correlation Id that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

-	.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

19th change

## 7.B	Nnwdaf_OutboundRoamerInputData Service

### 7.B.1	General

Service Description: This service enables the consumer to request or to subscribe/unsubscribe for input data related to outbound roaming UE for NWDAF analytics.

### 7.B.2	Nnwdaf_OutboundRoamerInputData_Subscribe service operation

Service operation name: Nnwdaf_OutboundRoamerInputData_Subscribe.

Description: Subscribes to input data related to outbound roaming UE.

Inputs, Required:

-

Inputs, Optional:

-	.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

### 7.B.3	Nnwdaf_OutboundRoamerInputData_Unsubscribe service operation

Service operation name: Nnwdaf_OutboundRoamerInputData_Unsubscribe.

Description: Unsubscribe to input data related to outbound roaming UE.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

### 7.B.4	Nnwdaf_OutboundRoamerInputData_Notify service operation

Service operation name: Nnwdaf_OutboundRoamerInputData_Notify.

Description: NWDAF notifies the consumer of input data related to outbound roaming UEs that the consumer has subscribed to.

Inputs, Required: Notification Correlation Information: this parameter indicates the Notification Correlation Id that has been assigned by the consumer during analytics subscription.

Inputs, Optional:

-	.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor´s note: Parameters for the service operation are ffs.

End of changes