3GPP TSG-SA WG2 Meeting #154-AH-e (e-meeting)	S2-2300221r04

# Elbonia, 16 – 20 January, 2023												 (revision of S2-xxxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3685 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of CN based MT handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_REDCAP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the LS reply from R3-226776, RAN3 has agreed that the triggering criteria from NG-RAN for request for CN based MT data/signalling will be implementation based and it’s a class 1 NGAP procedure. CT4 paper C4-225391 has agreed to use a new indication for AMF to inform SMF on the CN based MT data/signalling handling in RRC_INACTIVE state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the flow to reflect the possibility of RAN trigger the request in different conditions. Update the flow and service operation description to include the new indication from AMF to SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.1.1a, 5.2.8.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

#### 4.8.1.1a	Connection Inactive procedure with CN based MT communication handling

This procedure may be initiated by the serving NG-RAN node when CN based mobile terminating (MT) communication handling is requested for a UE that is configured with eDRX cycle value longer than 10.24 seconds for RRC_INACTIVE state and has at least one PDU session with active user plane as defined in clause 5.31.7 of TS 23.501 [2].

Figure 4.8.1.1a-1: NG-RAN initiated Connection Inactive procedure with CN based MT communication handling

0.	The UE is registered in the network with negotiated eDRX parameters for CM-IDLE state and is in CM-CONNECTED with RRC_CONNECTED state. The AMF provides the eDRX cycle value for CM-IDLE state to NG-RAN as part of the RRC Inactive Assistance Information as defined in clause 5.3.3.2.5 of TS 23.501 [2]. The AMF also includes support indication of CN based MT communication handling for UE in CM-CONNECTED with RRC_INACTIVE state.

1.	NG-RAN determines eDRX cycle value for UE in RRC_INACTIVE state and decides to initiate Connection Inactive with CN based MT communication handling as specified in clause 5.31.7.2.4 of TS 23.501 [2].

1a.	Optionally the NG-RAN may initiate state transition from RRC_CONNECTED to RRC_INACTIVE with RRC configuring eDRX value as specified in TS 38.300 [9]. The NG-RAN may send the request in step 2 towards CN immediately following step 1a or the NG-RAN may delay this request towards CN after connection release towards UE based as specified in clause 5.31.7.2.1 of TS 23.501 [2].

2.	Either immediately following Step 1a or after having delayed the request for NG-RAN based on implementation, the NG-RAN sends N2 message to AMF indicating the UE is transitioning to RRC_INACTIVE state and the CN handles MT communication. The NG-RAN also provides the eDRX cycle value for RRC_INACTIVE to AMF.

If the NG-RAN receives DL NAS message and UE is not reachable due to eDRX, the NG-RAN shall indicate to AMF with NAS non-delivery first and initiate the request towards the AMF immediately if no such request is sent previously.

Editor's note:	Whether the N2 message to be used is a new message or an existing message needs coordination with RAN WG.

3.	For each of the PDU sessions with user plane resources have been activated, the AMF invokes Nsmf_PDUSession_UpdateSMContext Request (PDU Session ID, Cause, Operation type, User Location Information, Age of Location Information, N2 SM Information (Secondary RAT usage data), CN based MT handling indication) towards SMF. The Operation Type is set to a value that indicates to stop user plane DL data transmissions towards the UE and enable data buffering. The SMF starts data buffering for MT data if the data buffering is handled in SMF.

4.	If data buffering is handled in the UPF, the SMF updates the UPF with proper rules for MT data handling.

5.	The SMF sends the Nsmf_PDUSession_UpdateSMContext response.

6.	The AMF sends N2 response to NG-RAN acknowledging the indication and the AMF considers the UE is in CM-CONNECTED with RRC_INACTIVE state.

7.	If the UE connection is not released as specified in step 1a, the NG-RAN initiates state transition from RRC_CONNECTED to RRC_INACTIVE with RRC configuring eDRX cycle value as specified in TS 38.300 [9],

else the UE connection is already released as specified in step 1a, the procedure ends.

****************** NEXT CHANGE ***************

##### 5.2.8.2.6	Nsmf_PDUSession_UpdateSMContext service operation

Service operation name: Nsmf_PDUSession_UpdateSMContext.

Description: It allows to update the AMF-SMF association to support a PDU Session and/or to provide SMF with N1/N2 SM information received from the UE or from the AN, or allows to establish forwarding tunnel between UPFs controlled by different SMFs (e.g. by UPF controlled by old I-SMF and UPF controlled by new I-SMF).

Input, Required: SM Context ID.

Input, Optional: N1 SM container received from the UE, N2 SM information received from the AN (e.g. N3 addressing information, notification indicating that the QoS targets cannot be fulfilled for a QFI, Secondary RAT Usage Data), Operation Type (e.g. UP activate, UP deactivate, UP To Be Switched), Serving GW Address(es) and Serving GW DL TEID(s) for data forwarding during HO from 5GS to EPS, UE location information, AN type, UE Time Zone, H-SMF identifier/address, EBI(s) to be revoked, PDU Session(s) to be re-activated, Direct Forwarding Flag, ARP list, S-NSSAI, Data Forwarding Tunnel (setup/release), UE presence in LADN service area, Target ID, Target AMF ID, GUAMI, backup AMF(s) (if NF Type is AMF), Indication of Access Type can be changed, RAT Type. Backup AMF(s) sent only once by the AMF to the SMF in its first interaction with the SMF. Release indication and release cause, forwarding tunnel information, Handover Complete Indication, Relocation Cancel Indication. MA PDU request indication, MA PDU Network-Upgrade Allowed indication, Indication on whether the UE is registered in both accesses, access on which signalling was received, Subscription to DDN Failure Notification, NEF Correlation ID, MO Exception Data Counter, access for MA PDU Session Release, list of NWDAF IDs and corresponding Analytics ID(s), Satellite backhaul category, N9 forwarding tunnel to support the EAS session continuity required, target UL CL tunnel info for N9 forwarding tunnel to support the EAS session continuity, value of the timer to detect the end of activity on the N9 forwarding tunnel to support the EAS session continuity, CN based MT handling indication.

Output, Required: Result Indication.

Output, Optional: PDU Session ID, Cause, released EBI list, allocated EBI information, N2 SM information (e.g. QFI, UE location information, notification indication indicating that the QoS targets cannot be fulfilled), N1 SM container to be transferred to the AN/UE, type of N2 SM information. MA PDU session Accepted indication, list of NWDAF IDs and corresponding Analytics ID(s), source UL CL tunnel info for N9 forwarding tunnel info to support the EAS session continuity.

See clause 4.3.3.2 and clause 4.3.3.3 for an example usage of this service operation.

See clause 4.9.1.2.2 for the usage of the "UP To Be Switched" Operation Type.

For the use of the "EBI(s) to be revoked" information, see clause 4.11.1.4.1.

For the use of the "Direct Forwarding Flag", see clause 4.11.1.2.2.2.

For the use of the "Indication of Access Type can be changed", see clause 4.2.3.2.

For the use of "release indication and release cause", see clause 4.3.4.2.

For the use of the "forwarding tunnel information", see clause 4.23.4.3.

If the consumer NF is AMF and the SMF determines that some EBIs are not needed, the SMF will put the EBIs back in the released EBI list.

If the consumer NF is AMF and Inter-system mobility happens, the SMF sends allocated EBI information to AMF.

If the ARP of QoS flow is changed, the SMF uses this operation to update EBI-ARP information in the AMF.

If the AMF does not have PDU Session ID, the PDU Session ID is not required for Input and is required for Output.

If consumer NF is AMF and SMF includes N2 SM information in the Output, the SMF indicates type of N2 SM information.

The Small Data Rate Control Status is included if a PDU Session is being released and the UPF or NEF provided Small Data Rate Control Status for the AMF to store. APN Rate Control Status is included if a PDU Session is being released and the UPF or NEF provided APN Rate Control Status for the AMF to store.

NOTE:	The N2 SM information is not interpreted by the AMF.

See clauses 4.22.6.3, 4.22.9 and 4.22.10.2 for detailed usage of this service operation for ATSSS.

****************** END CHANGES ***************