3GPP TSG-SA2 Meeting #154AH-e 		S2-2300285

16 - 20 January, 2023, Electronic meeting

Title:	[DRAFT] LS Response on TSCTSF usage determination in AF session with required QoS

Response to:	S2-2300032/C3-225700

Release:	Rel-17

Work Item:	IIoT

Source:	SA2

To:	CT3

Cc:

Contact person:	Dan Wang

wangdanyjy@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-230xxxx

# 1	Overall description

SA2 thanks CT3 for their LS on TSCTSF usage determination in AF session with required QoS. SA2 has the following answers to the 3 questions from CT3.

Q1	What is the scenario where the AF can update the required QoS from normal QoS requirement into time sensitive QoS requirement?

Answer:

Some discussion happen in SA2 and below is the common understanding. Generally, the AF can reuse the existing AF session to request update service requirement without changing the service itself, so the scenario of AF initially request QoS requirement and then create new request for time sensitive QoS requirement is NOT possible.

Q2	Is it possible for the AF to update the required QoS from time sensitive QoS requirement into normal QoS requirement? And then will the NEF always keep the initial decision about triggering the TSCTSF or not as determined in the initial request?

Answer:

Based on the Q1 answer, it is possible for the AF to add modify QoS requirement with reusing the established AF session without changing the service requirement. During the AF session existing period, the AF ID, DNN/S-NSSAI should not be changed, so the initial decision about TSCTSF should not be changed and as such request to change the service will be rejected. SA2 has approved the S2-230xxx to reflect the above.

Q3	If the answer to Question 1 is that the scenario exists, then could it be possible to have any solution where the NEF does not reject the AF update request?

Answer:

Based on the answer to Q2, if AF follows the SLA between the AF and 5GS, the AF should request with the correct AF ID, DNN/S-NSSAI. And the NEF can make the correct decision about TSCTSF invocation in the initial request, and the following AF request will not be rejected only if the service remains as TSC.

.

# 2	Actions

To CT3

ACTION: 	SA2 asks CT3 group to take the above information into consideration.

# 3	Dates of next TSG SA WG2 meetings

TSG-SA2 Meeting #155		Feb 20 – 24, 2023				Athens

TSG-SA2 Meeting #156   	Apr 17 – 21, 2023				Emeeting