3GPP TSG-SA WG2 Meeting #154AH-E	S2-2300500r01

# E-meeting, January 16 - 20, 2023											 (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0174 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifiers for 5G ProSe UE-to-UE Relay Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Identifiers for 5G ProSe UE-to-UE Relay Discovery need to be specified for both Model A and Model B discovery messages. The description of 5G ProSe UE-to-UE Relay Discovery sets needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.8.4.1 Correct the description of 5G ProSe UE-to-UE Relay Discovery sets. §5.8.4.2 Add description about identifiers for 5G ProSe UE-to-UE Relay Discovery for both Model A and Model B discovery messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No description about identifiers for 5G ProSe UE-to-UE Relay Discovery. The description of 5G ProSe UE-to-UE Relay Discovery sets is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.1.3, 5.8.4.1, 5.8.4.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

5.8.1.3	Source Layer-2 ID

Source Layer-2 ID is defined in clause 5.6.1 of TS 23.287 [2].

The UE self-selects a Source Layer-2 ID for 5G ProSe Direct Discovery and Group member discovery.

NOTE:	The UE implementation needs to ensure that when the UE self-selects Source Layer-2 IDs, the self-selected Source Layer-2 IDs are different between 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-UE Relay Discovery) in clause 6.3.2 and 5G ProSe Direct Communication (including 5G ProSe UE-to-Network Relay Communication and 5G ProSe UE-to-UE Relay Communication) in clause 6.4, 6.5 and 6.7, and are different from any other provisioned Destination Layer-2 IDs as described in clause 5.1 and any other self-selected Source Layer-2 IDs used in a simultaneous 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-UE Relay Discovery) with a different discovery model.

* * * * 2nd Change * * * *

5.8.4.1	General

The 5G ProSe UE-to-UE Relay Discovery message contains two sets of identifiers, a Direct Discovery set and a UE-to-UE Relay Discovery set.

-	The Direct Discovery set of identifiers are part of the contents of the 5G ProSe Direct Discovery message as defined in clause 5.8.1.

-	The UE-to-UE Relay Discovery set of identifiers contain information to support the discovery of the 5G ProSe UE-to-UE Relay and extensions of the Direct Discovery.

5G ProSe UE-to-UE Relay shall modify the UE-to-UE Relay Discovery set of identifiers, and forward the Direct Discovery set and the UE-to-UE Relay Discovery set of identifiers during the discovery procedures.

* * * * 3rd Change * * * *

5.8.4.2	Common identifiers for 5G ProSe UE-to-UE Relay Discovery

The following parameters are used for the 5G ProSe UE-to-UE Relay Discovery Announcement message (Model A), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message, and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-UE Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Announcement message.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Announcement message is selected based on the configuration as described in clause 5.1.5.1.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe UE-to-UE Relay.

-	list of User Info ID of 5G ProSe End UE: provides information about the 5G ProSe End UE.

-	Relay Service Code: information to indicate the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs.

The following parameters are used for the 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) between discoverer 5G ProSe End UE and 5G ProSe UE-to-UE Relay, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message, and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the discoverer 5G ProSe End UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message is selected based on the configuration as described in clause 5.1.5.1.

-	User Info ID of discoverer 5G ProSe End UE: provides information about the discoverer 5G ProSe End UE.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	Relay Service Code: information about connectivity service that the discoverer 5G ProSe End UE is interested in.

The following parameters are used in the 5G ProSe UE-to-UE Relay Discovery Response message (Model B) between discoverer 5G ProSe End UE and 5G ProSe UE-to-UE Relay, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message, and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-UE Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Response message.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe UE-to-UE Relay.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

The following parameters are used for the 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) between 5G ProSe UE-to-UE Relay and discoveree 5G ProSe End UE, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message, and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-UE Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Solicitation message is selected based on the configuration as described in clause 5.1.5.1.

-	User Info ID of discoverer 5G ProSe End UE: provides information about the discoverer 5G ProSe End UE.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe UE-to-UE Relay.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs.

The following parameters are used in the 5G ProSe UE-to-UE Relay Discovery Response message (Model B) between 5G ProSe UE-to-UE Relay and discoveree 5G ProSe End UE, where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message, and User Info ID and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the discoveree 5G ProSe End UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-UE Relay Discovery Response message.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-UE Relay Discovery Solicitation message.

-	User Info ID of discoveree 5G ProSe End UE: provides information about the discoveree 5G ProSe End UE.

-	User Info ID of discoverer 5G ProSe End UE: provides information about the discoverer 5G ProSe End UE.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

NOTE:	The UE implementation needs to ensure that when the UE self-selects Source Layer-2 IDs, the self-selected Source Layer-2 IDs are different between 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-UE Relay Discovery) in clause 6.3.2 and 5G ProSe Direct Communication (including 5G ProSe UE-to-Network Relay Communication and 5G ProSe UE-to-UE Relay Communication) in clause 6.4, 6.5 and 6.7, and are different from any other provisioned Destination Layer-2 IDs as described in clause 5.1 and any other self-selected Source Layer-2 IDs used in a simultaneous 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery and 5G ProSe UE-to-UE Relay Discovery) with a different discovery model.

* * * * End of Changes * * * *