SA WG2 Meeting #154-AH-e	S2-2300533r01

# Jan 16th - 20th, 2023; Elbonia	(was S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3929 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Considering ML model management capability during ADRF discovery and selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is concluded in TR 23.700-81 for KI#4 as follow: 9. NF consumer(s) (MTLF or AnLF) retrieves the ML model or URL (where the model is stored) from ADRF by using ADRF ML model retrieval service. ADRF shall not duplicate the functionality as MLModelProvision Service. ML model management capability should be an extra capability of ADRF, which needs to be indicated in the ADRF Profile registered in NRF and to be considered during the ADRF discovery and selection procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding general description of ML model management to the functional description of ADRF. - Adding ML model filter information per Analytics ID as one of the factors to be considered for ADRF discovery and selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ADRF service consumer cannot discover and select proper ADRF for ML model storage and retrieval. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.26, 6.3.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2.26	ADRF

The Analytics Data Repository Function (ADRF) supports the following functionality:

-	Storage and retrieval of analytics generated by NWDAFs and collected data.

-	Storage of ML model files trained by NWDAFs containing MTLF.

The Analytics Data Repository Function (ADRF) is specified in TS 23.288 [86].

SECOND CHANGE

6.3.20	ADRF discovery and selection

Multiple instances of ADRF may be deployed in a network.

The NF consumers shall utilize the NRF to discover ADRF instance(s) unless ADRF information is available by other means, e.g. locally configured on NF consumers. The ADRF selection function in NF consumers selects an ADRF instance based on the available ADRF instances.

NOTE:	When NF consumer is DCCF, the DCCF can have information available already from previous registrations of ADRFs. In this case, NRF discovery is not needed.

The following factors may be considered by the NF consumer for ADRF selection:

-	S-NSSAI.

-	ML model management capability.

END OF CHANGES