3GPP TSG-SA WG2 Meeting #154AH-E	S2-2300573r08

# Elbonia, January 16 – 20, 2023	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3939 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Improved network control of the UE beahviour for a network slice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to FS_eNS_Ph3 KI#6 conclusion in TR 23.700-41, AMF can provide slice specific polices to the UE so that the UE registers and establishes PDU Session according to the policy. Also network can monitors usage of slices and trigger PDU session release or deregisters a slice. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of impoved network control of UE behaviour as per conclusion of TR 23.700-41. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of improved network conrol of the UE behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4a, 5.15.1, 5.15.X (new), 5.15.7.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 5.15.Y will be introdcued by TS 23.501 CR#xxxx. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

### 5.4.4a	UE MM Core Network Capability handling

The UE MM Core Network Capability is split into the S1 UE network capability (mostly for E-UTRAN access related core network parameters) and the UE 5GMM Core Network Capability (mostly to include other UE capabilities related to 5GCN or interworking with EPS) as defined in TS 24.501 [47] and contains non radio-related capabilities, e.g. the NAS security algorithms, etc. The S1 UE network capability is transferred between all CN nodes at AMF to AMF, AMF to MME, MME to MME, and MME to AMF changes. The UE 5GMM Core Network Capability is transferred only at AMF to AMF changes.

In order to ensure that the UE MM Core Network Capability information stored in the AMF is up to date (e.g. to handle the situation when the USIM is moved into a different device while out of coverage, and the old device did not send the Detach message; and the cases of inter-RAT Registration Area Update), the UE shall send the UE MM Core Network Capability information to the AMF during the Initial Registration and Mobility Registration Update procedure within the NAS message.

The AMF shall store always the latest UE MM Core Network Capability received from the UE. Any UE MM Core Network Capability that an AMF receives from an old AMF/MME is replaced when the UE provides the UE MM Core Network Capability with Registration signalling.

If the UE's UE MM Core Network Capability information changes (in either CM-CONNECTED or in CM-IDLE state), the UE shall perform a Mobility Registration Update procedure when it next returns to NG-RAN coverage. See clause 4.2.2 of TS 23.502 [3].

The UE shall indicate in the UE 5GMM Core Network Capability if the UE supports:

-	Attach in EPC with Request type "Handover" in PDN CONNECTIVITY Request message (clause 5.3.2.1 of TS 23.401 [26]).

-	EPC NAS.

-	SMS over NAS.

-	LCS.

-	5G SRVCC from NG-RAN to UTRAN, as specified in TS 23.216 [88].

-	Radio Capabilities Signalling optimisation (RACS).

-	Network Slice-Specific Authentication and Authorization.

-	Parameters in Supported Network Behaviour for 5G CIoT as described in clause 5.31.2.

-	Receiving WUS Assistance Information (E-UTRA) see clause 5.4.9..

-	Paging Subgrouping Support Indication (NR) see clause 5.4.12.

-	CAG, see clause 5.30.3.3.

-	Subscription-based restrictions to simultaneous registration of network slices (see clause 5.15.12).

-	Support of NSAG (see clause 5.15.14).

-	Minimization of Service Interruption (MINT), as described in clause 5.40.

-	Equivalent SNPNs (see clause 5.30.2.11).

-	Unavailability Period, as described in clause 5.4.1.4.

-	UE Configuration of network-controlled Slice Usage Policies (see clause 5.15.X.2).

If a UE operating two or more USIMs, supports and intends to use one or more Multi-USIM features (see clause 5.38) in a PLMN for a USIM, it shall indicate in the UE 5GMM Core Network Capability for this USIM in this PLMN that it supports these one or more Multi-USIM features with the following indications:

-	Connection Release Supported.

-	Paging Cause Indication for Voice Service Supported.

-	Reject Paging Request Supported.

-	Paging Restriction Supported.

Otherwise, the UE with the capabilities of Multi-USIM features but does not intend to use them shall not indicate support of these one or more Multi-USIM features.

A UE not operating two or more USIMs shall indicate the Multi-USIM features are not supported.

NOTE:	It is not necessary for a UE operating two or more USIMs to use Multi-USIM features with all USIMs.

* * * * Start of 2nd Change * * * *

5.15	Network slicing

### 5.15.1	General

A Network Slice instance is defined within a PLMN or within an SNPN and shall include:

-	the Core Network Control Plane and User Plane Network Functions, as described in clause 4.2,

and, in the serving PLMN, at least one of the following:

-	the NG-RAN described in TS 38.300 [27];

-	the N3IWF or TNGF functions to the non-3GPP Access Network described in clause 4.2.8.2 or the TWIF functions to the trusted WLAN in the case of support of N5CW devices described in clause 4.2.8.5;

-	the W-AGF function to the Wireline Access Network described in clause 4.2.8.4.

The 5G System deployed in a PLMN shall always support the procedures, information and configurations specified to support Network Slice instance selection in the present document, TS 23.502 [3] and TS 23.503 [45].

Network slicing support for roaming is described in clause 5.15.6.

Network slices may differ for supported features and network functions optimisations, in which case such Network Slices may have e.g. different S-NSSAIs with different Slice/Service Types (see clause 5.15.2.1). The operator can deploy multiple Network Slices delivering exactly the same features but for different groups of UEs, e.g. as they deliver a different committed service and/or because they are dedicated to a customer, in which case such Network Slices may have e.g. different S-NSSAIs with the same Slice/Service Type but different Slice Differentiators (see clause 5.15.2.1).

The network may serve a single UE with one or more Network Slice instances simultaneously via a 5G-AN regardless of the access type(s) over which the UE is registered (i.e. 3GPP Access and/or N3GPP Access). The AMF instance serving the UE logically belongs to each of the Network Slice instances serving the UE, i.e. this AMF instance is common to the Network Slice instances serving a UE.

NOTE 1:	Number of simultaneous connection of Network Slice instances per UE is limited by the number of S-NSSAIs in the Requested/Allowed NSSAI as described in clause 5.15.2.1.

NOTE 2:	In this Release of the specification it is assumed that in any (home or visited) PLMN it is always possible to select an AMF that can serve any combination of S-NSSAIs that will be provided as an Allowed NSSAI.

The selection of the set of Network Slice instances for a UE is triggered by the first contacted AMF in a Registration procedure normally by interacting with the NSSF, and can lead to a change of AMF. This is further described in clause 5.15.5.

A PDU Session belongs to one and only one specific Network Slice instance per PLMN. Different Network Slice instances do not share a PDU Session, though different Network Slice instances may have slice-specific PDU Sessions using the same DNN.

During the Handover procedure the source AMF selects a target AMF by interacting with the NRF as specified in clause 6.3.5.

Network Slice-Specific Authentication and Authorization (NSSAA) enables Network Slice specific authentication as described in clause 5.15.10.

Network Slice Admission Control (NSAC) controls the number of registered UEs per network slice and the number of PDU Sessions per network slice as described in clause 5.15.11.

Support of subscription-based restrictions to simultaneous registration of network slices uses Network Slice Simultaneous Registration Group (NSSRG) information to enable control of which Network Slices that can be registered simultaneously by a UE as described in clause 5.15.12.

Support of data rate limitation per Network Slice for a UE enables enforcement of Maximum Bit Rate per Network Slice for a UE as described in clause 5.15.13.

The selection of N3IWF supporting a set of slice(s) is described in clause 6.3.6.

The Support of UE Configuration of network-controlled Slice Usage Policy is described in clause 5.15.X.2.

* * * * Start of 3rd Change * * * *

### 5.15.X	Support of network slice usage control

#### 5.15.x.1	General

Network slice usage control is achieved as follows::

1)	Configuring network Slice Usage Policies to supporting UEs (see caluse 5.15.x.2)

2)	Configuring PDU Sessions inactivity timers, and network slice deregistraion inactivity timers (see clause 5.15.x.3)

#### 5.15.x.2	UE Configuration of network-controlled Slice Usage Policies

The UE during the Registration procedure may indicate in UE MM Core Network Capability that it supports UE configuration of network-controlled Slice Usage Policy. If so, the AMF determines Slice Usage Policy for a network slice for the UE and may configure the UE with this information together with Configured NSSAI to control the usage of this network slice. The AMF may be locally configured with network Slice Usage Policies or receive policies from the (AM-)PCF.

The network-controlled Slice Usage Policy is provided to the UE in the Registration Accept or UE Configuration Update Command message and can include:

-	Indication, for the S-NSSAIs of the HPLMN in the Configured NSSAI, whether the UE needs to register the network slice with the network

when applications in the UE require data transmission in the network slice (i.e. the UE needs to register the network slice only on demand).

-	Indication, for on demand S-NSSAI of the HPLMN in the Configured NSSAI of a deregistration inactivity timer that causes the UE to deregister the network slice after the last PDU Session associated with the S-NSSAI is released. This deregistration inactivity timer is started at the UE and AMF when the last PDU Session associated with the S-NSSAI is released, or the network slice is included in the Allowed NSSAI and no PDU session is established. The deregistration inactivity timer is stopped when the first PDU session is established or the S-NSSAI is removed from the Allowed NSSAI. The AMF and UE may locally remove the S-NSSAI from the Allowed NSSAI when the timer expires. The AMF also send a UE Configuration Update Command message to remove the slice from the Allowed NSSAI.

If the UE and network state became misaligned, the UE may e.g. request connectivity in a network slice which is no longer allowed. In this case, the AMF provides the updated Allowed NSSAI in a UE Configuration Update Command message after rejecting the PDU Session establishment. The UE can then re-register with the network slice if needed.

#### 5.15.x.3	Network-based per UE network slice usage behaviour control

The 5G Core Network perfomrs usage monioring to be abe to enforce releasing of inactive PDU Sessions, and deregistering of UEs from network slices with no PDU Sessions on them according to its own policies. In order to support usage monitoring for a network slice:

-	the AMF runs a slice deregistration inactivity timer per S-NSSAI and access type to deregister the network slice which is started when the network slice is not used by any PDU Session over the corresponding access type. The deregistration inactivity timer is stopped and reset when at least a PDU Session associated with the network slice is successfully established or the network slice is removed form the Allowed NSSAI. When the deregistration inactivity timer for a network slice over an access type expires, the AMF removes the network slice from the Allowed NSSAI over the access type by sending the UE Configuration Update Command message to the imacpted UE(s).

-	the SMFs can provide to UPFs that handle the PDU sessions in the network slice. The PDU Session inactivity timer is started after data packet is not transmissted or received and runs till the next data packet transmitted or received which restarts the timer again. If the PDU Session inactivity timer expires before any packet is received or transmitted, the UPF reports this PDU Session inactivity event to the SMF to cause the SMF to release the PDU Session.While releasing the PDU session the SMF may indicate the AMF to remove the network slice from the Allowed NSSAI notifying the AMF that the PDU Session is released. When the AMF receives the notification of PDU Session release and it includes the indication of network slice removal, if the network slice of the released PDU Session is not used by other PDU Sessions (i.e. the last PDU Session using the network slice is released) over the corresponding access type, the AMF may trigger UE Configuration Update procedure to remove the network slice from the Allowed NSSAI over that corresponding access type. If the PDU session release notification does not include indication of network slice removal then the AMF shall re-start the slice usage control timer.

Editor's note:	Whethter SMF provides slice removal indication to AMF is FFS.

Editor's note:	Roaming asepcts are FFS.

The deregistration inactivity and PDU Session inactivity timers are either pre-configured in the AMF/SMF or received from theUDM. Only if a S-NSSAI is dedicated for a single AF, the AF may use external parameter provisioning procedure to provide deregistration inactivity and PDU session inactivity timers as described in clause 4.15.6.2 of TS 23.502 [3].

To enable a serving network to direct UEs to a preferred network slice, the AMF may request the UE to transfer a PDU Session from one S-NSSAI to another S-NSSAI as described in clause 5.15.Y.

* * * * Start of 4th Change * * * *

5.15.7	Network slicing and Interworking with EPS

#### 5.15.7.X	Support of network slice usage control and Interworking with EPC

As described in clause 5.15.X, if network slice usage control is required for a PDN Connection, the SMF+PGW-C configures PDU Session inactivity timer to the UPF+PGW-U. When the SMF+PGW-C receives inactivity report of the PDN Connection from the UPF+PGW-U, the SMF+PGW-C releases the PDN Connection.

* * * * End of Changes * * * *