3GPP TSG-WG SA2 Meeting #154-AH-e 	S2-2300595

Elbonia, January 16 – 20, 2023	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Subscription to Ranging/SL Positioning

Document for:	Approval

Agenda Item:	9.5.2

Work Item / Release:	Ranging_SL / Rel-18

Abstract: This paper proposes to add the subscription to Ranging/SL Positioning into TR 23.586.

1. Introduction/Discussion

This paper proposes to add the subscription to Ranging/SL Positioning based on the KI#1 conclusion in TR 23.700-86, i.e., 1) The discovery parameters for Ranging/SL positioning service. 2) The discovery parameters for indicating the UE is authorized to use Ranging/SL positioning service and discovery as the associating PLMN.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.586.

# * * * * First change* * * *

## 5.y	Subscription to Ranging/SL Positioning

The subscription information in the UDM contains information to give the user permission to use 5G Ranging/SL Positioning.

At any time, the operator can amend or remove the UE subscription rights from subscription information in the UDM, or to revoke the user's permission to use Ranging/SL Positioning.

The following subscription information is defined for Ranging/SL Positioning:

-	subscription for UE acting as Target UE.

-	subscription for UE acting as SL Reference UE.

-	subscription for UE acting as Located UE.

-	subscription for UE acting as Assistant UE.

-	Ranging/SL positioning QoS parameters.

-	the list of the PLMNs authorized for Ranging/SL Positioning services, including:

-	the list of the PLMNs where the UE is authorised for perform Ranging/SL positioning service over PC5 reference point.

-	the list of the PLMNs where the UE is authorised for Ranging/SL positioning service with Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for Ranging/SL positioning service with Direct Discovery Model B, i.e. to perform Discoverer operation or Discoveree operation or both.

# * * * * End of changes * * * *