SA WG2 Meeting #154AHe	S2-2300609r07

# January 16th – 20th, 2023; Elbonia               	  	 (revision of S2-230)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	               KI1 conclusions: NAUN3 devices

Document for: 	Approval

Agenda Item: 	9.20.1

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract of the contribution: KI1 conclusions: NAUN3 devices

# 1	Discussion

The conclusions take into account the feedback from BBF (LS in S2-2300038) and from CableLabs (LS in S2-230xxxx)

# 2 Proposal

It is proposed to update TR 23700-17 as follows (all text is new)

### 8.1.2	Conclusion for NAUN3 devices

Normative work will proceed according to following principles:

-	principles of solution 1 are endorsed as follows:

-	The RG is configured the mapping between SSID, RG physical ports, etc… and Connection Capabilities whose values are defined by the operator.

-	Policies configured by the PCF for the  RG are used to map from Connection Capabilities to parameters of the PDU session to use for the traffic of NAUN3 devices (as per Table 6.6.2.1-3 in TS 23.503 and optionally port number range used for the packets of when transferred in the 5GS)  associated with a given Connection Capability. .

NOTE: Whether these policies are separate from URSP or not is to be decided during normative phase.

-	principles of solution 21, which allows an end-user (i.e., wireline subscriber) to trigger differentiated QoS for individual devices behind RG via an AF/portal, is endorsed with following exception:  how the AF/portal is populated with the device information (e.g., host name, IP addresses, MAC addresses, etc.) connected behind RG is left to the operators.