3GPP TSG-WG SA2 Meeting #154-AH-e (e-meeting)	S2-2300860r01

# January 16 – 20, 2023, Elbonia		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0279 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures of positioning over the user plane connection between UE and LMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The high level features of support of positioning over user plane connection has been aggreed. Although security and transport protocols aspects for the user plane connection needs wait stage 3 evaluation, some other common high level features and procedures can be agreed in SA2 based on TR conclusion. - The following contents of the common TR conclusion have not been reflected in the TS 23.256: LMF and UE may maintain the established user plane connection between UE and LMF for subsequent LCS session. It is proposed to capture the concept in the high level feature clause. - What factors LMF would consider to determine whether or not utilize user plane positioning for a location request should be clairfied for guidance. - User plane positioning as a common sub-procedure and user plane connection management shoud be added. - Whether there is established user plane connection might be a factor for LMF selection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Further clarify the high level features. Add user plane positioning procedure as a common sub-procedure in clause 6.11. Add procedure for manangement of user plane connection between UE and LMF. Update LMF selection to cover whether there is user plane connection as a selection factor. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specificaions to support positioning over the user plane connection between UE and LMF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10, 6.11.X (new), 6.X (new), 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.10  Support of Positioning over user plane connection between UE and LMF.

LMF and UE may utilize a user plane connection to transfer supplementary services messages and LPP messages

If LMF decides to use user plane the LMF should indicate the UE to use user plane for positioning with the information to establish a secure connection, and via this secure connection, position messages can be transferred between UE and the LMF. LMF and UE may maintain the established user plane connection and the established user plane connection may be reused for subsequent user plane position messages transmission trigged by UE or LMF.

Following factors may be considered by LMF to determine whether or not to utilize user plane connection for positioning after receiving a positioning request from AMF.

- UE/network positioning capability.

- Service type.

- Client type.

- Location type.

- Location QoS.

- Positioning methods/modes to be used.

- Existing User Plane Connection Avaliability

- Other implementation factors, e.g., locally configured operator policy.

# * * * * Second change * * * *

6.11	Common Sub-Procedures

6.11.X	Positioning via a User Plane Connection between UE and LMF

Figure 6.11.X-1 shows a procedure that may be used by an LMF and UE to support positioning over the user plane connection between UE and LMF.

Figure 6.11.x-1: Positioning via a User Plane Connection between UE and LMF

Precondition:	UE uses URSP which includes user plane positioning related PDU session parameters (e.g., a dedicated DNN and S-NSSAI) to establish a PDU session used for user plane positioning. SMF should select a UPF connecting with the LMF for this PDU session, based on S-NSSAI, DNN and UE location information, etc. An LCS Correlation identifier, User Plane Positioning Capability of UE, identifier of UE, and the AMF identity has been passed to the LMF by the serving AMF.

1.	Based on UE user plane positioning capability and other factors listed in clause 5.10, LMF decides whether to continue the position procedure via a user plane connection between UE and LMF.

Steps 3-7 are skipped if there is already a user plane connection context of the target UE in LMF and LMF determines to utilize the user plane connection for positioning.

NOTE X:	LMF can select user plane positioning for specific positioning methods (e.g. motion sensor-based method) and it is based on implementation and local configuration to determine which positioning method requires user plane transport.

2. 	[Conditional] If LMF decides to utilize user plane for positioning and there is no established secure user plane connection between the UE and LMF, LMF sends a user plane information to AMF to indicate UE to utilize user plane for positioning. The user plane information includes the user plane positioning address of the LMF, security related information.

Editor's Note:	The security mechanism and security information provided by LMF to UE is subject to SA WG3's work. 	It is FFS how the UE user plane positioning capability and LMF user plane information is transferred, e.g., based on enhanced LPP, supplementary service or NAS message.

3.	[Conditional] When AMF receives the user plane information from LMF in step 2, AMF sends it to UE via a DL NAS TRANSPORT message.

4.	[Conditional] If there is no established applicable PDU session for the user plane positioning, the UE uses the URSP as defined in 23.503 which includes user plane positioning related PDU session parameters, e.g., a dedicated DNN and S-NSSAI, to establish the PDU session for user plane positioning. UE may send an acknowledgement to LMF through AMF to indiacate a success of utilization of a user plane connection for positioning service or a failure to utilize the user plane connection, e.g., no suitable PDU session established.

5.	[Conditional] AMF sends the acknowledgement received in step 4 to the LMF via Namf_N1messageNotify service.

6.	[Conditional] UE establishes a secure user plane connection with LMF.

7.	If LMF determines to utilize the user plane connection for positioning and the secure user plane connection is established, LPP messages are transferred between UE and LMF for UE based positioning, UE assisted positioning and delivery of assistance data. Supplementary services event report messages from UE are also transferred to LMF via the established user plane connection.

# * * * * Third change * * * *

6.X	Procedures for Management of User Plane Connection between UE and LMF

This clause describes the management of the user plane connection between UE and LMF. Based on operator’s policy, AMF may trigger the establishment of the user plane connection at UE applicable Registration events or when an LCS procedure is initiated using Nlmf_Location_UPConfig service operation provided by LMF, or LMF may trigger the user plane connection establishment after receiving a location request and notify the AMF there is a user plane connection between the UE and the selected LMF using Nlmf_Location_UPNotify service operation.

UE and LMF may maintain the established user plane connection. AMF and LMF may modifiy or terminate the established user plane connection between UE and LMF.

6.X.1 Establishment of User Plane Connection between UE and LMF

The figure 6.X.1-1 shows how AMF trigger the establishment of the user plane connection between UE and LMF at applicable Registration events or when an LCS positioning is initated and how LMF notify the AMF the user plane connection status if the user plane connection establishment is triggered by LMF.

Figure 6.x.1.-1: Connection establishment between UE and LMF

1.	[Conditional] Based on operator’s policy, at applicable Registration events or when an LCS procedure is initiated, AMF may decide to select an LMF and request the UE and the selected LMF to establish a user plane connection to be used for transfer of LPP signalling or supplementary services event report message. The decision may be based on e.g. UE capabilities, UE location, subscription information, LMF load and LMF capabilities.

When a new LCS procedure is initiated for the UE which already has a user plane connection between UE and LMF, the AMF may detect a need to reselect LMF used for user plane positioning as described in step 1b of clause 6.x.2.

2a.	[Conditional] If step 1 is performed, the AMF sends a Nlmf_Location_UPConfig Request towards the LMF to request set up of a user plane connection.

2b. [Conditional] Based on operator’s policy, if the establishenment of user plane connection between UE and LMF is not triggered by AMF, AMF subscribes the status of user plane connection between UE and LMF when an LCS procedure is initiated and UE supports user plane positioning, using an Nlmf_Location_UPNotify subscribe message.

3-7. Based on the request by AMF in step 2a or based on LMF’s own decision, LMF sends its user plane information via AMF to UE to trigger UE establish the user plane connection between UE and LMF, the message flow is same as step 2-6 of figure 6.11.X-1.

8a.	[Conditional] If step 2a is performed, The LMF sends Nlmf_Location_UPConfig Response message to AMF to inform of outcome of connection setup.

8b. [Conditional] If step 2b is performed, LMF indicates AMF in the Nlmf_Location_UPNotify message that user plane connection between the UE and LMF has been established.

9.	The AMF stores the user plane connection context as part of UE context.

After establishing the connection, UE and LMF may maintain the connection for later message transfer via user plane.

6.X.2 Modification of User Plane Connection between UE and LMF

The figure 6.x.2-1 shows how a secure user plane connection between UE and LMF is modified. The flow describes change of LMF but applies also when source and target LMF is the same. The procedure can also be used to terminate the user plane connection to Source LMF by AMF not selecting any Target LMF.

Figure 6.x.2-1: Connection modification between UE and LMFs

1a.	[Conditional] The LMF discovers a need to change LMF or re-establish the user plane connection between UE and LMF. The LMF sends an Nlmf_Location_UPNotify message that includes user plane information and indicates the reason for modification. The address of the AMF was provided to LMF as a "Notification Target Address" in latest Nlmf_Location_UPConfig message or Nlmf_Location_UPNotify Subscribe message.

1b.	[Conditional] At applicable Registration events and when an LCS procedure is initiated, AMF may detect a need to reselect an LMF used for user plane positioing. The detection may be based on e.g. UE capabilities, UE location, subscription information and LMF capabilities. Further at AMF relocation the target AMF needs to inform the LMF of the AMF change.

2.	[Conditional] If AMF reallocation has occurred, this step is skipped. Otherwise, steps 2a, step 3-7, step 8a and step 9 of clause 6.X.1 are performed between AMF, UE, and Target LMF with addition that UE also terminate connection to Source LMF.

3.	The AMF sends an Nlmf_Location_UPConfig Request towards the source LMF. The message may include a request for the Source LMF to terminate a specific user plane connection to the UE by indicateing that the serving LMF is changed. Alternatively, it may include information about AMF reallocation.

4-5.[Conditional] The source LMF may invoke an Nlmf_Location_LocationContextTransfer Request service operation towards the target LMF to provide the current location context of the UE, if there is periodic and triggered UE location events report context. The target LMF informs source LMF of the location context transfer operation results.

6.	[Conditional] If the user plane connection to source LMF is still active, the source LMF terminates the connection to the UE.

7.	The LMF sends Nlmf_Location_UPConfig Response message to AMF to confirm connection termination or acknowledge change of AMF.

# * * * * Fourth change * * * *

5.1	LMF Selection

LMF selection functionality is supported by the AMF to determine an LMF for location estimation of the target UE. The LMF selection functionality is also supported by the LMF if it determines that it is unsuitable or unable to support location for the current UE access network or serving cell for the deferred 5GC-MT-LR procedure for periodic, or triggered location events.

LMF reselection is a functionality supported by AMF when necessary, e.g. due to UE mobility.

The LMF selection/reselection may be performed at the AMF or LMF based on the locally available information i.e. LMF profiles are configured locally at AMF or LMF, or by querying NRF.

The following factors may be considered during the LMF selection:

-	LCS client type.

-	Requested Quality of Service information, e.g.:

-	LCS accuracy,

-	Response time (latency),

-	Access Type (3GPP /N3GPP).

NOTE 1:	Location methods may differ depending on the Access Type, e.g. in the case of WLAN Access Location determination may just correspond to retrieval of IP addressing information from the N3IWF/TNGF; As another example, for Wireline access, Location determination may just correspond to retrieval of geo coordinates corresponding to a GLI as defined in TS 23.316 [21] clause 4.7.8 or a HFC Node ID.

-	RAT type (e.g. 5G NR, eLTE, or any of the RAT Types specified for NR satellite access) and/or the serving AN node (i.e. gNB or NG-eNB) of the target UE.

-	RAN configuration information.

-	LMF capabilities.

-	LMF load.

-	LMF location.

-	Indication of either a single event report or multiple event reports.

-	Duration of event reporting.

-	Network slicing information, e.g. S-NSSAI and/or NSI ID.

-	LMF Service Area consisting of one or more TA(s).

-	Supported GAD shapes.

-	Requested UE has maintained user plane connection with certain LMFs.

When receiving a NAS message from UE, including an LMF ID together with a LPP message (refer to step 25 in clause 6.3.1 for event reporting for a deferred 5GC-MT-LR), AMF sends the LPP message to the LMF, as indicated by the LMF ID.

NOTE 2:	Description on how UE encapsulates the LMF ID in the NAS message is documented in TS 24.571 [36].

# * * * * End of changes * * * *