3GPP TSG-WG SA2 Meeting #154-AH	S2-2300934r01

# e-Meeting, January 16 – 20, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4020 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Wait range during discontinuous coverage. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18 FS_5GSAT_Ph2, the support of mobility management and power saving with discontinuous coverage is studied and concluded. This CR introduces these conclusions introduced as part of 8.2 in 23.700-28. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support of overload control due to discontinuous coverage as per agreement in 23.700-28 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Study agreements are not captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new) * * * *

### 5.4.X Support of discontinuous network coverage

#### 5.4.X.y	Overload control

Support of discontinuous coverage applies on satellite access with discontinuous coverage.

In order to avoid large number of UEs causing excessive signalling load on the network when re-gaining coverage after a coverage gap, the AMF may determine Disco wait range based on network configuration. In such case, the AMF may send the Disco wait range information to the UE during the Registration procedure or UE Configuration Update procedure.

If the UE has received Disco wait range information from the network, the UE determines the value for the Disco wait timer by selecting a random value between 0 and Disco wait range value. The UE starts the Disco wait timer when returning to coverage after being out of coverage on the same RAT Type and PLMN. The UE shall not initiate any NAS signalling on that RAT Type and PLMN when the Disco wait timer is running.

Editor's note:	Whether and how to negotiate UE/network support of Disco wait range information is FFS. Whether separate support indication is necessary or common support indication for all features of 5GSAT_ph2 has to be used is FFS.

# * * * * End of changes * * * *