3GPP TSG-WG SA2 Meeting #154-AH e-Meeting	S2-2300982r04

# Elbonia, Online, January 16 – 20, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4033 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of discontinuous coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Thales, Intel, Xiaomi, Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18 FS_5GSAT_Ph2, the support of mobility management and power saving with discontinuous coverage is studied and concluded. This CR introduces these conclusions to the specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support of mobility management and power saving with discontinuous satellite coverage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mobility management and power saving with discontinuous satellite coverage is not supported in Rel18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X (new), 5.4.X.1 (new), 5.4.X.2 (new), 5.4.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new) * * * *

### 5.4.X Support of discontinuous network coverage for satellite access

Editor's Note:	Which RATs these procedures are applied to is to be determined.

#### 5.4.X.1	Mobility Management and Power Saving Optimization

In case of a satellite constellation that provides discontinuous network coverage, a UE unreachability period may be determined, which indicates the timing information for when a UE is out of coverage, and when the UE has coverage recovered. The UE unreachability period may consider current and potential future locations of the UE.

Tracking Area or RAT specific configuration in the AMF may be used to set timer values based on typical coverage periods of a satellite system.

NOTE 1: 	For example, if a satellite system only provides coverage to a UE for 20 minutes when a satellite passes, and the maximum time before a satellite passes any point on the earth is 10 hours, the AMF could configure the periodic registration timer and Mobile Reachable timer to be just greater than 20 minutes and the Implicit Deregistration timer to be greater than 10 hours. Such configuration does not require AMF to be aware of detailed coverage times for each UE or for different locations.

A UE may be aware of coverage information for the satellite accesses, as described in clause 5.4.X.2. In this case the following applies:

The UE may use the coverage information and mobility pattern information it may have in order to determine its own UE unreachability period. If the UE is able to determine its own UE unreachability period then it requests MICO mode parameters, extended DRX in CM-IDLE parameters, or other timers taking the UE unreachability into account using the relevant procedures.

The UE can also use the Mobility Registration Update procedure to inform the network of a UE unreachability period, according to the procedure in clause 5.4.1.4, when it is about to leave satellite coverage and then also performs a Mobility Registration Update procedure when it returns to coverage using any access type, as described in clause 5.4.1.4. The AMF may indicate to the UE that the UE is required to perform this procedure when leaving and returning to coverage. If the UE requests MICO mode or usage of eDRX, the UE shall not include a UE unreachability period. If the AMF does not indicate to the UE that the UE is required to perform this procedure, the usage is implementation dependent.

NOTE 2:	A UE based on implementation can combine successive periods of no satellite coverage into a single continuous period that is notified to the network if the UE does not require network access during this period.

The AMF may be aware of satellite access coverage information, as described in clause 5.4.X.3. In this case the following applies:

The AMF may determine a UE unreachability period using the coverage information described in clause 5.4.X.3, expected UE behaviour (e.g. Stationary Indication, Expected UE moving trajectory), the UE current location and operator configuration.

The AMF uses existing procedures to provide the UE with timers (e.g. Periodic Registration Update timer), extended DRX in CM-IDLE (see clause 5.31.7.2), and MICO mode configuration (see clause 5.4.1.3). This is to keep UE in power saving mode and avoid the network attempting to page the UE if it is out of satellite network coverage. The AMF may take UE unreachability period into account when determining the timers.

Editor’s Note:	Co-existence and precedence between UE or AMF‘s UE unreachability determination and subsequent parameters adjustment is for discussion and may need further updates.

With discontinuous coverage information, the AMF may know for how long a UE is expected to remain unreachable and when an unreachability period starts. The AMF should consider these and adjust the mobile reachable timer and/or Implicit Deregistration timer such that the AMF does not implicitly deregister the UE while the UE is out of coverage, see clause 5.4.1.

The AMF may indicate to the UE that the UE is required to perform procedures for requesting MICO mode parameters, extended DRX in CM-IDLE parameters, or other timers when leaving and returning to coverage.

Features described for High latency communication in clause 5.31.8 may be used to handle mobile terminated (MT) communication with UEs being unreachable due to discontinuous coverage.

The AMF may optionally provide the UE with a back-off timer which prevents the UE from initiating MO data or signalling when returning to the coverage for satellite access and while the timer is running.

Editor’s Note:	Whether a new back-off timer is used or whether an existing timer can be used is to be determined.

When paging the UE that is using satellite access and discontinuous coverage the AMF may use location information provided by RAN when the UE is released for its paging strategy handling, as described in clause 5.4.3.

#### 5.4.X.2 UE coverage information provisioning to the UE

A UE may use coverage information for the satellite accesses to support discontinuous coverage operations, see clause 5.4.X.1. In addition to the satellite assistance information provided by the NG-RAN (see TS 38.413 [34]), coverage information can be provided to a UE by an AF accessed by a DN via a PDU Session. The coverage information can provide details of satellite access coverage over time for the UEs current and future locations.

Editor’s	Note: clause may need further updates to consider simplified coverage provided by the CN over NAS to the UE.

#### 5.4.X.3 UE Coverage information provisioning to the AMF

The AMF may use coverage information for the satellite access a UE is using at its current and future locations to support discontinuous coverage operations, see clause 5.4.X.1.

Editor’s Note: 	The procedures to obtain coverage information/UE unreachability via OAM or from an AF via the NEF are FFS.

# * * * * Next Change * * * *

### 5.31.8	High latency communication

Functions for High latency communication may be used to handle mobile terminated (MT) communication with UEs being unreachable while using power saving functions as specified in clause 5.31.7 or due to discontinous coverage as described in clause 5.4.X. "High latency" refers to the initial response time before normal exchange of packets is established. That is, the time it takes before a UE has woken up from its power saving state and responded to an initial downlink packet or signal.

When a NR RedCap UE requests to use the power saving functions as specified in clause 5.31.7, then the AMF may, based on local policy, reroute the Registration Request to another AMF that supports High latency communication as specified in clause 6.3.5.

High latency communication is supported by extended buffering of downlink data in the UPF, SMF or NEF when a UE is using power saving functions in CM-IDLE state or in RRC-INACTIVE state, or when the UE is using a satellite access with discontinous coverage, and the UE is not reachable. For UPF anchored PDU sessions the SMF configures during AN release or when NG-RAN indicates via the AMF the UE is in extended DRX for RRC-INACTIVE, the UPF with user data Forwarding Action Rule and user data Buffering Action Rule according to TS 29.244 [65]. The rules include instructions whether UPF buffering applies or the user data shall be forwarded to the SMF for buffering in the SMF. For NEF anchored PDU sessions only extended buffering in the NEF is supported in this release of the specification. During the Network Triggered Service Request procedure or Mobile Terminated Data Transport procedures when using Control Plane CIoT 5GS Optimisation, the AMF provides an Estimated Maximum Wait Time to the SMF if the SMF indicates the support of extended buffering. The SMF determines the Extended Buffering Time based on the received Estimated Maximum Wait Time or local configuration. The handling is e.g. specified in the Network Triggered Service Request procedure, clauses 4.2.3.3, 4.2.6, 4.24.2 and 4.25.5 of TS 23.502 [3].

High latency communication is also supported through notification procedures. The following procedures are available based on different monitoring events:

-	UE Reachability;

-	Availability after DDN failure;

-	Downlink Data Delivery Status.

An AF may request a one-time "UE Reachability" notification when it wants to send data to a UE which is using a power saving function (see event subscription procedure in clause 4.15.3.2 of TS 23.502 [3]). The SCS/AS/AF then waits with sending the data until it gets a notification that the UE is reachable (see notification procedures in TS 23.502 [3]).

An AF may request repeated "Availability after DDN failure" notifications where each UE reachability notification is triggered by a preceding DDN failure, i.e. the AF sends a downlink packet to request a UE reachability notification when the UE becomes reachable. That downlink packet is discarded by the UPF or SMF or NEF (see notification procedures in TS 23.502 [3]).

An AF may request repeated "Downlink Data Delivery Status" notifications when it wants indications that DL data has been buffered or when buffered DL data has been delivered to the UE.

If MICO mode or extended idle mode DRX is enabled, Idle Status Indication allows the AF to determine when the UE transitions into idle mode. When requesting to be informed of either "UE Reachability" or "Availability after DDN failure" notification, the AF may also request Idle Status Indication. If the UDM and the AMF support Idle Status Indication, then when the UE for which MICO mode or extended idle mode DRX is enabled transitions into idle mode, the AMF includes in the notification towards the NEF the time at which the UE transitioned into idle mode, the active time and the periodic registration update timer granted to the UE by the AMF, the eDRX cycle length and the Suggested number of downlink packets if a value was provided to the SMF.

An AF may provide parameters related to High latency communication for different methods to UDM, via NEF, as part of provisioning capability as specified in clause 5.20. The UDM can further deliver the parameters to other NFs (e.g. AMF or SMF) as specified in clause 4.15.6 of TS 23.502 [3].

If the AMF is aware that some signalling or data is pending in the network for an UE that is known as being unreachable for a long duration, e.g. for UE's having extended idle mode DRX, extended DRX for RRC-INACTIVE or MICO enabled, the AMF maintains the N2 connection for at least the Extended Connected Time and provides the Extended Connected Time value in a NG-AP message to the RAN. The Extended Connected Time value indicates the minimum time the RAN should keep the UE in RRC-CONNECTED state regardless of inactivity. At inter-RAN node handovers, if some signalling or data are still pending, the target AMF may send the Extended Connected Time value to the target RAN node.

# * * * * End of changes * * * *