3GPP TSG-WG SA2 Meeting #154-AH e-Meeting	S2-2300982r09

# Elbonia, Online, January 16 – 20, 2023		(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4033 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of discontinuous coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Thales, Intel, Xiaomi, Google Inc., Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18 FS_5GSAT_Ph2, the support of mobility management and power saving with discontinuous coverage is studied and concluded. This CR introduces these conclusions to the specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support of mobility management and power saving with discontinuous satellite coverage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mobility management and power saving with discontinuous satellite coverage is not supported in Rel18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.X (new), 5.4.X.1 (new), 5.4.X.2 (new), 5.4.X.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new) * * * *

#### 5.4.1.4	Support of Unavailability Period

During Registration procedure, the UE supporting the Unavailability Period feature provides "Unavailability Period Support" indication as part of 5GMM Core Network Capability in Registration Request message for initial registration and for every mobility registration. The AMF indicates whether the corresponding feature is supported in the AMF by providing the "Unavailability Period Supported" indication in Registration Accept message.

If the UE and network support Unavailability Period and an event is triggered in the UE that would make the UE unavailable for a certain period of time, the UE may store its MM and SM context in USIM or Non Volatile memory to be able to reuse it after its unavailability period. If the UE can store its contexts the UE may trigger Mobility Registration Update procedure otherwise the UE shall trigger UE-initiated Deregistration procedure.

NOTE:	How the UE stores its contexts depends upon the UE implementation. The UE can store some or all of its contexts in the USIM using existing USIM functionality.

Before the start of an event that makes the UE unavailable, the UE includes the Unavailability Period Duration and triggers either Mobility Registration Update or UE initiated Deregistration procedure:

a)	If the UE initiates Mobility Registration Update procedure:

1)	The AMF may take the Unavailability Period Duration into account when determining Periodic Registration Update timer value. The AMF may provide a Periodic Registration Update time longer than or equal to Unavailability Period Duration to avoid interfering with the UE dealing with the event that causes the unavailability;

2)	The AMF stores the information that the UE is unavailable in UE context, and considers the UE is unreachable (i.e. clear the PPF in AMF) until the UE enters CM-CONNECTED state;

3)	While the UE is unreachable, all high latency communication solutions (see clause 5.31.8) apply if supported in the network, e.g. extended data buffering, downlink data buffering status report, etc; and

4)	If there is "Loss of Connectivity" event subscription for the UE by AF, the AMF considers the remaining time in the Unavailability Period when constructing the "Loss of Connectivity" event report towards the NEF and the Unavailability Period is reported to the respective subscribed AF.

b)	If the UE initiates UE-initiated Deregistration procedure:

1)	If there is "Loss of Connectivity" event subscription for the UE by AF, the AMF considers the remaining time in the Unavailability Period when constructing the "Loss of Connectivity" event report towards the NEF and the Unavailability Period is reported to the respective subscribed AF;

Once the event which makes the UE unavailable is completed in the UE or the event is delayed to a future time or cancelled in the UE, the UE triggers Registration procedure to resume regular service. The UE shall not include the Unavailability Period in this Registration Request message. Depending on the UE state, the Registration procedure can be Initial Registration procedure or Mobility Registration Update procedure.

# * * * * Next Change * * * *

### 5.4.X Support of discontinuous network coverage for satellite access

Editor's Note:	Which RATs these procedures are applied to is to be determined.

#### 5.4.X.1	Mobility Management and Power Saving Optimization

In case of a satellite RAN that provides discontinuous network coverage, a UE out of coverage period may be determined, which indicates the timing information for when a UE is out of coverage, and when the UE has coverage  again. The UE out of coverage period may consider current and potential future locations of the UE.

Tracking Area or RAT specific configuration in the AMF may be used to set timer values based on typical coverage periods of a satellite system.

NOTE 1: 	For example, if a satellite system only provides coverage to a UE for 20 minutes when a satellite passes, and the maximum time before a satellite passes any point on the earth is 10 hours, the AMF could configure the periodic registration timer and Mobile Reachable timer to be just greater than 20 minutes and the Implicit Deregistration timer to be greater than 10 hours. Such configuration does not require AMF to be aware of detailed coverage times for each UE or for different locations.

A UE may be aware of coverage information for the satellite accesses, as described in clause 5.4.X.2.. The UE may use the coverage information and mobility pattern information it may have in order to determine its own UE out of coverage period. If the UE is able to determine its own UE out of coverage period and decides to remain in no service, then it can trigger the Mobility Registration Update procedure, with sufficient time to complete the procedure before the start of the unreachability period to:

a)	request MICO mode parameters, extended DRX in CM-IDLE parameters, or other timers taking the UE out of coverage period into account using the relevant procedures. If the UE requests MICO mode or usage of eDRX; or

b)	inform the network of its UE out of coverage period, when it is about to leave satellite coverage and then perform a Mobility Registration Update procedure when it returns to coverage using any access type. The AMF may indicate to the UE that the UE is required to perform this procedure when leaving and returning to coverage. If the AMF does not indicate to the UE that the UE is required to perform this procedure, the usage is implementation dependent.

NOTE 2:	A UE based on implementation can combine successive periods of no satellite coverage into a single continuous period that is notified to the network if the UE does not require network access during this period.

NOTE 3:	If AMF receives both an indication it is leaving coverage and a request for power saving parameters, the AMF ignores the request for power saving parameters.

Editor’s Note:	how a UE determines to send an indication it is leaving coverage or requests PSM parameters is FFS and may need further updates.

The AMF may be aware of the UEs out of coverage period as described in clause 5.4.X.3. In this case the following applies:

-	The AMF may determine timers (e.g. Periodic Registration Update timer), extended DRX in CM-IDLE (see clause 5.31.7.2), MICO mode configuration (see clause 5.4.1.3) and Estimated Maximum Wait Time based on the UE out of coverage period, Expected UE behaviour (e.g. Stationary Indication, Expected UE moving trajectory), the UE current location and operator configuration. The AMF may also use a UEs out of coverage information determined by the UE, if provided by the UE.

-	The AMF uses existing procedures to provide the UE with timers (e.g. Periodic Registration Update timer), extended DRX in CM-IDLE (see clause 5.31.7.2), and MICO mode configuration (see clause 5.4.1.3), and to the NG-RAN Extended Connected Time (see clause 5.31.7.3). This is to keep UE in power saving mode and avoid the network attempting to page the UE if it is out of satellite network coverage.

Editor’s Note:	Co-existence and precedence between UE or AMF‘s UE unreachability determination and subsequent parameters adjustment is for discussion and may need further updates.

With discontinuous coverage, the AMF may know for how long a UE is expected to remain unreachable and when an unreachability period starts. The AMF should consider these and adjust the mobile reachable timer and/or Implicit Deregistration timer such that the AMF does not implicitly deregister the UE while the UE is out of coverage, see clause 5.4.1.

Features described for High latency communication in clause 5.31.8 may be used to handle mobile terminated (MT) communication with UEs being unreachable due to discontinuous coverage.

The AMF may optionally provide the UE with a back-off timer which prevents the UE from initiating MO data or signalling when returning to coverage for satellite access and while the timer is running.

If a UE indicates support of discontinuous coverage back-off timer during registration procedure, The AMF may optionally provide the UE with a back-off timer before the UE is about to be out of coverage which prevents the UE from initiating MO data or signalling while it is running. The UE can initiate MO data or signalling after the timer expires (If the UE is still in the same satellite coverage). The UE shall stop the timer when it finds another cell (e.g., terrestrial or satellite) so that the UE can register and send MO data to the new access network. AMF triggers the AN release procedure (clause 4.2.6) after providing back-off timer.

Editor’s Note:	Whether a new back-off timer is used or whether an existing timer can be used is to be determined.

When paging the UE that is using satellite access and discontinuous coverage the AMF may use location information provided by RAN when the UE is released for its paging strategy handling, as described in clause 5.4.3.

#### 5.4.X.2 UE determination of UEs out of coverage period

A UE may use coverage information for the satellite accesses to support discontinuous coverage operations, see clause 5.4.X.1. In addition to the satellite assistance information provided by the NG-RAN (see TS 38.413 [34]), coverage information can be provided to a UE by an AF accessed by a DN via a PDU Session. The coverage information can provide details of satellite access coverage over time for the UEs current and future locations.

Editor’s	Note: clause may need further updates to consider simplified coverage provided by the CN over NAS to the UE.

#### 5.4.X.3 AMF determination of UEs out of coverage period

The AMF uses a UEs out of coverage period at its current and future locations to support discontinuous coverage operations, see clause 5.4.X.1.

Editor’s Note: 	The procedures to determine/obtain UEs out of coverage period via OAM or from an AF via the NEF are FFS.

# * * * * Next Change * * * *

### 5.31.8	High latency communication

Functions for High latency communication may be used to handle mobile terminated (MT) communication with UEs being unreachable while using power saving functions as specified in clause 5.31.7 or due to discontinous coverage as described in clause 5.4.X. "High latency" refers to the initial response time before normal exchange of packets is established. That is, the time it takes before a UE has woken up from its power saving state and responded to an initial downlink packet or signal.

When a NR RedCap UE requests to use the power saving functions as specified in clause 5.31.7, then the AMF may, based on local policy, reroute the Registration Request to another AMF that supports High latency communication as specified in clause 6.3.5.

High latency communication is supported by extended buffering of downlink data in the UPF, SMF or NEF when a UE is using power saving functions in CM-IDLE state or in RRC-INACTIVE state, or when the UE is using a satellite access with discontinous coverage, and the UE is not reachable. For UPF anchored PDU sessions the SMF configures during AN release or when NG-RAN indicates via the AMF the UE is in extended DRX for RRC-INACTIVE, the UPF with user data Forwarding Action Rule and user data Buffering Action Rule according to TS 29.244 [65]. The rules include instructions whether UPF buffering applies or the user data shall be forwarded to the SMF for buffering in the SMF. For NEF anchored PDU sessions only extended buffering in the NEF is supported in this release of the specification. During the Network Triggered Service Request procedure or Mobile Terminated Data Transport procedures when using Control Plane CIoT 5GS Optimisation, the AMF provides an Estimated Maximum Wait Time to the SMF if the SMF indicates the support of extended buffering. The SMF determines the Extended Buffering Time based on the received Estimated Maximum Wait Time or local configuration. The handling is e.g. specified in the Network Triggered Service Request procedure, clauses 4.2.3.3, 4.2.6, 4.24.2 and 4.25.5 of TS 23.502 [3].

High latency communication is also supported through notification procedures. The following procedures are available based on different monitoring events:

-	UE Reachability;

-	Availability after DDN failure;

-	Downlink Data Delivery Status.

An AF may request a one-time "UE Reachability" notification when it wants to send data to a UE which is using a power saving function (see event subscription procedure in clause 4.15.3.2 of TS 23.502 [3]). The SCS/AS/AF then waits with sending the data until it gets a notification that the UE is reachable (see notification procedures in TS 23.502 [3]).

An AF may request repeated "Availability after DDN failure" notifications where each UE reachability notification is triggered by a preceding DDN failure, i.e. the AF sends a downlink packet to request a UE reachability notification when the UE becomes reachable. That downlink packet is discarded by the UPF or SMF or NEF (see notification procedures in TS 23.502 [3]).

An AF may request repeated "Downlink Data Delivery Status" notifications when it wants indications that DL data has been buffered or when buffered DL data has been delivered to the UE.

If MICO mode or extended idle mode DRX is enabled, Idle Status Indication allows the AF to determine when the UE transitions into idle mode. When requesting to be informed of either "UE Reachability" or "Availability after DDN failure" notification, the AF may also request Idle Status Indication. If the UDM and the AMF support Idle Status Indication, then when the UE for which MICO mode or extended idle mode DRX is enabled transitions into idle mode, the AMF includes in the notification towards the NEF the time at which the UE transitioned into idle mode, the active time and the periodic registration update timer granted to the UE by the AMF, the eDRX cycle length and the Suggested number of downlink packets if a value was provided to the SMF.

An AF may provide parameters related to High latency communication for different methods to UDM, via NEF, as part of provisioning capability as specified in clause 5.20. The UDM can further deliver the parameters to other NFs (e.g. AMF or SMF) as specified in clause 4.15.6 of TS 23.502 [3].

If the AMF is aware that some signalling or data is pending in the network for an UE that is known as being unreachable for a long duration, e.g. for UE's having extended idle mode DRX, extended DRX for RRC-INACTIVE or MICO enabled, the AMF maintains the N2 connection for at least the Extended Connected Time and provides the Extended Connected Time value in a NG-AP message to the RAN. The Extended Connected Time value indicates the minimum time the RAN should keep the UE in RRC-CONNECTED state regardless of inactivity. At inter-RAN node handovers, if some signalling or data are still pending, the target AMF may send the Extended Connected Time value to the target RAN node.

# * * * * End of changes * * * *