3GPP TSG-WG SA2 Meeting #154-AH-E e-meeting  	S2-2301054r01

# Elbonia, Jan. 16 – 20, 2022	(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1259 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting AR Telephony Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | WID NG_RTC has objective to support AR telephony communication in IMS |  |  |  |  |  |  |  |  |
| Summary of change: |  | network, which has been concluded in TR 23.700-87. This CR is to reflect the |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | change concluded in the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * *

Annex XX (normative):
Support of Data Channel services in IMS

# XX.y	Support of AR Communication

To satisfy the needs of different AR devices that have different media processing capabilities, IMS needs to support media transmission and media rendering related to AR communication. This clause defines how IMS can be enhanced to support AR communication.

Figure XX.y-1 shows the architecture to support AR communication.

Figure xx.y-1: Architecture to support AR communication

NOTE 1:	The DCMF could be an enhanced MRF that support DC.

The AR communication architecture is based on the overall data channel architecture, including the following network functions:

1.	The AR Application Server is responsible for AR service control related to AR communication, including AR session media control, AR media rendering negotiation and AR service handling.

NOTE 2:	The AR Application Server is a specific DC Application Server and is out of scope of 3GPP.

2.	The AR media processing network function ARMF is responsible for AR media transmission and AR media rendering.

NOTE 3:	The ARMF is a logical network function element and can be collocated with other media functions such as DCMF or enhanced MRF.

The following reference points are used for IMS AR related signalling.

-	DC4: Reference point between the AR Application Server and the DCSF for AR service handling and AR session media control.

-	DC5: Service based reference point between the TAS and the ARMF for AR resource management.

NOTE 4:	The DC4 reference point is not specified in 3GPP.

The following reference points are used for IMS AR related media.

-	MDC2: Reference point between the AR Application Server and the DCMF for AR media rendering negotiation.

-	MDC4: Reference point between the AR Application Server and the ARMF for AR media transmission.

NOTE 5:	The MDC2 and MDC4 reference points are not specified in 3GPP.

-	Mb (RTP&DC): The media interface between UE and IMS-AGW, including RTP channel used for audio/video media stream transmission and data channel used for data media stream transmission.

-	Mb (RTP): The media interface between IMS-AGW and ARMF, using RTP channel for audio/video media stream transmission.

-	Mb (HTTP): The media interface between DCMF and ARMF, used by DCMF to transmit data in the HTTP channel after transforming data media stream received in data channel to HTTP format.

-	Mb (DC): the media interface between IMS-AGW and DCMF, using data channel for data media stream transmission.

# * * * * End of changes * * * *