3GPP TSG-SA2 Meeting #154 Adhoc	S2-2301147r03

# EMEETING, 16th – 22nd January, 2023                                              (revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2076 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of wireline access as access to SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conclusion of the eNPN_Ph2 study phase are captured in 23.316 instead of TS 23.501 in order to have allspecifications related to wireline network in a single specification The Onboarding for the wireline neywork is considered not applicable as specified in for NPN since wireline 5G-RG are configured at start-up as defined in BBf and as defined in Rel-16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The abbreviation of SNPN is added. A new clause 4.x is added in order to define how the wireline access network is supported in SNPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non complete support of wireline access in SNPN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2], TS 23.502 [3] and TS 23.503 [4] apply. An abbreviation defined in TS 23.501 [2], TS 23.502 [3] or TS 23.503 [4] takes precedence over the same abbreviation, if any, in any other specifications.

5G-RG	5G Residential Gateway

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

ACS	Auto-Configuration Server

FN-RG	Fixed Network RG

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FWA	Fixed Wireless Access

IGMP	Internet Group Management Protocol

L-W-CP	Legacy Wireless access Control Plane Protocol

L-W-UP	Legacy Wireless access User Plane Protocol

MLD	Multicast Listener Discovery

RG	Residential Gateway

RG-LWAC	RG Level Wireline Access Characteristics

SNPN	Stand-alone Non Public Network

USP	User Services Platform

W-5GAN	Wireline 5G Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-5GBAN	Wireline BBF Access Network

W-CP	Wireline access Control Plane protocol

W-UP	Wireline access User Plane protocol

# * * * * Next change * * * *

## 4.x Access to SNPN services via wireline access network

Access to SNPN defined in TS 23.501 clause 5.30 via a wireline access network follows the same specification and procedures used for accessing a PLMN via a wireline access with the following modifications:

- The SNPN is implicitly selected by wired physical connectivity between 5G-RG or FN-RG and W-AGF.

- In the case of 5G-RG connected via FWA, the 5G-RG shall behave as a SNPN-enabled UE to connect to SNPN as specified in TS 23.501 [2] clause 5.30.2.4.1 where the UE is replaced by 5G-RG.

- -

- The SUPI for a 5G-CRG containing IMSI is described in TS 23.501 clause 5.9.2. The home network domain of SUPI may include the NID of SNPN as defined in TS 23.003 clause 28.2.

- The SUPI for a 5G-CRG and FN-CRG containing GCI is specified in TS 23.316 clause 4.7.4 and TS 23.003 clause 28.15.2. The realm part of NAI format for a SUPI containing a GCI identifying the operator owning the subscription may include the NID of the SNPN.

- The SUPI for a 5G-BRG and FN-BRG containing GLI is specified in TS 23.316 clause 4.7.2, 4.7.3. and TS 23.003 clause 28.16.2. The realm part of NAI format for a SUPI containing a GLI identifying the operator owning the subscription may include the NID of the SNPN.

- The onboarding procedure specified in TS 23.501 clause 5.30.2.10 is not applicable for a 5G-RG accessing wireline access network.

Editor’s Note: The procedure for onboarding of a FWA is FFS

Editor’s Note: The specification in this clause may be revised considering feedback from BBF and CableLabs

# * * * * End of changes * * * *