3GPP TSG-SA WG2 Meeting #154AHE	S2-2301169r09

# E-meeting, 16th – 20th January 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0683 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for network data analytics in roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedures for network data analytics in roaming need to defined to support data and analytics exchange in roaming case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add general descriptions for network data analytics in roaming. - Add procedures for analytics exchange in roaming case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network data analytics in roaming case is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new), 6.x.2 (new), 6.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

## 6.1.x	Analytics Exposure in roaming case

### 6.1.x.1	General

Based on operator’s policy and local regulations (e.g. privacy), analytics may be exchanged between PLMNs (i.e. HPLMN and VPLMN). In this case, an NWDAF is used as entry point to exchange analytics in roaming scenario between HPLMN and VPLMN. It authorizes the request according to operator policies, and filters the information exposed in reply to the request.

The H-NWDAF with roaming entry capability may provide analytics to the V-NWDAF with roaming entry capability. The V-NWDAF with roaming entry capability may provide analytics to the H-NWDAF with roaming entry capability.

NOTE 1:	Analytics that rely on input data from the HPLMN are preferbly not provided from V-NWDAF to H-NWDAF, but generated by the H-NWDAF. Analytics that rely on input data from the VPLMN are preferbly not provided from H-NWDAF to V-NWDAF, but generated by the V-NWDAF.

Editor's note:	Analytics exchange between HPLMN and VPLMN require updates based on feedback from GSMA.

Editor's note:	The user consent check procedure and security aspects will align with SA3 WG's conclusion.

Editor’s note:	it is FFS if both H-NWDAF and V-NWDAF need to have roaming entry capability.

V-NWDAF may request or subscribe to HPLMN analytics from the H-NWDAF with roaming entry capability as described in clause 6.x.2, and then the analytics can be leveraged by the 5GC NF in the VPLMN, for example:

-	In home routed roaming scenarios, HPLMN analytics (i.e. slice load level analytics, NF load analytics, etc.) can be leveraged by the AMF in the VPLMN for network slice selection and SMF selection for PDU Session management.

-	UE-related analytics (e.g.. service experience analytics, etc.) can include statistics or predictions for outbound roaming UEs.

H-NWDAF may request or subscribe to VPLMN analytics from the V-NWDAF with roaming entry capability as described in clause 6.x.3, and then the analytics can be leveraged by the 5GC NF in the HPLMN, for example:

-	In home routed roaming scenarios, analytics information with statistics or predictions for outbound roaming UEs can be leveraged by the H-PCF for QoS control of the PDU Session.

-	Analytics (i.e. service experience analytics, slice load level analytics, etc.) can be leveraged by the H-PCF for decision of NSSP in URSP rules provisioned to the UE roaming in the VPLMN.

The consumer 5GC NF may indicate the following parameters, if available, in addition to those defined in the analytics related clauses for non-roaming case, in the request or subscription of analytics information in a different PLMN (i.e. HPLMN or VPLMN) towards the NWDAF (which is in the same PLMN as the consumer 5GC NF):

Editor's note:	How to indicate requesting roaming analytics (e.g. using Analytics Filter Information as below) by the analytics consumer is FFS.

-	to request/subscribe to HPLMN analytics, in Analytics Filter Information:

-	HPLMN ID;

-	mapped S-NSSAI of the HPLMN, if VPLMN S-NSSAI is included in the Analytics Filter Information.

-	to request/subscribe to VPLMN analytics, in Analytics Filter Information:

-	VPLMN ID;

-	VPLMN S-NSSAI which is mapped to the HPLMN S-NSSAI, if HPLMN S-NSSAI is included in the Analytics Filter Information.

Based on the analytics request or subscription from the 5GC NF in the HPLMN, or local configuration, the H-NWDAF may indicate the following parameters in the analytics request or subscription to the V-NWDAF with roaming entry capability, for requesting/subscribing to analytics in the VPLMN:

Editor's note:	The parameters are FFS and should be aligned with the service definition for roaming analytics.

-	Analytics ID;

-	HPLMN ID;

-	Analytics Filter Information:

-	VPLMN S-NSSAI which is mapped to the HPLMN S-NSSAI, if the HPLMN S-NSSAI and/or VPLMN S-NSSAI is provided by the consumer 5GC NF in the HPLMN;

-	other Analytics Filter Information as provided by consumer 5GC NF in the HPLMN, if applicable in the VPLMN.

-	other parameters as provided by consumer 5GC NF in the HPLMN, if applicable in the VPLMN.

Based on the analytics request or subscription from the 5GC NF in the VPLMN, or local configuration, the V-NWDAF may indicate the following parameters in the analytics request or subscription to the H-NWDAF with roaming entry capability, for requesting/subscribing to analytics in the HPLMN:

Editor's note:	The parameters are FFS and should be aligned with the service definition for roaming analytics.

-	Analytics ID;

-	VPLMN ID;

-	Analytics Filter Information:

-	HPLMN S-NSSAI which is mapped to the VPLMN S-NSSAI, if the VPLMN S-NSSAI and/or HPLMN S-NSSAI is provided by the consumer 5GC NF in the VPLMN;

-	other Analytics Filter Information as provided by consumer 5GC NF in the VPLMN, if applicable in the HPLMN.

-	other parameters as provided by consumer 5GC NF in the VPLMN, if applicable in the HPLMN.

### 6.1.x.2	Analytics Exposure from HPLMN to VPLMN

Figure 6.1.x.2-1 shows the procedure that the V-NWDAF requests network analytics (i.e. slice load level analytics, NF load analytics, service experience analytics, etc.) in HPLMN from the H-NWDAF with roaming entry capability, upon receiving an analytics information/subscription request from the NF in VPLMN (e.g. AMF).

Editor's note:	The service names in the following procedure are descriptive and may need to be updated.

Figure 6.1.x.2-1: Procedure for analytics exposure from HPLMN to VPLMN

1.	Consumer NF in VPLMN (e.g. AMF) sends an Analytics request/subscribe (Analytics ID, Analytics Filter Information) to V-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2.	V-NWDAF discovers the H-NWDAF with roaming entry capability as described in clause 5.2. V-NWDAF sends a roaming analytics request/subscribe (Analytics ID, Analytics Filter Information) to H-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request or a Nnwdaf_RoamingAnalytics_Subscribe, based on the Analytics request/subscribe received from the Consumer NF in VPLMN. If V-NWDAF has the roaming entry capability, it checks the policies and regulatory constraints between the HPLMN and the VPLMN to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the request is rejected, the following steps can be skipped.

3.	H-NWDAF checks the policies and regulatory constraints between the HPLMN and the VPLMN, and user consent for analytics as defined in clause 6.2.9 if needed, to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the roaming analytics request/subscribe is accepted, H-NWDAF collects data from the NF(s) and/or OAM in HPLMN and derives the requested analytics.

Editor's note:	The user consent check procedure and security aspects will align with SA3 WG's conclusion.

Editor’s NOTE: If the HNWDAF with roaming entry capability can not support to generate the requested analytics, then how to exposue the requested analytics is FFS.

4.	H-NWDAF sends the HPLMN analytics information to the V-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or  Nnwdaf_RoamingAnalytics_Notify, depending on the service used in step 2. The H-NWDAF may restrict the exposed analytics information based on HPLMN operator polices.

5.	[Optional] V-NWDAF collects data from the NF(s) and/or OAM in VPLMN and derives the requested analytics. These steps can be executed in parallel with step 3-6.

6.	V-NWDAF sends the HPLMN analytics information received in step 6, together with the VPLMN analytics information if step 5 are performed, to the Consumer NF in VPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

Editor's note:	Whether and how the V-NWDAF aggregates the analytics from HPLMN and the analytics derived in VPLMN is FFS.

### 6.1.x.3	Analytics Exposure from VPLMN to HPLMN

Figure 6.1.x.3-1 shows the procedure that the H-NWDAF requests network analytics (i.e. service experience analytics, slice load level analytics, etc.) in VPLMN from the V-NWDAF with roaming entry capability, upon receiving an analytics information/subscription request from the NF in HPLMN (e.g. H-PCF).

Editor's note:	The service names in the following procedure are descriptive and may need to be updated.

Figure 6.1.x.3-1: Procedure for analytics exposure from VPLMN to HPLMN

1.	Consumer NF in HPLMN (e.g. H-PCF) sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting, Analytics Filter Information) to H-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2.	The H-NWDAF checks user consent for analytics as defined in clause 6.2.9 if needed, then the H-NWDAF discovers the V-NWDAF with roaming entry capability as described in clause 5.2.

H-NWDAF sends a roaming analytics request/subscribe (Analytics ID, Analytics Filter Information) to V-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request or a Nnwdaf_RoamingAnalytics_Subscribe, based on the Analytics request/subscribe received from the Consumer NF in HPLMN. If H-NWDAF has the roaming entry capability, it checks the policies and regulatory constraints between the HPLMN and the VPLMN to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the request is rejected, the following steps can be skipped.

3.	V-NWDAF checks the policies and regulatory constraints between the HPLMN and the VPLMN, to determine if the roaming analytics request/subscribe can be accepted or must be rejected. If the roaming analytics request/subscribe is accepted, V-NWDAF collects data from the NF(s) and/or OAM in VPLMN and derives the analytics.

Editor's note:	The user consent check procedure and security aspects will align with SA3 WG's conclusion.

Editor’s NOTE: If the VNWDAF with roaming entry capability can not support to generate the requested analytics, then how to exposue the requested analytics is FFS.

4.	V-NWDAF sends the VPLMN analytics information to the H-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or Nnwdaf_RoamingAnalytics_Notify, depending on the service used in step 2. The V-NWDAF may restrict the exposed analytics information based on VPLMN operator polices.

5.	[Optional] H-NWDAF collects data from the NF(s) and/or OAM in HPLMN and derives the analytics. These steps can be executed in parallel with step 3-6.

6.	H-NWDAF sends the VPLMN analytics information received in step 6, together with the HPLMN analytics information if step 5 are performed, to the Consumer NF in HPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

Editor's note:	Whether and how the H-NWDAF aggregates the analytics from VPLMN and the analytics derived in HPLMN is FFS.

* * * End of Changes * * * *