3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2301196r04

# Elbonia, January 16 – 20, 2023		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4088 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify on periodicity adaption on Proactive feedback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarify how RAN provides periodicity feedback for AF to adaption based on SA1 confiramtion in S1-223726 Proposed to align the titles with the overview in 5.27.2.5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Update the TSCAC/TSCAI with periodicity Range parameters 2.Upate the TSCAC/TSCAI determination based on periodicity Range 3. Beside BAT offset, add proposed periodicity for the Proactive RAN feedback adaptation 4. It is proposed to delete with BAT in the title of 5.27.2.5.2 and add for Burst Arrival Time adaptation in the title of 5.27.2.5.3 to align the titles |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN provides periodicity feedback will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.2.1, 5.27.2.3, 5.27.2.4, 5.27.2.5, 5.27.2.5.1, 5.27.2.5.2, 5.27.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.27.2	TSC Assistance Information (TSCAI) and TSC Assistance Container (TSCAC)

5.27.2.1	General

TSC Assistance Information (TSCAI) is defined in Table 5.27.2-1 and describes TSC traffic characteristics for use in the 5G System. TSCAI may be used by the 5G-AN, if provided by SMF. The knowledge of TSC traffic pattern is useful for 5G-AN as it allows more efficiently scheduling of QoS Flows that have a periodic, deterministic traffic characteristics either via Configured Grants, Semi-Persistent Scheduling or with Dynamic Grants.

The TSCTSF determines the TSC Assistance Container (defined in Table 5.27.2-2) based on information provided by an AF/NEF as described in clause 5.27.2.3 and provides it to the PCF for IP type and Ethernet type PDU Sessions. In the case of integration with IEEE TSN network, the TSN AF determines TSC Assistance Container as described in clause 5.27.2.2 and provides it to the PCF for Ethernet PDU Sessions. The PCF receives the TSC Assistance Container from the TSCTSF or the TSN AF and forwards it to the SMF as part of PCC rule as described in clause 6.1.3.23a of TS 23.503 [45].

The SMF binds a PCC rule with a TSC Assistance Container to a QoS Flow as described in clause 6.1.3.2.4 of TS 23.503 [45]. The SMF uses the TSC Assistance Container to derive the TSCAI for that QoS Flow and sends the derived TSCAI to the NG-RAN. The Periodicity, Burst Arrival Time, and Survival Time components of the TSCAI are specified by the SMF with respect to the 5G clock. The SMF is responsible for mapping the Burst Arrival Time and Periodicity from an external clock (when available) to the 5G clock based on the time offset and cumulative rateRatio (when available) between the external clock time and 5GS time as measured and reported by the UPF. The SMF determines the TSCAI as described in clause 5.27.2.4.

A Survival Time, which indicates the time period an application can survive without any data burst, may be provided by TSN AF/AF either in terms of maximum number of messages (message is equivalent to all packets of a data burst) or in terms of time units. Only a single data burst is expected within a single time period referred to as the periodicity.

The SMF may send an update of the TSCAI to the NG-RAN as defined in clauses 4.3.3.2, 4.9.1.2.2 and 4.9.1.3.2 of TS 23.502 [3].

Table 5.27.2-1: TSC Assistance Information (TSCAI)

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two data bursts. |
| Burst Arrival Time (optional) | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| Survival Time (optional) | Survival Time, as defined in TS 22.261 [2], refers to the time period an application can survive without any data burst. |
| Burst Arrival Time Window (BAT Window) (optional) (NOTE 1) (NOTE 2) | Indicates the acceptable earliest and latest arrival time of the first packet of the data burst at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| Capability for BAT adaptation (optional) (NOTE 1) | Indicates that the AF will adjust the burst sending time according to the network provided Burst Arrival Time offset (see clause 5.27.2.5). |
| Capability for Periodicity adaptation (optional) (NOTE 4) | It refers to the capability of AF to adapt to upstream/downstream transmission periodicity according to NG-RAN proposed periodicity. |
| Periodicity Range (optional) (NOTE 3) | It indicates that the AF will adjust the periodicity and provides the acceptable range. It can be formulated as lower bound and upper bound of the Periodicity. |
| NOTE 1: Only one of the parameters (BAT Window or Capability for BAT adaptation) can be provided. NOTE 2: The parameter will only be provided together with Burst Arrival Time. NOTE 3: The Periodicity Range will only be provided together with Periodicity when Burst Arrival Time and either the capability for BAT adaptation or a Burst Arrival Time Window are present. NOTE 4: When Capability for Periodicity adaptation is provided, Periodicity Range shall not be provided. Periodicity Range is provided only when capability for periodicity adaptation is not provided. |  |

Table 5.27.2-2: TSC Assistance Container (TSCAC)

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two data bursts. |
| Burst Arrival Time (optional) | The time when the first packet of the data burst arrives at the ingress port of 5GS for a given flow direction (DS-TT for uplink, NW-TT for downlink). |
| Survival Time (optional) | It refers to the time period an application can survive without any data burst, as defined in TS 22.261 [2]. |
| Time Domain (optional) | The (g)PTP domain of the TSC flow. |
| Burst Arrival Time Window (BAT Window) (optional) (NOTE 1) (NOTE 2) | Indicates the acceptable earliest and latest arrival time of the first packet the data burst at the ingress port of 5GS for a given flow direction (DS-TT for uplink, NW-TT for downlink). |
| Capability for BAT adaptation (optional) (NOTE 1) | It indicates that the AF will adjust the burst sending time according to the network provided Burst Arrival Time offset (see clause 5.27.2.5). |
| Capability for Periodicity adaptation (optional) (NOTE 4) | It refers to the capability of AF to adapt to upstream/downstream transmission periodicity according to NG-RAN proposed periodicity. |
| Periodicity Range (optional) (NOTE 3) | It indicates that the AF will adjust the periodicity and provides the acceptable range. It can be formulated as lower bound and upper bound of the Periodicity. |
| NOTE 1: Only one of the parameters (BAT Window or Capability for BAT adaptation) can be provided. NOTE 2: The parameter will only be provided together with Burst Arrival Time. NOTE 3: The Periodicity Range will only be provided together with Periodicity when Burst Arrival Time and either the capability for BAT adaptation or a Burst Arrival Time Window are present. NOTE 4: When Capability for Periodicity adaptation is provided, Periodicity Range shall not be provided. Periodicity Range is provided only when capability for periodicity adaptation is not provided. |  |

# * * * * Second change * * * *

5.27.2.3	TSC Assistance Container determination by TSCTSF

The TSCTSF constructs TSC Assistance Container (defined in Table 5.27.2-2) based on information provided (directly or via NEF) by the AF for IP or Ethernet type PDU Sessions.

The AF may provide Flow Direction, Burst Arrival Time (optional) at the UE/DS-TT (uplink) or UPF/NW-TT (downlink), Maximum Burst Size, Periodicity, Survival Time (optional), and a Time Domain (optional) to the TSCTSF. If the AF is able to adjust the burst sending time, the AF may in addition provide a BAT Window or the Capability for BAT adaptation to the TSCTSF. Addtionally if the AF is able to adjust the periodicity, the AF may also provide its capability for periodicity adaptation or the Periodicity Range along with the Periodicity to the TSCTSF. Based on these parameters, the TSCTSF constructs a TSC Assistance Container and provides it to PCF. If the AF provides to the TSCTSF a Burst Arrival Time or Periodicity without corresponding Time Domain, the TSCTSF sets the Time Domain = "5GS" in the TSC Assistance Container.

NOTE:	The Maximum Burst Size is signalled separately, i.e. it is not part of the TSC Assistance Container.

The AF provides these parameters to the NEF and the NEF forwards these parameters to the TSCTSF. The AF trusted by the operator provides these parameters to the TSCTSF directly.

The TSCTSF sends the TSC Assistance Container to the PCF as follows:

-	The TSCTSF uses the UE IP address/DS-TT port MAC address to identify the PCF and N5 association related to the PDU Session of a UE/DS-TT.

5.27.2.4	TSCAI determination based on TSC Assistance Container

The SMF determines the TSCAI (defined in Table 5.27.2-1) for the QoS Flow based on the TSC Assistance Container of the PCC rule bound to the QoS Flow. This clause is applicable irrespective of whether the TSC Assistance Container is determined by the TSN AF or by the TSCTSF.

The Burst Arrival Time and Periodicity component of the TSCAI that the SMF sends to the 5G-AN are specified with respect to the 5G clock. The SMF is responsible for mapping the Burst Arrival Time and Periodicity in the TSC Assistance Container from an external clock to the 5G clock based on the time offset and cumulative rateRatio (when available) between external time and 5GS time as measured and reported by the UPF. The SMF may correct the TSCAI based on the UPF report for time offset and cumulative rateRatio between external PTP time and 5GS time as measured and reported by the UPF.

The TSCAI parameter determination in SMF is done as follows:

-	For traffic in downlink direction, the SMF corrects the Burst Arrival Time in the TSC Assistance Container based on the latest received time offset measurement from the UPF and sets the TSCAI Burst Arrival Time as the sum of the corrected value and CN PDB as described in clause 5.7.3.4, representing the latest possible time when the first packet of the data burst arrives at the AN.

- In addition, for downlink direction, if the TSCAC includes Burst Arrival Time Window instead of Burst Arrival Time, the SMF corrects the earliest BAT in window (opening edge) and latest BAT in window (closing edge) based on the latest received time offset measurement from the UPF and sets the TSCAI opening and closing edges of the Burst Arrival window as a sum of the corrected values and PDB as described in clause 5.7.3.4, representing the AF preferred BAW at the AN.

-	For traffic in uplink direction, the SMF corrects the Burst Arrival Time in the TSC Assistance Container based on the latest received time offset measurement from the UPF and sets the TSCAI Burst Arrival Time as the sum of the corrected value and UE-DS-TT Residence Time, representing the latest possible time when the first packet of the data burst arrives at the egress of the UE. How the SMF corrects the Burst Arrival Time if the UE-DS-TT Residence Time has not been provided by the UE is up to SMF implementation.

- In addition, for uplink direction, if the TSCAC includes Burst Arrival Window instead of Burst Arrival Time, the SMF corrects the earliest BAT in window (opening edge) and latest BAT in window (closing edge) based on the latest received time offset measurement from the UPF and sets the TSCAI opening and closing edges of the Burst Arrival window as the sum of the corrected value and UE-DS-TT Residence Time, representing the AF preferred BAT window at the AN.

-	The SMF corrects the Periodicity in the TSC Assistance Container using the cumulative rateRatio if the cumulative rateRatio was previously received from the UPF and sets the TSCAI Periodicity as the corrected value. Otherwise, the SMF sets the received Periodicity in the TSCAI without any correction.

-	The SMF sets the TSCAI Flow Direction as the Flow Direction in the TSC Assistance Container.

-	If Survival Time is provided in terms of maximum number of messages, the SMF converts maximum number of messages into time units by multiplying its value by the TSCAI Periodicity, and sets the TSCAI Survival Time to the calculated value. If Survival Time is provided in time units, the SMF corrects the Survival Time using the cumulative rateRatio if the cumulative rateRatio was previously received from the UPF and sets the TSCAI Survival Time to the corrected value. Otherwise, SMF sets the TSCAI Survival Time without correction.

-	If the TSC Assistance Container contains a BAT Window, the SMF sets and corrects the indicated earliest and latest possible arrival time of the first packet in the same way it is described for the correction of the Burst Arrival Time above.

-	If the TSC Assistance Container contains a Capability for BAT adaptation, the SMF sets the Capability for BAT adaptation in the TSCAI. When the SMF determines the TSCAI containing the Capability for BAT adaptation and without a BAT, the SMF enables notification control notification control for the QoS Flow in order to receive the BAT offset along with the "GFBR can no longer be guaranteed" notification described in clause 5.7.2.4.

-    If the TSC Assistance Container contains a Periodicity Range, the SMF sets and corrects the Periodicity Range in the same way it is described for the correction of the Periodicity above.

-	If the TSC Assistance Container contains Capability for Periodicity adaptation, the SMF sets this value to Capability for Periodicity adaptation parameter in the TSCAI.

Depending on whether the Time Domain is provided in the TSC Assistance container, SMF may perform the following:

-	the SMF provisions the UPF/NW-TT to report the clock drifting between 5G clock and the external GM clock for the (g)PTP time domain number that is configured to the NW-TT.

-	the SMF provisions the UPF/NW-TT to report the clock drifting between 5G clock and the external GM clock for the given Time Domain number.

The SMF uses the N4 Association Setup or Update procedures as described in clause 4.4.3 of TS 23.502 [3] to provision the UPF to report the clock drifting.

If the SMF has clock drift information for a Time Domain and if the Time Domain matches with the Time Domain in the TSC Assistance Container (i.e. clock drift between 5G timing and AF supplied Time Domain determined based on UPF reporting), or Time Domain information is not provided in the TSC Assistance Container, then the SMF may adjust the TSCAI information so that it reflects the 5GS Clock as described in clause 5.27.2.1.

If the SMF does not have synchronization information for a requested Time Domain in the TSC Assistance Container, or the Time Domain in the TSC Assistance Container is set to a value = "5GS", then the TSCAI information will be used without adjustment.

In the case of drift between external GM clock and 5G clock, the UPF updates the offset to SMF using the N4 Report Procedure as defined in clause 4.4.3.4 of TS 23.502 [3]. If the cumulative rateRatio is available and in the case of change of cumulative rateRatio between external PTP time and 5G time, the UPF updates the cumulative rateRatio to SMF using the N4 Report Procedure as defined in clause 4.4.3.4 of TS 23.502 [3]. The SMF may then trigger a PDU Session Modification as defined in clause 4.3.3 of TS 23.502 [3] in order to update the TSCAI to the NG-RAN without requiring AN or N1 specific signalling exchange with the UE.

NOTE 4:	In order to prevent frequent updates from the UPF, the UPF sends the offset or the cumulative rateRatio only when the difference between the current measurement and the previously reported measurement is larger than a threshold as described in clause 4.4.3.4 of TS 23.502 [3].

# * * * * Third change * * * *

5.27.2.5	RAN feedback for Burst Arrival Time offset and Periodicity

5.27.2.5.1	Overview

If the NG-RAN receives a TSCAI containing a BAT Window or the Capability for BAT adaptation for a QoS Flow, the NG-RAN can determine a BAT offset in order to align the arrival of the traffic bursts with the next expected transmission opportunity over the air interface in each direction (i.e. DL or UL). The BAT offset can take a positive or a negative values.

If the NG-RAN receives a TSCAI containing a Periodicity Range or the Capability for Periodicity adaptation for a QoS Flow, the NG-RAN can determine a proposed Periodicity along with above specified BAT offset, in order to align the arrival of the traffic bursts with the next expected transmission opportunity over the air interface in each direction (i.e. DL or UL).

NG-RAN may support the following feedback mechanisms:

-	Proactive RAN feedback for adaptation of Burst Arrival Time and Periodicity: NG-RAN may provide a Burst Arrival Time offset, Periodicity as part of QoS flow establishment or modification as illustrated in clause 5.27.2.5.2;

-	Reactive RAN feedback for Burst Arrival Time adaptation: NG-RAN may provide a Burst Arrival Time offset after QoS flow establishment as illustrated in clause 5.27.2.5.3.

5.27.2.5.2	Proactive RAN feedback for Burst Arrival Time offset and Periodicity

If the NG-RAN receives a Burst Arrival Time and either the capability for BAT adaptation or a Burst Arrival Time Window in the TSCAI for a QoS Flow, the 5GS will perform the following actions:

-	The NG-RAN can determine a BAT offset in order to align the expected arrival of the traffic bursts (as indicated in the BAT) with the time when the next transmission over the air interface in each direction (i.e. DL or UL) is expected. Alternatively, NG-RAN may choose to not send 'BAT offset' in response if AF provided BAT is acceptable by NG-RAN. If BAT window was included in TSCAI, then the BAT offset shall always be provided by NG-RAN and it shall be within the BAT Window. The BAT offset is calculated with reference to earliest arrival time of received BAT Window.

-	The BAT offset is provided from NG-RAN to the SMF in the response to the QoS Flow establishment or modification request. The SMF provides the BAT offset to the PCF and the PCF notifies the AF as described in clause 6.1.3.23a of TS 23.503 [45].

NOTE:	It is assumed that the feedback from RAN implies the RAN accepts the BAT offset.

-	If the RAN also receives either the capability for Periodicity adaptation or a Periodicity Range along with the Periodicity in the TSCAI for a QoS flow, the 5GS will further perform the following actions:

-	The RAN may provide a periodicity feedback together with a BAT offset mentioned above. The periodicity feedback shall be within the Periodicity Range (if available) and the BAT offset is based on the proposed periodicity.

-	If interworking with a TSN network deployed in the transport network is supported, the SMF/CUC uses the periodicity and BAT offset accepted by the RAN to adjust the EarliestTransmitOffset and LatestTransmitOffset in the Talker/Listener Group in IEEE 801.Qcc [95] as described in clause 5.28a.2.

5.27.2.5.3	Reactive RAN feedback for Burst Arrival Time adaptation

If the RAN receives the capability for BAT adaptation without Burst Arrival Time in the TSCAI and notification control is enabled for this QoS Flow, the 5GS will perform the following actions:

-	If NG-RAN determines that the PDB of the QoS flow cannot be fulfilled in DL direction, then if supported, NG-RAN shall determine a BAT offset value which reduces the time between the arrival of the traffic bursts and the time of the next possible transmission over the air interface. NG-RAN shall not provide a BAT offset with the same value until the PDB of the QoS Flow can be fulfilled again.

NOTE:	NG-RAN determines BAT offset value in reference to the current arrival time of the bursts experienced by RAN.

-	The BAT offset is provided from NG-RAN to the SMF when sending the notification towards the SMF that the "GFBR can no longer be guaranteed" described in clause 5.7.2.4. The SMF provides the BAT offset to the PCF and the PCF provides the BAT offset to the AF as part of notifying the AF as described in clause 6.1.3.23a of TS 23.503 [45]

Editor's note:	UL BAT adaptation is subject to feedback from RAN WG2.

# * * * * End of changes * * * *