3GPP TSG-SA2 Meeting #154 Adhoc	S2-2301208

# EMEETING, 16th – 22nd January, 2023

Source:	Huawei, Hisilicon

Title:	Solve open ENs

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	FS_PIN / Rel-18

Abstract: this paper proposes to update the conclusions on all KIs related to open ENs

1. Introduction

This paper proposes to solve the open EN

2)	PIN ID is uniquely identifiable within the 5G network,

NOTE 2:	It will be determined during the normative phase whether the PIN ID is defined as an external PIN identifier in similar way of the External Group Identifier and GPSI, and an 5G internal PIN identifier similar as Internal Group Identifier.

Editor's note:	It is FFS whether 5GC knows PINE ID.

As the PINE is managed by the PEMC and the PINE may not be the 5GC equipment, so there is no need for the 5GC to know the PINE ID.

The Key Issue #7 conclusion includes the following editor’s note.

Editor's note:	It is FFS whether 5GC knows PINE ID.

The SA1 requirements are clear that 5G System should support identifying PINEs in order to “to create charging data” and so that “Network settings can be adapted and services offered to users according to their needs, independent of the subscription that is used to establish the connection”. Therefore, this paper proposes to remove the editor’s note and add a statement that “The identity of the PINEs that join a PIN are reported to the 5GC and stored in the PIN Profile”.

As stated in the FS_PIN SID (SP-211643), the PIN requirements come from several SA1 work items, including UIA (User Identities and Authentication). The UIA requirements are in section 26A of TS 22.101. The text box below shows the introduction section from section 26A of TS 22.101, this text explains some of the reasons why PINEs need to be identified and the benefits that will be gained by network operators and 3rd party service providers. Highlighting is added on requirements that are particularly related to Key Issue #7.

Identifying distinguished user identities of the user (provided by some external party or by the operator) in the operator network enables an operator to provide an enhanced user experience and optimized performance as well as to offer services to devices that are not part of a 3GPP network. The user to be identified could be an individual human user, using a UE with a certain subscription, or an application running on or connecting via a UE, or a device (“thing”) behind a gateway UE.

Network settings can be adapted and services offered to users according to their needs, independent of the subscription that is used to establish the connection. By acting as an identity provider, the operator can take additional information from the network into account to provide a higher level of security for the authentication of a user.

NOTE: The basic concept and relations of user identity management is described in TR 22.904 [61].

In terms of identifying PINEs, TS 22.101 includes the following requirements which are important for Key Issue #7.

The 3GPP system shall be able to provide User Identities with related User Identifiers for a user.

The User Identifier shall be independent of existing identifiers relating to subscription or device (e.g. IMSI, MSISDN, IMPI, IMPU, SUPI, GPSI, IMEI) and of other User Identifiers.

The User Identifier may be provided by some entity within the operator’s network or by a 3rd party.

The 3GPP system shall support to interwork with a 3rd party network entity for authentication of the User Identity.

The 3GPP system shall support to perform authentication of a User Identity regardless of the user's access, the user's UE and its HPLMN as well as the provider of the User Identifier.

The 3GPP network shall be able to provide a User Identifier for a non-3GPP device that is connected to the network via a UE that acts as a gateway.

The 3GPP network shall support to perform authentication of a User Identity used by devices that are connected via a UE that acts as a gateway.

The 3GPP system shall be able to take User Identity specific service settings and parameters into account when delivering a service.

A subscriber shall be able to link and unlink one or more user Identities with his 3GPP subscription.

The 3GPP system shall support user authentication with User Identifiers from devices that connect via the internet; the 3GPP system shall support secure provisioning of credentials to those devices to enable them to access the network and its services according to the 3GPP subscription that has been linked with the User Identity.

The 3GPP system shall support secure provisioning of credentials to a non-3GPP device connected via a gateway UE, whose User Identifier has been linked with the 3GPP subscription of the gateway UE, to enable the non-3GPP device to access the network and its services according to the linked 3GPP subscription when connected via non-3GPP access.

The 3GPP system shall be able to assess the level of confidence in the User Identity by taking into account information regarding the used mechanism for obtaining that User Identity (e.g. algorithms, key-length, time since last authentication), information from the network (e.g. UE or device in use, access technology, location).

The operator and the subscriber shall be able to restrict the number of simultaneously active User Identifiers per UE.

The 3GPP system shall be able to create charging data containing the User Identifier and the subscription to which it is linked for access and use of network services by a non-3GPP device that was authorized with its User Identifier linked to a subscription

Note that identifying PNEs is also necessary for sharing purposes. For example, TS 22.115 states:

The SA1 requirements are clear that 5G System should support identifying PINEs in order to “to create charging data” and so that “Network settings can be adapted and services offered to users according to their needs, independent of the subscription that is used to establish the connection”.

There is one EN under the conclusions for KI#7

Editor's note:	It is FFS whether 5GC knows PINE ID.

Based on the conclusions for KI#4, the AF for PIN can, based on exposure of the capabilities from 5GC, the steer QoS and routing of the PIN traffic. It’s also agreed that the AF for PIN may provide necessary parameters to 5GC to generate URSP for PEGC or PEMC.

The conclusion for KI#4 further indicates that one PDU session is used for one PIN while one PEGC may supports multiple PINs. Based on this, we consider the granularity to control/steer PIN traffic is sufficient at the PIN level and no need to reach each individual PIN element.

Based on above, it’s proposed to support the conclusions below for KI#7.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-88.

# * * * * First change * * * *

8.7	Conclusion on Key Issue #7

The following principles are concluded for Key Issue #7 "Identification of PIN and PIN Elements":

1)	NEF/UDM supports storing PIN ID in UDR, which may be received from AF.

NOTE 1:	Whether external and internal PIN ID are needed, and who allocates the PIN ID, are determined in normative phase.

2)	PIN ID is uniquely identifiable within the 5G network,

NOTE 2:	It will be determined during the normative phase whether the PIN ID is defined as an external PIN identifier in similar way of the External Group Identifier and GPSI, and an 5G internal PIN identifier similar as Internal Group Identifier.

3a)	The identity of the PINEs that join a PIN are reported to the 5GC and stored in the PIN Profile.

3b)	PINE ID awareness in 5GC is not needed.

# * * * * End of changes * * * *