3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2301216r02

# Elbonia, 16-20 January 2023	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0889 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting trade-off of QoE and power saving requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on conclusions of FS_XRM on KI#9 in TR 23.700-60 clause 8.9, the PCF can get the media codec information from the AF, and generate or update the PCC rules considering the application information (i.e. media codec information). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of media codec information to the list of inputs for PCC decisions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 specifications not aligned with the Rel-18 study conclusions on trade-off between QoE and power savings, captured in TR 23.700-60 clause 8.9. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Begin of changes * * * *

#### 6.2.1.2	Input for PCC decisions

The PCF shall accept input for PCC decision-making from the SMF, the AMF, the CHF, the NWDAF if present, the UDR and if the AF is involved, from the AF, as well as the PCF may use its own predefined information. These different nodes should provide as much information as possible to the PCF. At the same time, the listed information below describes only examples of the information provided by the respective node and is not intended to be complete. Depending on the particular scenario all the information may not be available or is already provided to the PCF.

The AMF may provide information related to the UE as defined in clause 5.2.5.2 of TS 23.502 [3], for example:

-	SUPI;

-	PEI of the UE;

-	Location of the subscriber;

-	Service Area Restrictions;

-	RFSP Index;

-	RAT Type;

-	GPSI;

-	Access Type;

-	Serving Network identifier (PLMN ID or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]);

-	Allowed NSSAI;

-	UE time zone;

-	Subscribed UE-AMBR;

-	Configured NSSAI for the serving PLMN;

-	Mapping Of Allowed NSSAI;

-	S-NSSAI for the PDU Session;

-	Requested DNN.

NOTE 1:	The Access Type and RAT Type parameters should allow extension to include new types of accesses.

The UE may provide information such as:

-	OSId;

-	List of PSIs;

-	Indication of UE support for ANDSP.

The SMF may provide information related to the PDU Session as defined in clause 5.2.5.4 of TS 23.502 [3], for example:

-	SUPI;

-	PEI of the UE;

-	IPv4 address of the UE;

-	IPv6 network prefix assigned to the UE;

-	Default 5QI and default ARP;

-	Request type (initial, modification, etc.);

-	Type of PDU Session (IPv4, IPv6, IPv4v6, Ethernet, Unstructured);

-	Access Type;

-	RAT Type;

-	GPSI;

-	Internal-Group Identifier;

-	Location of the subscriber;

-	S-NSSAI;

-	DNN;

-	Serving Network identifier (PLMN ID or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]);

-	Application Identifier;

-	Allocated application instance identifier;

-	Detected service data flow descriptions;

-	UE support of reflective QoS (as defined in clause 5.7.5.1 of TS 23.501 [2]);

-	Number of supported packet filters for signalled QoS rules for the PDU Session (indicated by the UE as defined in clause 5.7.1.4 of TS 23.501 [2]);

-	3GPP PS Data Off status;

-	DN Authorization Profile Index (see clause 5.6.6 of TS 23.501 [2]);

-	DN authorized Session AMBR (see clause 5.6.6 of TS 23.501 [2]);

-	Satellite backhaul category information;

-	Provisioning Server address(es) (see clause 5.30 of TS 23.501 [2]).

The UDR may provide the information for a subscriber connecting to a specific DNN and S-NSSAI, as described in the clause 6.2.1.3.

The UDR may provide policy information related to an ASP as defined in clause 5.2.12.2 of TS 23.502 [3], for example:

-	The ASP identifier;

-	A transfer policy together with a Background Data Transfer Reference ID, the volume of data to be transferred per UE, the expected amount of UEs;

-	An PDTQ policy together with an PDTQ Reference ID, the requested QoS for each of the AF session for each of the UEs involved and the expected amount of UEs.

NOTE 2:	The information related with AF influence on traffic routing may be provided by UDR when the UDR serving the NEF is deployed and stores the application request.

The UDR may provide the service specific information as defined in clause 4.15.6.7 of TS 23.502 [3].

The AF, if involved, may provide application session related information as defined in clause 5.2.5.3 of TS 23.502 [3] directly or via NEF, e.g. based on SIP and SDP, for example:

-	Subscriber Identifier;

-	IP address of the UE;

-	Media Type;

-	Media Format, e.g. media format sub-field of the media announcement and all other parameter information (a= lines) associated with the media format;

-	Bandwidth;

-	Sponsored data connectivity information;

-	Flow description information, e.g. source and destination IP address and port numbers and the protocol and optionally, ToS (IPv4) or TC (IPv6) value (as described in clause 6.1.3.6);

-	Media Codec Information (e.g. Media Codec Type, Media Codec Configuration Currently in Use).

NOTE 3:	Media Codec Information is a value that represents the requirements of the current codec setting (e.g. latency and throughput).

Editor’s Note: 	How the AF assigns a value to the identifier which represents the Media Codec Information and understands how the value maps to requirements (e.g. latency and throughput) is FFS (e.g. whether it is based on SLA, left to stage-3, etc).

-	AF application identifier, i.e. an identifier that refers to the application the AF session belongs to, containing either an AF identifier, an external application identifier (if the NEF is involved and performs the mapping to the application identifier) or an application identifier (if the AF is configured accordingly);

NOTE 4:	Either Flow description or (external) application identifier for application detection control can be provided.

-	DNN and possibly S-NSSAI;

-	AF Communication Service Identifier (e.g. IMS Communication Service Identifier), UE provided via AF;

-	AF Application Event Identifier;

-	AF Record Information;

-	Flow status (for gating decision);

-	Priority indicator, which may be used by the PCF to guarantee service for an application session of a higher relative priority;

NOTE 5:	The AF Priority information represents session/application priority and is separate from the MPS 5GS Priority indicator.

-	Emergency indicator;

-	Application service provider;

-	DNAI;

-	Information about the N6 traffic routing requirements;

-	GPSI;

-	Internal-Group Identifier;

-	Temporal validity condition;

-	Spatial validity condition;

-	AF subscription for early and/or late notifications about UP management events;

-	AF transaction identifier;

-	TSC individual QoS information as described in clause 6.1.3.22;

-	QoS information to be monitored;

-	Service area coverage;

-	Indication that high throughput is desired;

-	Reporting frequency;

-	User Plane Latency Requirement.

The AF may provide BDT related information as defined in clause 5.2.5.5 of TS 23.502 [3] via NEF, for example:

-	Background Data Transfer Reference ID;

-	BDT Policy;

-	Volume per UE;

-	Number of UEs;

-	Desired time window;

-	Network Area Information.

The CHF, if involved, may provide the following information for a subscriber as defined in clause 5.2.5.17 of TS 23.502 [3], for example:

-	Policy counter status for each relevant policy counter.

The NWDAF, if involved, may provide analytics information as described in clause 6.1.1.3.

In addition, the predefined information in the PCF may contain additional rules based on charging policies in the network, whether the subscriber is in its home network or roaming, depending on the QoS Flow attributes.

The 5QIs (see clause 5.7.4 of TS 23.501 [2]) in the PCC rule is derived by the PCF from AF or UDR interaction if available. The input can be SDP information or other available application information, in line with operator policy.

The Allocation and Retention Priority in the PCC Rule is derived by the PCF from AF or UDR interaction if available, in line with operator policy.

# * * * * End of changes * * * *