3GPP TSG-WG SA2 Meeting #154-AH-e	S2-2301261r03

# Jan 16 – 20, 2023, Elbonia

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0291 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LCS User Plane Function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-71 has concluded the LCS User Plane support. This CR further provides some details of the UP function in TS 23.273. It is necessary to add the UP part to complete the solution in the TS 23.273. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add some general description for the user plane positioining. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The functional descrioption is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 4.2.2, 4.3.5, 4.3.6, 4.3.7, 5.13, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change  * * * *

## 4.2	Architectural Reference Model

### 4.2.1	Non-roaming reference architecture

Figure 4.2.1-1 shows an architectural reference model for 5GS LCS for a non-roaming UE in reference point representation.

Figure 4.2.1-1: Non-roaming reference architecture for Location Services in reference point representation

NOTE 1:	(R)AN represents NG-RAN, trusted non-3GPP access or untrusted non-3GPP access.

NOTE 2:	Reference point interface related to charging functionality is not shown in this specification.

Figure 4.2.1-2 shows an architectural reference model for 5GS LCS for a non-roaming UE in SBI representation.

Figure 4.2.1-2: Non-roaming reference architecture for Location Services in SBI representation

### 4.2.2	Roaming reference architecture

Figure 4.2.2-1 shows an architectural reference model for 5GS LCS for a roaming UE in reference point representation.

Figure 4.2.2-1: Roaming reference architecture for Location Services in reference point representation

NOTE 1:	(R)AN represents NG-RAN, trusted non-3GPP access or untrusted non-3GPP access.

NOTE 2:	Reference point interface related to charging functionality is not shown in this specification.

Figure 4.2.2-2 shows an architectural reference model for 5GS LCS for a roaming UE in SBI representation.

Figure 4.2.2-2: Roaming reference architecture for Location Services in SBI representation

# * * * * Next change * * * *

### 4.3.5	UE

A target UE may support positioning according to four different modes:

-	UE assisted mode (the UE obtains location measurements and sends the measurements to another entity (e.g. an LMF) to compute a location);

-	UE based mode (the UE obtains location measurements and computes a location estimate making use of assistance data provided by serving PLMN);

-	standalone mode (the UE obtains location measurements and computes a location estimate without making use of assistance data provided by serving PLMN);

-	network based mode (a serving PLMN obtains location measurements of signals transmitted by a target UE and computes a location estimate).

NOTE:	The transmission of UE signals for network based mode may or may not be transparent to the UE.

Positioning procedures used by a UE for NG-RAN access are described in TS 38.305 [9].

A limited set of UE positioning capabilities can be transferred to the 5GCN during registration of the UE as described in TS 24.501 [11]. Some of these positioning capabilities may be transferred subsequently to an LMF as described in TS 29.572 [12]. UE positioning capabilities may also be transferred directly to a location server (e.g. LMF).

Additional functions which may be supported by a UE to support location services include the following.

-	Support location requests received from a network for 5GC-MT-LR, 5GC-NI-LR or a deferred 5GC-MT-LR for periodic or triggered location.

-	Support location requests to a network for a 5GC-MO-LR.

-	Support privacy notification and verification for a 5GC-MT-LR or deferred 5GC-MT-LR for periodic or triggered location.

-	Send updated privacy requirements to a serving AMF (for transfer to a UDR via UDM).

-	Support periodic or triggered location reporting to an LMF.

-	Support change of a serving LMF for periodic or triggered location reporting.

-	Support cancelation of periodic or triggered location reporting.

-	Support multiple simultaneous location sessions.

-	Support the reception of unciphered and/or ciphered assistance data broadcast by NG-RAN.

-	Support the reception of ciphering keys for the assistance data from the AMF.

-	Support handling of 5GC-MT-LR, 5GC-NI-LR, 5GC-MO-LR and deferred 5GC-MT-LR for periodic or triggered location over a user plane connection between UE and LMF.

-	Support user plane capability in registration procedures with AMF

-	Support reporting of location events for a periodic or triggered 5GC-MT-LR over a user plane connection to an LCS Client or AF with periodic cumulative event reports being sent over control plane to the LMF, H-GMLC and LCS Client or AF.

# * * * * Next change * * * *

### 4.3.6	UDM

The UDM contains LCS subscriber LCS privacy profile and routing information. The UDM is accessible from an AMF, GMLC or NEF via the Nudm interface.

Within UE’s subscription data, UDM also stores the information showing whether UE is authorized to use user plane positioning with LMF.

# * * * * Next change * * * *

### 4.3.7	Access and Mobility Management Function, AMF

The AMF contains functionality responsible for managing positioning for a target UE for all types of location request. The AMF is accessible to the GMLC and NEF via the Namf interface, to the RAN via the N2 reference point and to the UE via the N1 reference point.

Functions which may be performed by an AMF to support location services include the following.

-	Initiate an NI-LR location request for a UE with an IMS emergency call or to know a UE geographical area with NR satellite access for PLMN selection verification.

-	Receive and manage location requests from a GMLC for a 5GC-MT-LR and deferred 5GC-MT-LR for periodic, triggered and UE available location events.

-	Receive and manage location requests from a UE for a 5GC-MO-LR.

-	Receive and manage Event Exposure request for location information from an NEF.

-	Select an LMF.

-	Receive updated privacy requirements from a UE and transfer to a UDR via UDM.

-	Support cancelation of periodic or triggered location reporting for a target UE.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	When assistance data is broadcast by 5GS in ciphered form, the AMF receives ciphering keys from the LMF and forwards to suitably subscribed UEs using mobility management procedures.

-	Store UE Positioning Capability received from an LMF and send the UE Positioning Capability along with the received location request to an LMF. UE Positioning Capability includes the user plane positioning capability of the UE.

NOTE:	Details of UE Positioning Capability is defined in TS 37.355 [20].

-	Store user plane positioning capability of UE during registration phase and include it in the UE Positioning Capability along with the received location request to an LMF.

-	Query whether user plane positioning is authorized to UE from UDM and provide this information to LMF along with the received location request.

# * * * * Next change * * * *

## 5.13	Support of location service in PNI-NPN with signalling optimisation

Support of location service in PNI-NPN is based on the PNI-NPN description defined in clause 5.30.3 of TS 23.501 [18]. PNI-NPN architecture described in Annex C are supported.

Under the PNI-NPN architecture described in Annex C, the relevant Network Functions are defined as follows:

-	Local AMF, an AMF type function deployed in the local network, it supports:

-	Functionalities related to AMF service operation "NonUeN2MessageTransfer" within Namf_Communication service, defined in TS 29.518 [16].

-	Handling "UL and DL NON UE ASSOCIATED NRPPA TRANSPORT", defined in TS 38.413 [38].

NOTE 1:	Local AMF can support other functionalities defined in TS 23.501 [18].

NOTE 2:	It is assumed only one local AMF is deployed in the local network.

-	LMF, deployed in the local network, it supports:

-	Pre-configured with local AMF FQDN or communication address.

-	For DL UE associated NRPPa signalling, sends the signaling message to the serving AMF.

-	For DL NON UE associated NRPPa signalling, sends the signaling message to the local AMF.

-	Reporting location result towards (local) GMLC using Nlmf_Location_EventNotify service operation.

NOTE 3:	Local LMF is serving LMF for the support of location service in PNI-NPN.

-	NG-RAN, deployed in the local network, it supports:

-	Pre-configured with local AMF FQDN or communication address.

-	Establishes TNL association with local AMF.

-	GMLC, deployed in the local network.

NOTE: Not all UEs in local network use local LMF. It depends on the decision of serving AMF as defined in clause 5.1.

When UE access the NG-RAN in the local network, during the registration procedure, NG-RAN selects the AMF in the public network. With appropriate configuration, local AMF cannot be selected as the serving AMF for the UE.

During the positioning procedure, if LMF determines network-based positioning method, the positioning procedure defined in clause 6.11.2 is used.

For both immediate MT-LR, and deferred MT-LR, when LMF determines UE location, LMF provides the UE location to GMLC, as defined in clause 6.3.1.

# * * * * Next change * * * *

Annex C (informative):
PNI-NPN architecture to support location service with signalling optimisation

Figure C-1 shows the PNI-NPN architecture to support location services with optimisation of signalling latency and privacy, with the corresponding functional descriptions are defined in clause 5.13.

When UE access the NG-RAN in the local network, during the registration procedure, NG-RAN selects the AMF in the public network. With appropriate configuration, local AMF cannot be selected as the serving AMF for the UE.

Assuming NG-RAN 1 is the serving RAN of UE, NG-RAN 2 and NG-RAN 3 illustrated in the figure below is for positioning signal measurement, and only supports NON UE NRPPA signalling transmission.

Figure C-1 PNI-NPN architecture to support location service with signalling optimisation

# * * * * End of changes * * * *