3GPP TSG-SA2 Meeting #154AHE (e-meeting)	S2-2301270

# Jan 16-20, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0689 | rev | 0 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MTLF-based ML Model Accuracy Monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph3 |  |  |  |  |  | Date: |  |  | 2023-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MTLF-based ML model accuracy monitoring has been agreed in the conclusions of TR 23.700-81 and needs to be implemented in TS 23.288 rel 18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2E (new) introduces MTLF-based ML model accuracy monitoring, and it is further split into: - 6.2E.1 General, to provide overview of 2 procedures - 6.2E.2 Procedure for MTLF-based ML Model Accuracy Mornitoring, to specify the procedure without AnLF assistance - 6.2E.3 Procedure for AnLF-assisted MTLF-based ML Model Accuracy Monitoring, to specify the procedure with AnLF assistance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MTLF-based ML Model Accuracy Monitoring is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E (new clause) 6.2E.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

## 6.2E	MTLF-based ML Model Accuracy Monitoring

### 6.2E.1	General

MTLF-based ML Model Accuracy Monitoring procedure is specified in clause 6.2E.2, where MTLF determines ML Model degradation based on newly collected test data and retrain the existing ML Model. Clause 6.2E.3 specifies the procedure for AnLF-assisted MTLFML Model Accuracy Monitoring, where MTLF determines ML Model degradation based on analytics accuracy information provided by AnLF and retrain the existing ML Model.

### 6.2E.2	Procedure for MTLF-based ML Model Accuracy Mornitoring

Figure 6.2E.2-1 illustrates the procedure for monitoring the accuracy of an ML model using newly collected data. NWDAF containing AnLF may provide inference data to NWDAF containing MTLF for model accuracy monitoring and retraining.

Figure 6.2E.3-1: Procedure for MTLF-based ML Model Accuracy Monitoring

1.	The NWDAF containing AnLF requests an ML model from the appropriate NWDAF containing MTLF, using the MLModelProvision_Subscribe service operation. NWDAF containing AnLF may include the Monitoring Indicator to indicate to MTLF to execute the accuracy monitoring operations. NWDAF containing AnLF may include a DataSetTag, which is used to store and fetch the inference data from ADRF which are relevant for the accuracy monitoring and retraining.

NOTE. If an ML model is already provisioned by the MTLF to the AnLF, then AnLF may update the subscription for the ML Model to include the Monitoring indicator and the DataSetTag.

2.	The NWDAF containing MLTF provides the ML model to the AnLF as per current specifications.

3.	The NWDAF containing MTLF collects new data for ML model accuracy monitoring and retraining from various data source NFs. The NWDAF containing MTLF may subscribe the inference data used by AnLF from ADRF, if it receives a DataSetTag from the AnLF in step 1.

4.	Based on the collected analytics and data, the MLTF computes the accuracy using the predictions and the actual measured data observed at the time which the prediction refers to.

5.	An accuracy report is sent to the AnLFwhen the reporting threshold is met.. The accuracy report is sent in an MLModelProvision notification from MTLF.

6.	Based on the computed accuracy, MLTF may decide to re-train the ML model.

7.	When the newly generated ML model is ready, the MTLF notifies the AnLF about the new ML model .

* * * * End of Changes * * * *