3GPP TSG-SA2 Meeting #154	S2-2210299

# Toulouse, France, Nov 14 - 18, 2022											 (revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3760 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of uplink-downlink transmission coordination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Tencent, Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusion for uplink-downlink transmission coordination to meet Round-Trip latency requirements has been agreed in TR 23700-60. In order to satisfy the RT latency requirement provided by AF, the 5GS shall split the RT latency into an UL PDB and a DL PDB. Both UL/DL can be unequal but their sum shall not exceed the RT lantency. If the PCF splits the RT latency, the PCF shall generate two PCC rules for the UL/DL media streams, and assign the 5Qis according to the UL/DL PDBs respectively. The two PCC rules shall also have associated QoS monitoring policies to enable RT latency tracking. This enables the UL/DL PDB to be tracked independently. Based on the QoS monitoring result, the PCF can adjust the UL PDB and the DL PDB to better fit the new situation. In addition, different 5QIs shall apply to different QoS Flows as defined in current QoS model. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds uplink-downlink transmission coordination to meet the Round-Trip latency requirements of XRM service based on the TR conclusion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No able to support uplink-downlink transmission coordination to address the Round-Trip latency requirements of XRM service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_XRM and the WI code for the work is not assigned yet & will replace WI code DUMMY when available |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

5.X	Support for Extended Reality and Media Services

5.x.y Uplink-Downlink Transmission Coordination

For XR/media services which require very low Round-Trip latency, Uplink-Downlink Transmission Coordination is supported to meet the RT (Round-Trip) latency requirement. Uplink-Downlink Transmission Coordination is enabled at PCF by the local configuration or by a RT coordination indication from AF.

RT latency requirement is the upper bound for the sum of UL delay and DL delay of a media stream between UE and N6 termination point at the UPF.  When Uplink-Downlink Transmission Coordination is enabled, the RT latency requirement is the doubling of the single direction delay requirement which is equal for UL and DL preconfigured in PCF or provided by the AF. The delay requirement of single direction from the AF is a QoS reference parameter or a Requested PDB (as defined in clause 6.1.3.22 of TS 23.503 [45]). The AF provides the RT coordination indication to the PCF during AF session with required QoS procedure as defined in clause 4.15.6.6 of TS 23.502 [3].

PCF decides the UL PDB and the DL PDB for the media stream according to the RT latency requirement, the UL PDB and DL PDB can be unequal but their sum shall not exceed the RT latency requirement. The PCF shall generate two PCC rules for the media stream, one for the UL SDF and one for the DL SDF and assigns the 5QIs according to the UL PDB and DL PDB respectively. For the PCC rule of the UL SDF, QoS monitoring is enabled to track the UL packet delay, for the PCC rule of the DL SDF, QoS monitoring is enabled to track the DL packet delay. Based on the QoS monitoring result, the PCF adjusts the UL PDB and the DL PDB under limitation of the RT latency requirement.

* * * End of Changes * * *