3GPP TSG-WG SA2 Meeting #154 	S2-2210343

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3: Conclusion update

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract: It is proposed to update the conclusion of KI#3.

1. Introduction/Discussion

There are multiple options to resolve KI#3, the main idea is to let the UE know about the validation criteria (i.e. time window and location) of the service thus the UE can be aware of the unavailability of the service in advance:

For support of limited AoS slices not matching deployed TAs:

Option 1 is to introduce multi-level TA. Then the UE can know the valid location (per) Based on RAN2/3 feedback, Option 1 has big RAN and UE AS layer impact.

The purpose of Option 4 is the same as Option 2, i.e. to let the UE know the area of service but there is no need to introduce new parameters. In details, as described in TS 24.526, the Location Criteria has already support cell level handling. So no new mechanism is required.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of E-UTRA cell identities |  |  |  |  |  |  |  | octet e+1 |
| E-UTRA cell id 1 |  |  |  |  |  |  |  | octet e+2 octet e+8 |
| E-UTRA cell id 2 |  |  |  |  |  |  |  | octet e+9 octet e+15 |
| … |  |  |  |  |  |  |  | octet e+16 octet j-1* |
| E-UTRA cell id n |  |  |  |  |  |  |  | octet j* octet f=(j+6)* |

Figure 5.2.7: Location area contents {Type of location area = E-UTRA cell identities list}

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of NR cell identities |  |  |  |  |  |  |  | octet e+1 |
| NR cell id 1 |  |  |  |  |  |  |  | octet e+2 octet e+9 |
| NR cell id 2 |  |  |  |  |  |  |  | octet e+10 octet e+17 |
| … |  |  |  |  |  |  |  | octet e+18 octet k-1* |
| NR cell id n |  |  |  |  |  |  |  | octet k* octet f=(k+7)* |

Figure 5.2.8: Location area contents {Type of location area = NR cell identities list}

Option 3 is pure network configuration. It is possible to have some optimization in the RAN or CN side on slice configuration. Besides, based on the LS response R3-226083, all the options will be affected by the regardless of the slice support of the target NG-RAN node during handover due to the handover (currently, only the signal quality of the neighbour cell is considered). Thus, UE mobility, e.g. handover should also be taken into account.

For improved support of temporary network slices:

The purpose of Option 1,3 and 4 are the same as Option 2, i.e. to let the UE know the validity time window of the service （then the UE can release the related PDU Session (s) gracefully with its own flexibility） but there is no need to introduce new parameters.

In details, it is also noted that “Temporary network slices does not mean that the network slices are decommissions and created as per the timing information, but the network slices are not meant to be available for use by the UE”. So the S-NSSAI is still available but the UE cannot use the service of it. The URSP

For the graceful and gradual termination aspect:

Besides the above, it should be highlighted that the validation criteria shall not be obligatory as the UE should be able to make its own decision based on the application layer/OS Layer (traffic status) and release the PDU Sessions by itself.

In details, the URSP rule can fulfil the above requirement as described in TS 23.503:

- The UE will not establish new PDU Sessions if the UE is out of Time Window or Location Criteria.

The Route Selection Descriptor of a URSP rule shall be only considered valid if all of the following conditions are fulfilled:

-	If a Time Window is present and the time matches what is indicated in the Time Window.

-	If a Location Criteria is present and the UE location matches what is indicated in the Location Criteria.

- The UE will release the existing PDU Session in a timely manner if the UE is out of Time Window or Location Criteria.

When URSP rules are updated or their validity according to the conditions above change, the association of existing applications to PDU Sessions may need to be re-evaluated. The UE may also re-evaluate the application to PDU Session association due to the following reasons:

-	periodic re-evaluation based on UE implementation;

-	an existing PDU Session that is used for routing traffic of an application based on a URSP rule is released;

-	The expiration of Time Window in Route Selection Validation Criteria, i.e. the expiration of Time Window, or UE's location no longer matches the Location Criteria.

If the re-evaluation leads to a change of the application to PDU Session association, e.g. the application is to be associated with another PDU Session or a new PDU Session needs to be established, the UE may enforce such changes in a timely manner based on implementation, e.g. immediately or when UE enters CM-IDLE state.

It is proposed to update the conclusion based on the above discussion.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-41.

# * * * * First change * * * *

8.3	Conclusions for KI#3

Editor's note:	These are interim conclusions.

For support of limited AoS slices not matching deployed TAs it is proposed that:

Option 2: reuse existing URSP rules with per cell level granularity:

-	Clarify that URSP rules allows a per cell level granularity that the UE is required to validate before using a URSP including an S-NSSAI, and also that the UE to apply the Route Selection Validation Criteria also for already established PDU Sessions and their traffic. This would then ensure cell level granularity of service availability without any protocol impacts, unless URSP rules is to be explicitly enhanced with new indication that the UE is to apply the Route Selection Validation Criteria also for already established PDU Sessions and their traffic. If the UE does not support enhancement of URSP to apply the Route Selection Validation Criteria also for already established PDU Sessions and their traffic, SM policies are enhanced to release the PDU Sessions for the concerned S-NSSAI when the UE moves out of the AoS.

NOTE:	How the handover can be optimized to prevent the UE from leaving the slice service area (or entering into the slice service area) will be considered during normative phase based on RAN WG feedback.

Option 3: Reconfiguration of TAs:

-	Reconfiguration of TAs to keep the end-to-end significance of slice unchanged, but an operator configures the cells of a TA that are outside AoS to have no or limited resources using existing NG-RAN OAM configuration. The solution can be combined with a mechanism that limited the SM signalling for slices when the UE is outside the AoS e.g. Option 2.

For improved support of temporary network slices:

Option 3: Reusing existing URSP rule:

-	The PCF should generate URSP based on Temporary network slices related information stored in the UDR. UE can be aware of the validity timer of the slice or applicable area of the service in the validation criteria in URSP and manage the PDU Sessions for the related S-NSSAI accordingly based on the current design as described in clause 6.6.2.3 of TS 23.503 [12].

NOTE:	Temporary network slices does not mean that the network slices are decommissions and created as per the timing information, but the network slices are not meant to be available for use by the UE.

For the graceful and gradual termination aspect:

Option 1:

-	For decommission of a network slice, OAM and NF implementation (e.g. by updating URSP rules and to deregister not used S-NSSAIs) can handle the functionality without further standardization. Whether OAM provides timing information to an NF as to allow the NF to apply a change of resource utilization in advance, e.g. a time for how long time a Shutting Down state is to be valid, would be up to SA WG5.

-	UE can be aware of the validity timer of the slice or applicable area of the service in the validation criteria in existing URSP and release the PDU Sessions for the related S-NSSAI gracefully based on the current design as described in clause 6.6.2.3 of TS 23.503 [12].

# * * * * Second change * * * *

# * * * * Third change * * * *

# * * * * Fourth change * * * *

# * * * * Fifth change * * * *

# * * * * End of changes * * * *