SA WG2 Meeting #154	S2-2210377

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	Update of conclusion on KI#3

Document for:	Approval

Agenda Item:	9.14.1 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes to update conclusion on KI#3.

Discussion

This contribution proposes to release editor's notes in KI#3 conclusion.

1. For support of limited AoS slices not matching deployed TAs it is proposed that:

There are 4 candidate options. According to RAN feedback, Option 1 has many impacts on RAN so it is not preferred. Option 2 and Option 3 is re-using existing mechanism. Option 4 is providing information on cell-level information via NAS message. However, such information can be sent to the UE via URSP rule.

It is proposed to use Option 2, Option 3.

2. For improved support of temporary network slices:

There are 4 candidate options. For Option 1, it assumes that NG-RAN is configured with timing information and sent to the AMF, which has impact on the RAN. For Option 2, mandating re-evaluation of URSP rule causes burden to the UE so it is not recommended. Option 3 is re-using existing validity condition of URSP rule. Option 4 is providing timing information via NAS message. However, such information can be sent to the UE via URSP rule.

It is proposed to use Option 3.

3. For the graceful and gradual termination aspect:

There are 2 candidate options. For Option 1, there is no impact to the UE while for Option 2, timing information is provided to the UE via NAS message. However, timing information can be provided via URSP rule.

It is proposed to use Option 1.

There is editor's note on UDM enhancement. It is not clear why UDM needs to be enhanced since graceful termination of slice is not specific to a UE but decommission from the whole network. It is proposed to remove this EN.

Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change * * * *

8.3	Conclusions for KI#3

For support of limited AoS slices not matching deployed TAs it is proposed that:

Option 2: reuse existing URSP rules with per cell level granularity:

-	Clarify that URSP rules allows a per cell level granularity that the UE is required to validate before using a URSP including an S-NSSAI, and also that the UE to apply the Route Selection Validation Criteria also for already established PDU Sessions and their traffic. This would then ensure cell level granularity of service availability without any protocol impacts, unless URSP rules is to be explicitly enhanced with new indication that the UE is to apply the Route Selection Validation Criteria also for already established PDU Sessions and their traffic. If the UE does not support enhancement of URSP to apply the Route Selection Validation Criteria also for already established PDU Sessions and their traffic, SM policies are enhanced to release the PDU Sessions for the concerned S-NSSAI when the UE moves out of the AoS.

NOTE:	How the handover can be optimized to prevent the UE from leaving the slice service area (or entering into the slice service area) will be considered during normative phase based on RAN WG feedback.

Option 3: Reconfiguration of TAs:

-	Reconfiguration of TAs to keep the end-to-end significance of slice unchanged, but an operator configures the cells of a TA that are outside AoS to have no or limited resources using existing NG-RAN OAM configuration. The solution can be combined with a mechanism that limited the SM signalling for slices when the UE is outside the AoS e.g. Option 2.

For improved support of temporary network slices:

Option 3: Reusing existing URSP rule:

-	The PCF should generate URSP based on Temporary network slices related information stored in the UDR. UE can be aware of the validity timer of the slice or applicable area of the service in the validation criteria in URSP and manage the PDU Sessions for the related S-NSSAI accordingly based on the current design as described in clause 6.6.2.3 of TS 23.503 [12].

NOTE:	Temporary network slices does not mean that the network slices are decommissions and created as per the timing information, but the network slices are not meant to be available for use by the UE.

For the graceful and gradual termination aspect:

Option 1:

-	For decommission of a network slice, OAM and NF implementation (e.g. by updating URSP rules and to deregister not used S-NSSAIs) can handle the functionality without further standardization. Whether OAM provides timing information to an NF as to allow the NF to apply a change of resource utilization in advance, e.g. a time for how long time a Shutting Down state is to be valid, would be up to SA WG5.

-	UE can be aware of the validity timer of the slice or applicable area of the service in the validation criteria in existing URSP and release the PDU Sessions for the related S-NSSAI gracefully based on the current design as described in clause 6.6.2.3 of TS 23.503 [12].

* * * * End of Changes * * * *