SA WG2 Meeting #154	S2-2210378

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	Update of conclusion on KI#5

Document for:	Approval

Agenda Item:	9.14.1 Study on Network Slicing Phase 3

Work Item / Release:	FS_eNS_Ph3 / Rel-18

Abstract of the contribution: This paper proposes to update conclusion on KI#5.

Discussion

This contribution proposes to release editor's notes in KI#5 conclusion.

Editor's note:	It is FFS whether the list of TAIs associated with the rejected S-NSSAI includes TAs where the S-NSSAI is supported or not supported.

Number of TAIs provided together with rejected S-NSSAI is different according to the deployment and network configuration. It is proposed to have both options.

Editor's note:	It is FFS whether the list of TAIs associated with the partially allowed S-NSSAI includes TAs where the S-NSSAI is supported or not supported.

Editor's note:	The partially allowed S-NSSAI approach above is subject to further discussion and analysis also based on RAN feedback.

According to RAN feedback, in order to support partially allowed S-NSSAI, RAN impact is expected to handle handover. It is proposed to remove partially allowed S-NSSAI.

Proposal

It is proposed to agree the following changes into TR 23.700-41 on FS_eNS_Ph3.

* * * * Start of 1st Change * * * *

8.5	Conclusions for KI #5

The following principle is proposed to be the conclusion for normative work:

Partly rejected S-NSSAI:

-	AMF provides information related to the supported or not supported TA(s) regarding a rejected S-NSSAI in the RA to UE in the registration procedure and UE configuration update procedure if the UE indicates that it supports this feature. The UE shall be able to request a rejected S-NSSAI, by initiating a registration update procedure, in a supported TA based on the supported/not supported TA information associated with this S-NSSAI. The current concept of allowed NSSAI and uniform support of it in UE’s RA is not impacted by this feature based on indication of where in RA certain rejected S-NSSAIs are supported/not supported. In this approach, the Allowed NSSAI is still uniformly supported in the RA.

* * * * End of Changes * * * *