SA WG2 Meeting #154	S2-2210382

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Source:	LG Electronics

Title:	Update of conclusion on KI#4

Document for:	Approval

Agenda Item:	9.4.1 Study on enhanced support of Non-Public Networks phase 2

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes to update conclusion on KI#4.

Discussion

In KI#4 conclusion, there are two alternatives, Alt 1 and Alt 2. This was discussed during the conference call and there were different view on automatic hosting network selection.

Scenario A: Automatic hosting network selection is done without any user interaction, i.e. no need to consider localized services a user intends to receive.

Scenario B: Automatic hosting network selection considers localized services a user intends to receive

Scenario A: Some companies think that automatic network selection should be performed regardless of localized service, i.e. a UE selects a hosting network without considering a user's intention. This is similar as PLMN selection, which does not considers any services provided by PLMNs.

Scenario B: On the other hand, some companies think that hosting network selection should be performed considering a localized service selected by a user. For example, if a user selects localized service A, the UE should select a hosting network that supports the localized service A.

Following are requirements from TS 22.261, which gives some hints.

Subject to the automatic localized services agreements between the hosting network operator and home network operator, for UE with only home network subscription and with authorization to access hosting networks the 5G system shall support: - access to the hosting network and use home network services or selected localized services via the hosting network, - seamless service continuity for home network services or selected localized services when moving between two hosting networks or a host network and the home network. … The 5G system shall enable the home network to allow a UE to select a hosting network or change to another hosting network, without any additional user intervention as long as the delivered services, both localized services and home routed services, are unchanged.

According to above requirements, it seems that the same localized service can be provided to the UE via multiple hosting networks. This implies that selection of localized service may not result in selection of hosting network. Therefore, we proposes to consider Scenario B.

Regarding Alt 1 and Alt 2, we proposes to use both Alt 1 and Alt 2. The network selection information provided via SoR has prioritized network lists without any localized service information. The UE gets localized service information from serving network or hosting network via NAS signalling. The information contains mapping between localized service and supported hosting networks. Based on those information, the UE selects a hosting network that supports localized services a user wants to receive.

Proposal

It is proposed to agree the following changes into TR 23.700-08 on FS_eNPN_Ph2.

* * * * Start of 1st Change * * * *

8.4	Key Issue #4: Enabling UE to discover, select and access NPN as hosting network and receive localized services

8.4.1	General

The conclusion for KI #4 is made for each component that is evaluated in clause 7.4.

When UE accesses the Hosting network using the subscription/credentials of its Home network, only two cases are considered:

-	If Home network is PLMN, the Hosting network can be PNI-NPN or SNPN.

-	If Home network is SNPN, the Hosting network can be only SNPN

If the UE accesses the Hosting network using the other credentials rather than the subscription/credentials from the UE Home network, the determination of the subscription used to access the Hosting network is by implementation specific prior to automatic network selection as described in NOTE 1 of clause 5.30.2.4.2 of TS 23.501 [3].

8.4.2	Conclusion for the content of the localized service information

The following interim conclusions are reached.

The localized service information is provided to UE for UE to discover and select hosting network to receive desired localized service. The content of the localized service information can include the following elements:

a.	Identifier/name of the localized service.

b.	Validity conditions for the localized service, e.g. duration of remaining service operation ,time and location.

d.	Hosting network related information, per hosting network in the case of SNPN as hosting network:

i.	hosting network identifier, e.g. SNPN ID, GIN;

e.	Hosting network related information, per hosting network in the case of PNI-NPN as hosting network:

i.	PLMN ID of the PNI-NPN.

ii.	Optionally, if hosting network is associated with CAG IDs, a list of CAG IDs corresponding to the localized service.

The UE is provided with hosting network selection information:

a.	a list of prioritized hosting networks (i.e. SNPN ID or GIN) for localized services, in the case of SNPN as hosting network.

b.	allowed CAG ID list, in the case of PNI-NPN as hosting network and the PNI-NPN is associated with CAG ID.

NOTE:	There is no localized service information in the hosting network selection information. The UE receives localized service information via NAS signalling. The UE selects a hosting network that supports localized services the UE wants to use based on received hosting network selection information.

8.4.3	Conclusion for from where and how UE obtains the localized service information

The information for localized service and hosting network discovery, selection and access can also be obtained by UE at the application layer from the home network or the localized service provider via means that are outside of 3GPP scope.

The following interim conclusions are reached.

1.	The localized service information is formulated as application data that can be stored in the home network, or hosting network.

2.	The external parameter provisioning procedure in clause 4.15.6 of TS 23.502 [4] is extended to support the provisioning of localized service information to home, or hosting network, or the OAM method is used to provision the data to home network.

3.	The localized service information can be preconfigured in the UE or dynamically provisioned via signalling. For dynamic provisioning, the AMF could provide the UE requested localized service information (e.g. by specifying the identifier or name of the localized service) from either home network, hosting network or serving network via NAS signalling (e.g. UCU).

NOTE:	How UE obtains localized service identifier or name to make the request for dynamic provisioning is out of 3GPP scope.

4.	If PNI-NPN as hosting network is associated with CAG ID, separated from other localized service information, the UE obtains the Allowed CAG ID list from the home network according to clause 5.30.3.3 TS 23.501 [3].

a.	The home network may send the updated Allowed CAG ID list to the UE

5.	The information for localized service and hosting network discovery, selection and access can be preconfigured in the UE or dynamically provisioned by the hosting network or home network (via the VPLMN when roaming).

6.	In the case of SNPN as hosting network, the dynamic provisioning of prioritized list of hosting network information can be done via SoR.

NOTE:	How SOR-AF and/or UDM acquires hosting network information is outside the scope for 3GPP.

i.	The home network UDM may determine to update UE with prioritized list of hosting network information using SoR procedure. Following triggers may apply:

-	UE location as part of Registration procedure.

-	UE subscription data change, e.g. via external parameter provisioning.

7. When UE has accessed with the hosting network, the AMF can provide the latest localized service information to UE as part of Registration procedure (Mobility Update) or UE Configuration Update procedure.

8.	In the case of PNI-NPN as hosting network, the dynamic provisioning of allowed CAG ID list reuses existing procedure in clause 5.30.3.3 of TS 23.501 [3].

9.	Hosting network's NG-RAN may broadcast localized service information (e.g. service identifiers and/or human readable service information) to help UEs discover the service and the hosting network. via SIB information.

8.4.4	Conclusion for how the localized service information is used by UE

The following principles based on the evaluation in clause 7.4.4 are recommended for the normative work:

1.	If UE uses home network credential to access a hosting network:

a.	When the end user intends to access localized service and the validity conditions of localized service are met, the UE initiates hosting network selection using the hosting network related information received as part of the localized service information using the lists in clause 8.4.2.

i.	For SNPN as hosting network, the UE can switch between PLMN selection and hosting network selection following Rel-17 specification for SNPN selection. How the UE switches among the network selections is up to UE implementation. The UE selects an SNPN that supports localized service the end user intends to access.

ii.	For PNI-NPN as hosting network associated with CAG ID, the UE selects a CAG ID that supports localized service the end user intends to access. This may potentially initiate a new registration procedure to a PLMN.

NOTE 1:	Whether a new network selection mode is required for UE to initiate hosting network selection is to be determined by CT WG1.

NOTE 2:	Details regarding priority list for hosting network selection, including if a new selection mode is required, is up to CT WG1 to decide.

b.	Hosting network selection needs to be authorized by the home network, via UE initiated SoR procedure with SoR information. After the home network authorization, the UE is allowed to initiate hosting network selection, applicable for automatic hosting network selection.

i.	For manual hosting network selection, the UE presents available localized service information it has received in clause 8.4.3 to the end user.

ii.	The home network may reject the UE's request to access hosting network when the UE selected hosting network by manual selection if the home network does not want to accept the UE to access the selected hosting network.

Editor's note 4-1:	It is FFS whether the home network authorization is needed before the UE performs manual selection of a hosting network.

c.	When validity criteria of the localized service the end user intends to use are no longer met, the UE stops hosting network selection for the localized service.

2.	If the UE needs to obtain a new set of credentials/subscription to access the hosting network:

a.	It is up to UE implementation to decide how to switch to the new subscription profile for accessing hosting network.

3.	The UE determines SNPN access mode is activated/de-activated using implementation specific means as specified in existing Release 17, or using received localized service/hosting network assistance information as input.

8.4.5	Conclusion for what credentials are used to access hosting network and how to obtain them

The following principles based on the evaluation in clause 7.4.5 are recommended for the normative work in the case of SNPN as hosting network:

1.	The UE checks whether it is possible to re-use home network credentials to access the hosting network:

a.	If the hosting network related information indicates support of CH credentials, the UE determines that home network credential can be used if the SNPN ID of the hosting network is included in the SNPN priority lists associated with home network subscription.

b.	If the UE obtains hosting network selection information from the home network, the UE uses the credentials provided by the home network to access the hosting network.

2.	If new credentials for accessing hosting network are needed, the UE may use the existing credential onboarding mechanism as per Rel-17.

NOTE:	The method of delivering credentials needs to be evaluated by SA WG3.

The following principles based on the evaluation in clause 7.4.5 are recommended for the normative work in the case of PNI-NPN as hosting network:

3.	Only UEs equipped with a USIM configured with PLMN credentials can access a hosting network which is a PNI-NPN. When the UE requests to access the hosting network, the home PLMN credential(s) are used during authentication procedure.

8.4.6	Conclusion for how localized service is accessed in hosting network per agreed conditions

The following principles based on the evaluation in clause 7.4.6 are recommended for the normative work:

1.	Validity conditions provided to the UE as part of the localized service information can be used to restrict the UE's access of the hosting network.

2.	Existing methods, such as network slicing, forbidden area restriction, service area restriction, CAG, LADN, URSP rules can also be utilized to restrict UE's access, i.e. no need for additional normative work for access control in hosting network.

3.	In order to restrict access to a hosting network to a specific area, a hosting network operator may deploy and broadcast multiple hosting network IDs, i.e. SNPN IDs for SNPN case and CAG IDs in the case of PNI-NPN, in different areas depending on localized service area validity. Each localized service is mapped to a specific hosting network ID. Multiple localized service areas can be mapped to the same hosting network ID if their allowed service areas are the same. Validity conditions are also used by hosting network to restrict access.

* * * * End of Changes * * * *