3GPP TSG-SA WG2 Meeting #154	S2-2210384

# Toulouse, France, Novebmer 14 – 18, 2022	(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3766 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NSAG validity when the AMF does not provide associated TAI list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NRslice |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to CT1 LS on NSAG Validity (S2-2210186/C1-226030), CT1 made the decision that the NSAG is valid in the whole RPLMN and asked whether NSAG is also valid in the ePLMM(s) of the RPLMN. In clause 5.15.14, there is description on equivalent PLMNs but overall descriptions are not aligned, e.g. in green highlight, UE considers the received NSAG information for RPLMN while yellow highlighted text shows that UE stores NSAG information for EPLMNs. The UE shall store and consider the received NSAG Information, valid for the Registered PLMN or SNPN until: - the UE receives new NSAG information in a Registration Accept message or UE Configuration Command message in this PLMN or SNPN; or - the UE receives a Configured NSSAI without any NSAG information in this PLMN or SNPN. The UE shall store the currently valid NSAG information received in the registered PLMN or SNPN when registered in this PLMN or SNPN and: - The UE should be able to store the NSAG information for at least the R-PLMN and equivalent PLMNs, or the SNPN. - Only the RPLMN or the SNPN can provide NSAG information to the UE. - There can be at most 32 NSAGs configured in the UE at a time for a PLMN or SNPN. - At most 4 NSAGs can have an optional TAI associated with it. In addtion, green highlight text gives impression that AMF can provide NSAG infomration only when there is need to update it, i.e. the AMF stores the NSAG information provided to the UE. However, there is no description that the AMF stores NSAG informatio in the UE context. So, the AMF does not know whether NSAG information needs to be updated. Even though the AMF stores NSAG information in the UE context, it will not transferred between AMFs as there is no such requirement for now. Considering above aspects, it is proposed to clarify that the NSAG information is valid within the Registration Area if the AMF does not provide associated TAI list. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that NSAG informatio is valid within the Registration Area if the AMF does not provide associated TAI list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether NSAG infomration is valid outside of Registration Area if the AMF does not provide associated TAI list. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.15.14	Network Slice AS Groups support

The NG-RAN may support Network Slice AS Groups (NSAGs) which are used as specified in TS 38.300 [27], TS 38.331 [28], TS 38.321 [143] and TS 38.304 [50]. A Network Slice AS Group is an identifier of a group of network slices which are associated with it. A Network Slice AS Group association with a group of network slices may be valid in one or more Tracking Areas. An S-NSSAI can be associated with at most one NSAG values for Random Access and at most one NSAG value for Cell Reselection within a Tracking Area. An S-NSSAI can be associated with different NSAG values in different Tracking Areas.

The NG-RAN provides (and updates) the AMF with the values of the NSAG(s) an S-NSSAI is associated with in a TA using the NG Set Up and RAN Configuration Update procedures (see TS 38.413 [34]). The AMF in turn provides this information to the NSSF. In deployments where the total number of groups does not exceed the number of groups associated with the NSAG size limit defined in TS 38.331 [28]), all the NSAGs configured in the NG-RAN may be unique per PLMN or SNPN. If the UE has indicated that the UE supports NSAG in the 5GMM Core Network Capability (see clause 5.4.4a), the AMF may, with or without NSSF assistance, configure the UE with NSAG Information for one or more S-NSSAIs in the Configured NSSAI, by including this NSAG Information in the Registration Accept message or the UE Configuration Command message. The UE uses the NSAG Information as defined in clause 5.3.4.3.1. The AMF shall indicate in the NSAG Information in which TA a specific NSAG association to S-NSSAI(s) is valid if the AMF provides in the UE configuration a NSAG value which is used in different TAs with a different association with NSSAIs. If the AMF does not provide associated TAIs for a NSAG value, the UE considers that the received NSAG value is valid within the Registration Area. The configuration the AMF provides includes at least the NSAGs for the UE for the TAs of the Registration Area.

The UE shall store and consider the received NSAG Information, valid for the Registration Area until:

-	the UE receives new NSAG information in a Registration Accept message or UE Configuration Command message in this PLMN or SNPN; or

-	the UE receives a Configured NSSAI without any NSAG information in this PLMN or SNPN.

The UE shall store the currently valid NSAG information received in the registered PLMN or SNPN when registered in this PLMN or SNPN and:

-	The UE should be able to store the NSAG information for at least the R-PLMN and equivalent PLMNs, or the SNPN.

-	Only the RPLMN or the SNPN can provide NSAG information to the UE.

-	There can be at most 32 NSAGs configured in the UE at a time for a PLMN or SNPN.

-	At most 4 NSAGs can have an optional TAI associated with it.

The NSAG information is not required to be stored after power off or after the UE becomes Deregistered as it is not used for cell selection.

* * * * End of Changes * * * *