SA WG2 Meeting #154	S2-2210386

November 14 – 18, 2022, Toulouse, France	(revision of S2-221xxxx)

Source:	LG Electronics

Title:	KI#5, Conclusion update

Document for:	Approval

Agenda Item:	9.26.1

Work Item / Release:	FS_5G_ProSe_Ph2 / Rel-18

Abstract of the contribution: This paper updates conclusion for KI#5 "Support of multi-path transmission for UE-to-Network Relay".

Discussion

RAN3 made the following working assumption at their October RAN3#117bis-e meeting related to support of multi-path transmission for UE-to-Network Relay. (Please see RAN3_117bis-e_agenda_20221018_EOM1.zip)

WA: The multi-path authorization can be added in the 5G ProSe Authorized IE if it is needed.

SA2 concluded that multi-path transmission via direct Uu path and Layer 2 U2N Relay will be defined by RAN WGs as described in clause 8.5 of TR 23.700-33.

It is considered valuable to provide authorization information for multi-path transmission by AMF to NG-RAN so that the NG-RAN will be able to apply multi-path transmission via direct Uu path and Layer 2 U2N Relay only to the UEs authorized to perform it.

Observation: Authorization information for multi-path transmission from AMF to NG-RAN is considered valuable so that the NG-RAN will be able to apply multi-path transmission via direct Uu path and Layer 2 U2N Relay only to the UEs authorized to perform it.

Proposal: It is proposed that the AMF provides authorization information for multi-path transmission via direct Uu path and Layer 2 U2N Relay to the NG-RAN.

Proposal

It is proposed to agree the following changes into TR 23.700-33v1.1.0.

* * * * Start of 1st Change * * * *

8.5	Key Issue #5: Support of multi-path transmission for UE-to-Network Relay

For Key Issue #5 "Support of multi-path transmission for UE-to-Network Relay", the followings are taken as conclusions:

-	Policy authorization for multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with and without N3IWF via URSP is needed.

-	To support multi-path transmission via direct Uu path and Layer 3 U2N Relay UE without N3IWF, the RSD is extended with additional components to indicate multipath transmission via Layer 3 U2N Relay UE without N3IWF and Uu.

NOTE 1:	For policy authorization for multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF, whether the current Access Type preference in URSP needs to be updated will be decided in normative phase..

-	Multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF can rely on MA-PDU Session (introduced in clause 5.32 of TS 23.501 [7]) as described in Sol #28 and Sol #41, no system enhancement is required; in normative phase, how to use the MA PDU Session to support multi-path transmission via direct Uu path and Layer 3 U2N Relay UE with N3IWF will be described for information (e.g. as described in Sol #28 and Sol #41).

NOTE 2:	The conclusions of support of redundant traffic steering in Rel-18 FS_ATSSS_Ph3 study item can be applied to MA PDU Session for multi-path transmission over direct Uu and Layer-3 UE-to-Network Relay with N3IWF.

-	For multi-path transmission via direct Uu path and Layer 2 U2N Relay UE:

-	Multi-path transmission via direct Uu path and Layer 2 U2N Relay will be defined by RAN WGs and alignment work can be made by SA2 based on the progress and outcome in RAN WGs.

-	The AMF provides authorization information for multi-path transmission via direct Uu path and Layer 2 U2N Relay to the NG-RAN, i.e. whether the UE is authorized to perform multi-path transmission via direct Uu path and Layer 2 U2N Relay.

* * * * End of Changes * * * *