3GPP TSG-SA WG2 Meeting #154	S2-2210396

# November 14 – 18, 2022, Toulouse, France							      (revision of S2-22xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0132 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Layer-3 UE-to-UE Relay Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Layer-3 UE-to-UE Relay Communication has been studied and concluded to be supported in Rel-18 per TR 23.700-33 clasue 8.1, so this feature needs to be specified in TS 23.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add descriptions and procedures for Layer-3 UE-to-UE Relay Communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Layer-3 UE-to-UE Relay Communication not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.X (new), 6.3.2.X (new), 6.3.2.X.1 (new), 6.3.2.X.2 (new), 6.3.2.X.3 (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

5.X	5G ProSe UE-to-UE Relay Communication

5.X.1	5G ProSe Layer-3 UE-to-UE Relay Communication

The 5G ProSe Layer-3 UE-to-UE Relay shall provide generic function that can relay any IP, Ethernet or Unstructured traffic.

The type of traffic supported over PC5 reference point is indicated by the 5G ProSe Layer-3 UE-to-UE Relay e.g. using the corresponding RSC.

* * * * 2nd Change * * * *

6.X	5G ProSe UE-to-UE Relay Communication

6.X.1	5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay

Figure 6.X.1-1 shows the procedure for 5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay.

Figure 6.X.1-1: 5G ProSe Communication via 5G ProSe Layer-3 UE-to-UE Relay

1.	Service authorization and provisioning are performed for Source 5G ProSe Layer-3 End UE, Target 5G ProSe Layer-3 End UE and 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.2.

2.	The Source 5G ProSe Layer-3 End UE performs discovery of a 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.3.2.X.

3.	The Source 5G ProSe Layer-3 End UE sends a Direct Communication Request message to initiate the unicast Layer-2 link establishment procedure with the 5G ProSe Layer-3 UE-to-UE Relay. The Direct Communication Request message includes User Info ID of Source 5G ProSe Layer-3 End UE, User Info ID of 5G ProSe Layer-3 UE-to-UE Relay, User Info ID of Target 5G ProSe Layer-3 End UE, RSC and Security Information to 5G ProSe Layer-3 UE-to-UE Relay.

The Source Layer-2 ID of the DCR message is self-assigned by the Source 5G ProSe Layer-3 End UE, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the discovery message of the 5G ProSe Layer-3 UE-to-UE Relay.

4.	If the User Info ID of Target 5G ProSe Layer-3 End UE, User Info ID of 5G ProSe Layer-3 UE-to-UE Relay and RSC are included in the Direct Communication Request, the 5G ProSe Layer-3 UE-to-UE Relay responds by establishing the security with the Source 5G ProSe Layer-3 End UE. When the security protection is enabled, the Source 5G ProSe Layer-3 End UE sends IP Address Configuration, QoS Info (as described in clause 6.4.3.1) to the 5G ProSe Layer-3 UE-to-UE Relay.

The Source Layer-2 ID used for the security establishment procedure is self-assigned by the 5G ProSe Layer-3 UE-to-UE Relay, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the security establishment procedure messages, the Source 5G ProSe Layer-3 End UE obtains the 5G ProSe Layer-3 UE-to-UE Relay's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

5.	After the Security Establishment procedure in step 4 is completed, the 5G ProSe Layer-3 UE-to-UE Relay sends a Direct Communication Request message to initiate the unicast Layer-2 link establishment procedure with the Target 5G ProSe Layer-3 End UE. The Direct Communication Request message includes User Info ID of Source 5G ProSe Layer-3 End UE, User Info ID of 5G ProSe Layer-3 UE-to-UE Relay, User Info ID of Target 5G ProSe Layer-3 End UE, RSC and Security Information to the Target 5G ProSe Layer-3 End UE.

The Source Layer-2 ID of the DCR message is self-assigned by the 5G ProSe Layer-3 UE-to-UE Relay, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the discovery message of Target 5G ProSe Layer-3 End UE.

6.	If the User Info ID of Target 5G ProSe Layer-3 End UE and RSC are included in the Direct Communication Request, the Target 5G ProSe Layer-3 End UE responds by establishing the security with the 5G ProSe Layer-3 UE-to-UE Relay. When the security protection is enabled, the 5G ProSe Layer-3 UE-to-UE Relay sends IP Address Configuration, QoS Info (as described in clause 6.4.3.1) to the Target 5G ProSe Layer-3 End UE.

The Source Layer-2 ID used for the security establishment procedure is self-assigned by the Target 5G ProSe Layer-3 End UE, and the Destination Layer-2 ID is set to the Source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the security establishment procedure messages, the 5G ProSe Layer-3 UE-to-UE Relay obtains the Target 5G ProSe Layer-3 End UE's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

7.	The Target 5G ProSe Layer-3 End UE sends a Direct Communication Accept message to the 5G ProSe Layer-3 UE-to-UE Relay that has successfully established security with. The Direct Communication Accept message includes User Info ID of Source 5G ProSe Layer-3 End UE, User Info ID of 5G ProSe Layer-3 UE-to-UE Relay, User Info ID of Target 5G ProSe Layer-3 End UE, RSC, QoS Info, IP Address Configuration and optionally IP address of Target 5G ProSe Layer-3 End UE.

8.	After receiving the Direct Communication Accept message from the Target 5G ProSe Layer-3 End UE, the 5G ProSe Layer-3 UE-to-UE Relay sends a Direct Communication Accept message to the Source 5G ProSe Layer-3 End UE that has successfully established security with. The Direct Communication Accept message includes User Info ID of Source 5G ProSe Layer-3 End UE, User Info ID of 5G ProSe Layer-3 UE-to-UE Relay, User Info ID of Target 5G ProSe Layer-3 End UE, RSC, QoS Info, IP Address Configuration and IP address of Target 5G ProSe Layer-3 End UE if received in step 7.

9.	For IP communication, the 5G ProSe Layer-3 UE-to-UE Relay stores an association of User Info ID and the IP address of Target 5G ProSe Layer-3 End UE into its DNS entries, and the 5G ProSe Layer-3 UE-to-UE Relay acts as a DNS server to other UEs. The Source 5G ProSe Layer-3 End UE sends a DNS query to the 5G ProSe Layer-3 UE-to-UE Relay to request IP address of Target 5G ProSe Layer-3 End UE after step 8, and the 5G ProSe Layer-3 UE-to-UE Relay returns the IP address of the Target 5G ProSe Layer-3 End UE to the 5G ProSe Layer-3 End Source UE.

10.	The Source 5G ProSe Layer-3 End UE communicates with the Target 5G ProSe Layer-3 End UE via the 5G ProSe Layer UE-to-UE Relay.

In the case of one Source 5G ProSe Layer-3 End UE communicates with multiple Target 5G ProSe Layer-3 End UEs, the PC5 link between the Source 5G ProSe Layer-3 End UE and the 5G ProSe Layer-3 UE-to-UE Relay can be shared for multiple Target 5G ProSe Layer-3 End UEs per RSC while the PC5 links may be established individually between the 5G ProSe Layer-3 UE-to-UE Relay and Target 5G ProSe Layer-3 End UEs per RSC. For the shared PC5 link, the Layer-2 link modification procedure can be used.

In the case of multiple Source 5G ProSe Layer-3 End UEs communicate with one 5G ProSe Layer-3 End Target UE, the PC5 link between the 5G ProSe Layer-3 UE-to-UE Relay and the Target 5G ProSe Layer-3 End UE can be shared per RSC while the PC5 links may be established individually between the Source 5G ProSe Layer-3 End UEs and the 5G ProSe Layer-3 UE-to-UE Relay per RSC. For the shared PC5 link, the Layer-2 link modification procedure can be used.

* * * * End of Changes * * * *