3GPP TSG-WG SA2 Meeting #154 	S2-2210442

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2: Conclusion update

Document for:	Approval

Agenda Item:	9.18.1

Work Item / Release:	FS_5MBS_Ph2 / Rel-18

Abstract: Based on the corresponding discussion paper (S2-2210445), it is proposed in this document to update the conclusion of KI#2.

1. Introduction

KI#2 is focusing on efficient resource usage for broadcast MBS session, when multiple CNs are connected to the same NG-RAN.

This KI is RAN dependent, and companies in SA2 proposed to wait until further inputs from RAN WGs are available.

In this meeting RAN3 in R3-225987 provides the evaluation for the solutions in TR 23.700-47 of KI#2, and as analyzed in the corresponding discussion paper (S2-2210445), the conclusion can be updated as per the feedback accordingly.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-47.

# * * * * First change * * * *

8.2	Key Issue #2: MOCN network sharing

For conclusions, the following aspects will be considered:

-	For solutions where the broadcast MBS sessions for different PLMNs are established towards a NG-RAN node, the NG-RAN node shall be able to identify the same MBS service and avoid multiple deliveries over radio.

-	A solution compatible with Rel-17 UEs and Rel-17 RAN nodes are preferred.

-	The AF should provide the associated session identifier comprising of SSM to be passed to RAN nodes to enable shared RAN nodes to identify the same MBS service.

The following will be taken into account:

-	It should be possible not to establish all the shared delivery tunnels to the same NG RAN from different PLMNs for the same MBS service. And it is the preference of NG-RAN nodes to determine to not establish all the shared delivery tunnels.

-	The solution should support scenario where all RAN nodes are shared by PLMNs and the scenario where only part of the RAN nodes are shared by PLMNs.

# * * * * End of changes * * * *