3GPP TSG-WG SA2 Meeting #154	S2-2210509

# Toulouse, France, November 14 – 18, 2022		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0073 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS re-discovery with EASDF for HR roaming scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of EAS re-discovery with EASDF in HR roaming scenario is concluded in clause 8.1.2 of TR 23.700-48. This contribution proposes to add corresponding descriptions into the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause “6.2.3.x EAS Re-discovery Procedure at Edge Relocation for HR roaming” and related sub-clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EAS Re-discovery Procedure with EASDF for HR roaming is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.x(new), 6.2.3.x.1(new), 6.2.3.x.2(new), 6.2.3.x.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 3619 TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.3.x	EAS Re-discovery Procedure at Edge Relocation for HR roaming

6.2.3.x.1	General

The EAS re-discovery in VPLMN can be triggered due to 1) UE mobility, see clause 6.2.3.x.2; 2) AF interacting with HPLMN, see clause 6.2.3.x.3; 3) AF interacting with VPLMN.

Editor's note:	AF triggered EAS Re-discovery via interacting with VPLMN is FFS.

6.2.3.x.2	UE Mobility Triggered EAS Re-discovery

Figure 6.2.3.x.2-1 shows the procedure of EAS re-discovery in VPLMN for HR roaming.

Figure 6.2.3.x.2-1: EAS rediscovery procedure triggered by UE mobility

1.	The procedure described in clause 4.23.7 (N2 based handover) or 4.23.11 (Xn based handover) of TS 23.502 [3] is performed by replacing I-SMF with V-SMF and SMF with H-SMF.

1a1.	(V-SMF insertion) The (target) V-SMF retrieves SM context from H-SMF.

1a2.	(V-SMF change) The target V-SMF retrieves SM context from the source V-SMF.

1b.	H-SMF (in case of V-SMF insertion) or source V-SMF (in case of V-SMF change) to target V-SMF: Nsmf_PDUSession_Context Response including SM context. The SM context includes Authorization Information and Policy for HR roaming with selective traffic routing in VPLMN from H-SMF.

The SM context also includes impact field corresponding to the old target DNAI if it has been inserted by the H-SMF or the source V-SMF.

In the case of V-SMF insertion or V-SMF change, when the AMF sends Nsmf_PDUSession_CreateSMContext Request to the target V-SMF, the AMF includes indication which indicates HR roaming with selective traffic routing in VPLMN is allowed if received from the UDM.

Step 1 is skipped if UE moves without V-SMF change.

Steps 1-5 are skipped if UE moves with V-SMF removal.

After the handover procedure, the (target) V-SMF performs UL CL/BP and local PSA insertion/change/removal as described in clause 4.23.9 of TS 23.502 [3] by replacing I-SMF with V-SMF and SMF and H-SMF. Differences are shown as steps 2 to 5:

2.	The (target) V-SMF may (re-)select a new V-EASDF based on the DNAI corresponding to the target UE location. The (target) V-SMF configures the new V-EASDF using Neasdf_DNSContext_Create service.

If a new V-EASDF is selected, the DNS context in the old V-EASDF is removed by the (source/target) V-SMF using Neasdf_DNSContext_Delete service.

3.	The (target) V-SMF invokes Nsmf_PDUSession_Update Request (new target DNAI if available, the new V-EASDF address if available, impact field if available) to the H-SMF.

For the case of local PSA removal, the request message is sent to the H-SMF without including any DNAI.

The impact field corresponds to:

-	the new target DNAI. For the case of local PSA insertion, impact field corresponding to the new target DNAI is sent from the V-SMF to the H-SMF.

-	the new target DNAI and old target DNAI. For the case of local PSA change, the (target) V-SMF determines the impact field based on the new target DNAI and the impact field corresponds to the old target DNAI received in step 1.

-	the old target DNAI. For the case of local PSA removal, only impact field corresponding to the old target DNAI received in step1 is sent from the V-SMF to the H-SMF.

4.	The H-SMF determines DNAI change occurrence based on the received information in step 3, and triggers EAS rediscovery by initiating Nsmf_PDUSession_Update Request towards the (target) V-SMF including the PCO.

The PCO includes EAS rediscovery indication, optionally impact field and optionally V-EASDF address.

5.	The V-SMF initiates PDU Session Modification command including the PCO to the UE.

6.	(V-SMF removal case) The H-SMF initiates PDU Session Modification command including PCO to the UE.

In the case that the H-SMF has the knowledge of EAS deployment information in VPLMN, the H-SMF determines the impact field corresponding to old target DNAI (if has) inserted by the removed V-SMF. The H-SMF includes EAS rediscovery indication and the impact field into the PCO.

In the case that the H-SMF does not have the knowledge of EAS deployment information in VPLMN, the H-SMF only includes EAS rediscovery indication into the PCO.

If the H-SMF selects an EASDF in HPLMN, the H-SMF also includes the new EASDF address to the UE.

6.2.3.x.3	AF triggered EAS Re-discovery via interacting with HPLMN

The AF deployed in VPLMN may interact with H-SMF via H-NEF and provides the target DNAI to the H-SMF.

The H-SMF sends target DNAI information including the target DNAI to the V-SMF, which then relays this information to the AMF. The AMF uses the target DNAI to select a target V-SMF, if needed, as described in clause 4.23.5.4 of TS 23.502 [3] by replacing I-SMF with V-SMF, SMF with H-SMF.

The target V-SMF performs steps 3-5 as described in clause 6.2.3.x.3.

# * * * * End of changes * * * *