3GPP TSG-SA WG2#154	S2-2210518

Toulouse, France,14 – 18 November 2022                              	(revision of S2-220xxxx)

Source:	vivo

Title:	KI#1, Sol#3: Sol Update: Accuracy based NWDAF Analytics Correctness Improvement

Document for:	Approval

Agenda Item:	9.23

Work Item / Release:	FS_eNA_Ph3 / Rel-18

Abstract of the contribution: This contribution proposes to update solution#3 based on the discussion paper.

1. Introduction/Discussion

Solution #3 introduced that the NWDAF containing AnLF compute the Accuracy in Use or the MAE in Use, which reflects the performance of ML model used in live network, however, it missed the impact of the consumers’ actions and may calculated an incorrect accuracy.

As discussed in S2-2210517, the option a describes that consumer NF provides a feedback information which contains whether the consumer NFs perform actions to the NWDAF, and the NWDAF take it into account in accuracy calculating. The updated pCR assumed option a in S2-2210517 is agreed.

- Regarding feedback information, threes alternative options are listed:

  - Option a: whether the consumer NFs do actions.

  - Option b: In addition to option a, if action is performed, further indicate whether the actions affect the network (based on the consumer NFs’ internal logic).

  - Option c:  indicate whether the network performance or user experience are improved e.g. by comparing network KPIs without/with action taken by NF consuming data analytics (proposed by Huawei).

For simplicity, this pCR only modify solution #3 and the modification applies to other solutions as well.

2. Text Proposal

It is proposed to update the following text proposal in TR 23.700-81.

# * * * * First change * * * *

6.3	Solution #3: Accuracy based NWDAF Analytics Correctness Improvement

6.3.1	Description

This solution is about KI #1: How to improve correctness of NWDAF analytics to address how to improve correctness of NWDAF analytics.

In AI community, there is a concept called 'accuracy' which is used to indicate the correctness of an AI/ML model's predictions. The Accuracy is the percentage of correct predictions in all predictions. One of methods to determines a prediction is correct is to compare the prediction with its corresponding ground truth data i.e. the label data.

For the regression problems, the predictions and the ground truth data may be not same but similar, and their difference can be calculated by other methods e.g. Mean Absolute Error (MAE). If this difference is smaller than a threshold, this prediction should be considered as correct.

Figure 6.3.1-1: Accuracy in AI Community

As shown in Figure 6.3.1-2, Accuracy in Training (AiT) and Accuracy in Use (AiU) can be used to measure the correctness of a ML model, the Accuracy in Use should be observed/monitored by the NWDAF/network, which could be a trigger for NWDAF(MTLF) to re-train the ML Model if Accuracy in Use cross a threshold:

Figure 6.3.1-2: Accuracy in Training with Training dataset vs Accuracy in Use with live network dataset

As shown in left side of the Figure, a NWDAF(MTLF) trained a ML Model with Training dataset (input data, ground truth data) and the Accuracy in Training is to indicate the performance of ML model in training stage by comparing prediction with ground truth data in validation dataset reserved from training dataset.

The NWDAF(MTLF) deliver the trained ML Model to a NWDAF(AnLF), which will be used by the NWDAF(AnLF) to perform inference with input data from live network.

1)	As shown in right side of the Figure, Accuracy in Use is to indicate the performance of ML model used in live network by comparing prediction with the observed label data i.e. ground truth from the live network.

Please note that the NWDAF(AnLF) performs inference with live network input data to get the prediction and also retrieves the label data from live network.

2)	However, due to some reasons, e.g. different data distribution between training data set and data in live network, poor model generalization ability, etc. there may be a gap between Accuracy in Training and Accuracy in Use, and the Accuracy in Use usually is not as good as Accuracy in Training especially as times goes by.

The NWDAF(AnLF) decides to indicate the NWDAF(MTLF) to re-train the AI/ML model e.g. if Accuracy in Use crosses a threshold or the gap between Accuracy in Training and Accuracy in Use crosses a threshold.

3)	Consumer NF may perform actions based on the analytics results received from NWDAF(AnLF) and may lead to an incorrect accuracy since the ground truth may be affected.

To avoid the incorrect accuracy calculation, the consumer could provide a feedback information (e.g. whether the consumer NFs do actions) to the NWDAF, and the NWDAF takes the feedback information into account to improve Accuracy Calculating.

In order to improve training data quality e.g. the data distribution, the NWDAF(AnLF) also provides the input data, ground truth data, where prediction is regarded as incorrect prediction by comparing with ground truth data, to help the NWDAF(MTLF) to re-train a better AI/ML Model e.g. with model generalization ability.

On the other hand, based on local configuration, the NWDAF(MTLF) could decide to calculate the Accuracy/MAE in Use after collecting the data (the input data, predictions and the corresponding ground truth data) from the NWDAF(AnLF) or ADRF, and decides to re-train models.

Note:	Input data is the necessary data which is collected by AnLF to perform inference to generate prediction and the ground truth data is the actual measured data of the prediction. In other words, the "ground truth data" is the measured data that corresponds or relates to the one predicted.

Similarly, regarding regression, MAE in training and MAE in use is introduced to measure the ML model correctness.

6.3.2	Procedures

6.3.2.1	AnLF-based Accuracy calculating

Figure 6.3.2-1 depicts the procedure for the proposed solution that the AnLF collects the ground truth data and calculates the AiU.

Figure 6.3.2-1: AnLF collect ground truth data and calculate Accuracy/MAE in use

1.	The NWDAF(AnLF) service consumer subscribes to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.1.3 of TS 23.288 [5].

2.	The NWDAF(AnLF) subscribes to a NWDAF(MTLF) a trained ML Model by invoking the Nnwdaf_MLModelProvision (Analytics ID, ML Model Filter Information, Accuracy/MAE requirement) service operation.

3~4.	The MTLF trains the model according to the requirement from AnLF and notifies the AnLF with the ML model information by invoking Nnwdaf_MLModelProvision_Notify (Analytic ID, the file address of the trained ML model, Accuracy in Training (AiT) or MAE in Training) service operation.

5~8.	The AnLF collects the input data from data providers, e.g. OAM, NFs, makes predictions and sends the predictions to the NF consumer. The AnLF also collects the ground truth data, which is actual measured data and corresponding to the prediction calculated by AnLF (based on the trained ML Model provided by the MTLF).

9.	The AnLF calculates the Accuracy in Use or the MAE in Use, which reflects the performance of ML model used in live network by comparing prediction with the corresponding ground truth data retrieved from the live network by the AnLF in step 6. Optionally, the AnLF may take the feedback information received from the consumer NF into account to improve Accuracy Calculating.

10.	The AnLF compares the Accuracy in Use with the Accuracy in Training and/or MAE in Use with MAE in Training and decide whether or not to send notification to MTLF to re-train the AI/ML model e.g. if Accuracy in Use cross a threshold or by comparing the Accuracy in Training and Accuracy in Use.

11~12.	The AnLF reuse MLModelProvision service (subscription Correlation ID, Analytic ID, AiU or MAE in Use, the corresponding time scale, input data and ground truth data) to modify of the original ML model subscription in step 2, indicating to the MTLF that the Accuracy in Use or MAE in Use is not good as the Accuracy in Training or MAE in Training and the MTLF retrains the model with new data from the AnLF.

Both AiU or MAE in Use and the corresponding time scale are indicated to MTLF, indicating AiU or MAE in Use is calculated within the corresponding time scale.

In order to improve training data quality e.g. the data distribution, the AnLF provides the input data, ground truth, where prediction is regarded as incorrect prediction by comparing with the ground truth:

-	In the case of classification, if the prediction is not same with its corresponding ground truth data retrieved from live network then this prediction should be considered as incorrect.

-	In the case of regression, if the difference e.g. Mean Absolute Error (MAE) between the prediction and its corresponding ground truth data is greater than a threshold, this prediction should be considered as incorrect.

14~15.	The AnLF notifies the consumer that the Accuracy in Use is not as good as the Accuracy in Training to consequently stop using the provided analytic result by invoking the Nnwdaf_AnalyticInfo_Notify (Analytic ID, AiU, Disable notification).

6.3.2.2	MTLF-based Accuracy calculating

Figure 6.3.2-2 depicts the procedure for the proposed solution that the AnLF collects the ground truth data but MTLF calculates the AiU.

Figure 6.3.2-2: MTLF calculate Accuracy/MAE in use with collected data by AnLF

1-8.	The procedures are same as described in step 1-8 of clause 6.3.2.1 except in step 4 the MTLF in addition indicates that the MTLF need data (input data, prediction and its corresponding ground truth data) to monitor the performance of the ML model.

9.	The AnLF sends the data to the MTLF by reusing the MLModelProvision service (subscription Correlation ID, Analytic ID, input data, prediction and its corresponding ground truth data, feedback information).

Alternatively, the AnLF could store the data in the ADRF by using Nadrf_DataManagement_StorageRequest , which could be retrieved by MTLF to calculate Accuracy/MAE in use.

10-11.	The MTLF calculates the Accuracy/MAE in Use by comparing predictions and the corresponding ground truth data retrieved from the AnLF in step 9, and compares the Accuracy/MAE in Use with the Accuracy/MAE in Training and decide whether or not to re-train the ML model. Optionally, the AnLF may take the feedback information received from the consumer NF into account to improve Accuracy Calculating.

12.	The MTLF may collect the new data for re-training from data providers, e.g. OAM, NFs.

13.	The MTLF re-trains the ML model with the data collected in step 9 and step 12, and notifies the AnLF with the re-trained model and accuracy report (Accuracy/MAE in Use and/or Accuracy/MAE in re-training).

14-15.	The procedures are same as described in step 13-14 of clause 6.3.2.1.

6.3.3	Impacts on services, entities and interfaces

The solution has the following impacts:

NWDAF(AnLF):

-	Collects data from corresponding data provider(s) to calculate the Accuracy in Use and MAE in Use.

-	Sends the notifications to the NWDAF(MTLF) and consumer to inform the Accuracy in Use or MAE in Use has decreased.

-	Sends data (input data, prediction and its corresponding ground truth data, feedback information) to the MTLF or ADRF.

-	Receives the feedback information (whether the consumer NFs perform actions) from consumer NF.

NWDAF(MTLF):

-	Receives the notification that the Accuracy in Use or MAE in Use has decreased from NWDAF(AnLF) and re-trains the AI/ML Model.

-	Receives data from the AnLF to calculate Accuracy/MAE in Use, which are notified to NWDAF(AnLF).

-	Receives the feedback information (whether the consumer NFs perform actions) from the AnLF.

NWDAF service Consumer:

-	Receives the notification that the Accuracy in Use or MAE in Use has decreased from NWDAF(AnLF) and stops using the analytics provided by the NWDAF(AnLF).

-	Provides the feedback information (whether the consumer NFs perform actions) to the AnLF.

# * * * * End of changes * * * *