SA WG2 Meeting #154	S2-2210538

Toulouse, France, Nov 14 - 18, 2022                   	(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0139 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for Communication Path Switching between Two UE-to-Network Relays |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 5G_ProSe_Ph2 studies the key issue for support of path switching between two indirect network communication paths for UE-to-Network Relaying with service continuity consideration; and it concuded the path switching between two UE-to-Network Realys will be included in Release 18 of TS 23.304. This CR is submitted to introduce the procedures for communication path switching between two UE-to-Network Realys, and focuses on one of the Relays is Layer-2 UE-to-Network Relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures for communication path switching between two UE-to-Network Realys with one of Relays is Layer-2 UE-to-Network Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures for communication path switching between two UE-to-Network Realys are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** FIRST CHANGE (All New Text) ******************

6.5.X	Procedures for Communication Path Switching fo Two UE-to-Network Relays

6.5.X.1	General

The communication path switching between two UE-to-Network Relays includes the following scenarios:

-	Layer-3 UE-to-Network Relay with N3IWF switching from/to Layer-3 UE-to-Network Relay with N3IWF.

-	Layer-3 UE-to-Network Relay without N3IWF switching from/to Layer-3 UE-to-Network Relay without N3IWF.

-	Layer-3 UE-to-Network Relay without N3IWF switching from/to Layer-3 UE-to-Network Relay with N3IWF.

-	Layer-2 UE-to-Network Relay switching from/to Layer-2 UE-to-Network Relay.

-	Layer-2 UE-to-Network Relay switching from/to Layer-3 UE-to-Network Relay without N3IWF.

-	Layer-2 UE-to-Network Relay switching from/to Layer-3 UE-to-Network Relay with N3IWF.

6.5.X.2	Procedures for Communication Path Switching for Layer-2 UE-to-Network Relay

This clause specifies the communication path switching between two UE-to-Network Relays with one of the UE-to-Network Relays is Layer-2 UE-to-Network Relay.

When Layer-2 Remote UE in CM-CONNECTED state switches the communication path to another Layer-2 UE-to-Network Relay, the handover procedures defined for 3GPP access in clause 4.9.1 of TS 23.502 [5] are reused:

-	When Xn interface is supported between the source NG-RAN and target NG-RAN, Xn based inter NG-RAN handover procedures specified in clause 4.9.1.2 of TS 23.502 [5] are reused.

-	When Xn interface is not supported the source NG-RAN and target NG-RAN, Inter NG-RAN node N2 based handover procedures specified in clause 4.9.1.3 of TS 23.502 [5] are reused.

When Layer-2 Remote UE in CM-CONNECTED state switches the communication path to Layer-3 UE-to-Network Relay with N3IWF, Layer 2 Remote UE's PDU session(s) can handover to the indirect path via Layer-3 UE-to-Network Relay with N3IWF by reusing the following procedures:

-	The handover procedures specified in clause 4.9.2 of TS 23.502 [5] for UE mobility between a 3GPP access and untrusted non-3GPP access are reused.

When Layer-3 Remote UE switches the communication path from Layer-3 UE-to-Network Relay with N3IWF to an indirect path via Layer-2 UE-to-Network Relay, Layer-3 Remote UE’s PDU session(s) can handover to the indirect path via Layer-2 UE-to-Network Relay by reusing the following procedures:

-	The handover procedures specified in clause 4.9.2 of TS 23.502 [5] for UE mobility between a 3GPP access and untrusted non-3GPP access are reused.

****************** END OF CHANGES ***************