SA WG2 Meeting #154	S2-2210539

Toulouse, France, Nov 14 - 18, 2022                   	(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0140 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Multi-path Transmission for Layer-3 UE-to-Network Relay without N3IWF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 5G_ProSe_Ph2 studies the key issue of multi-path transmission for UE-to-Network Relay; and it concuded for Layer-3 UE-to-Network Relay without N3IWF multi-path transmission as following: - To support multi-path transmission via direct Uu path and Layer 3 U2N Relay UE without N3IWF, the RSD is extended with additional components to indicate multipath transmission via Layer 3 U2N Relay UE without N3IWF and Uu. This CR is submitted to support multi-path transmission for Layer-3 UE-to-Network Relay without N3IWF and introduce the related procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description and procedures for supporting multi-path transmission for Layer-3 UE-to-Network Relay without N3IWF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to support multi-path transmission for Layer-3 UE-to-Network Relay without N3IWF is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** FIRST CHANGE (All New Text) ******************

6.5.X	Support of Multi-path Transmission for Layer-3 UE-to-Network Relay without N3IWF

6.5.X.1	General

To support multi-path transmission of application traffic with one direct network communication path and one indirect network communication path via Layer-3 UE-to-Network Relay without N3IWF, a new item "Multi-Path ProSe Layer-3 UE-to-Network Relay Offload indication" is added in the RSD of URSP. This indication is used to route the traffic of the matching application using multi-path with one direct Uu path and another indirect path via a Layer-3 UE-to-Network Relay without N3IWF outside of a PDU Session.

The procedures for multi-path transmission for Layer-3 UE-to-Network Relay without N3IWF are described in clause 6.5.X.2.

6.5.X.2	Procedures for Multi-path Transmission for Layer-3 UE-to-Network Relay without N3IWF

Figure 6.5.X.2-1 shows the procedures for multi-path transmission for Layer-3 UE-to-Network Relay without N3IWF.

Figure 6.5.X.2-1: Multi-path transmission for Layer-3 UE-to-Network Relay without N3IWF

0a.	The Remote UE is authorized by network to act as Layer-3 Remote UE and use multi-path transmission with Layer-3 UE-to-Network Relay without N3IWF.

0a.	The Relay UE is authorized by network to act as Layer-3 UE-to-Network UE.

1. 	Remote UE evaluates the matched URSP rule for application traffic and decides whether to use Multi-Path PDU Session for the application traffic.

When "Multi-Path ProSe Layer-3 UE-to-Network Relay Offload indication" is included in the RSD, Remote UE will route the traffic of the matching application using multi-path with one direct Uu path and another indirect path via a Layer-3 UE-to-Network Relay without N3IWF.

2. 	Remote UE establishes or modifies PDU session to transmit application traffic on direct Uu path.

3. 	The application traffic is transmitted on direct Uu path.

4. 	Remote UE discovers a Layer-3 UE-to-Network UE and establishes the unicast link; Layer-3 UE-to Network UE establishes or modifies its PDU session for application traffic from Remote UE.

5. 	The application traffic is transmitted on indirect path via a Layer-3 UE-to-Network Relay without N3IWF.

NOTE 1:	There is no sequential dependency on step 2~3 and step 4~5.

NOTE 2:	The multi-path transmission could be used for application traffic splitting or duplication which is controlled by application layer.

****************** END OF CHANGES ***************