3GPP TSG-WG SA2 Meeting #154	S2-2210547

# Toulouse, France, November 14 – 18, 2022		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0075 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4 common EAS enforcement for set of UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion of FS_EDGE_Ph2 KI#4, 5GC selection and enforcement of common EAS/DNAI is supported, and different UEs can be served by multiple SMFs. Since “collection” in 3GPP specs usually refers to “process”, so “UE set” is used to replace “UE collection” in the KI#4 conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Definition of Common EAS discovery for a set of UEs. 2. EAS discovery corresponding to Common DNAI for set of UEs. 3. Coordination among SMFs for Common EAS/DNAI determination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EDGE_Ph2 feature is incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.X, 6.2.3.2.Y, 6.2.3.2.Z. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.501 CR3788, TS23.502 CR3625, TS23.503 CR0776 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.3.2.X	Common EAS discovery for set of UEs

The following is the procedure for discovery common EAS for set of UEs accessing the same application. Different UEs can be served by different SMFs.

1. The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [9] is used to request selecting the common EAS for set of UEs accessing the application as identified in the AF Request.

An EAS Correlation indication can be provided for selecting the same EAS for set of UEs accessing the same application.

Spatial Validity Condition could be provided for limiting the location of the UEs, and also "any UE" or an UE list or group ID can be provided for defining set of UEs accessing the same EAS.

In step 5 of figure 4.3.6.2-1 in TS 23.502 [9], PCF determines the UEs influenced by the AF Request, and based on AF request, PCF creates PCC rule with Traffic Correlation ID and EAS Correlation indication to SMF.

2. Based on steps 1~19 in figure 6.2.3.2.2-1, with the following updates:

In step 9:

If FQDN in Neasdf_DNSContext_Notify Request is for the application indicated in PCC rule , and if EAS Correlation indication is set, SMF determines the UE belongs to set of UEs identified by Traffic Correlation ID and accesses the application and determines the UE needs to use the common EAS for the set of UEs.

NOTE 1:	SMF can determine the relationship between FQDN and application indicated in PCC rule based on configuration or PFD.

If the common EAS is not available for the set of UEs:

steps 10~15 are used for discovering an EAS, SMF could take multiple UEs into consideration for EAS discovery. In step 15 SMF synchronizes the selected EAS IP according to clause 6.2.3.2.Z.

If the common EAS is available for the set of UEs:

Step 10~Step 15 are skipped.

In step 17:

SMF sends DNS message handling rule with IP address for the common EAS and instructs EASDF to return the Common EAS IP address in a DNS response to UE directly.

In step 19:

EASDF sends DNS response with the IP address of the common EAS instructed in step 17 to UE.

# * * * * Second change * * * *

6.2.3.2.Y	EAS discovery corresponding to Common DNAI for set of UEs

The following is the procedure for discovery EAS corresponding to a Common DNAI for set of UEs accessing the same application.

Figure 6.2.3.2.X-1: EAS discovery corresponding to Common DNAI for set of UEs

1. The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [9] is used to request selecting the common DNAI for set of UEs accessing the application as identified in the AF Request.

An indication of traffic correlation can be provided for indicating selecting the same DNAI (i.e. selecting EAS corresponding to the same DNAI) for set of UEs accessing the same application.

Spatial Validity Condition could be provided for limiting the location of the UEs, and also "any UE" or an UE list or group ID can be provided for defining set of UEs accessing the same DNAI.

In step 5 of figure 4.3.6.2-1 in TS 23.502 [9], PCF determines the UEs influenced by the AF Request, and based on AF request, PCF creates PCC rule with Traffic Correlation ID and indication of traffic correlation to SMF.

2. Based on steps 1~19 in figure 6.2.3.2.2-1, with the following changes:

In step 10:

If FQDN in Neasdf_DNSContext_Notify Request is for the application indicated in PCC rule, and if indication of traffic correlation is set, SMF determines the UE belongs to set of UEs identified by Traffic Correlation ID and  accessing the application and determines the UE needs to connect to EAS corresponding to the same DNAI for the set of UEs.

If the common DNAI is not available, SMF determines the common DNAI and SMF could take multiple UEs into consideration for the determination of common DNAI. SMF could synchronize the selected common DNAI according to clause 6.2.3.2.Z.

Once the common DNAI is available, for Option A, SMF provisions EASDF with the information to build EDNS Client Subnet option that refers to a location that is topologically close to the common DNAI; for Option B, SMF provisions EASDF with Local DNS server related to the common DNAI.

# * * * * Third change * * * *

6.2.3.2.Z	Coordination among SMFs for Common EAS/DNAI determination

For the case that UEs in the set of UEs are served by multiple SMFs, coordination is needed among SMFs for determining the common EAS/DNAI. The common EAS/DNAI is stored as part of AF traffic influence request information in UDR.

Figure 6.2.3.2.X-1: Synchronization of Common EAS/DNAI for SMFs

1-4.	In case a new EAS/DNAI has been determined, SMF sends the EAS/DNAI, Resource URI to PCF by invoking Npcf_SMPolicyControl_Update Request, triggering the PCF to update the corresponding AF traffic influence request information with common EAS/DNAI in UDR and UDR will lock the data to prevent the data to be modified to other EAS/DNAI by other PCF providing different EAS/DNAI for the same set of UEs. PCF updates PCC rule to SMF including the common EAS/DNAI of AF traffic influence request information in UDR.

In case SMF decides to trigger EAS rediscovery for set of UEs, SMF removes the common EAS/DNAI from AF traffic influence request information, UDR notifies PCF, and then PCF updates SMF with updated PCC rule removing the common EAS/DNAI, and triggers other SMF for EAS rediscovery.

NOTE 1:	The common EAS/DNAI for set of UEs can be deleted in case the AF traffic influence request information is removed by AF.

5-7.	In case the common EAS/DNAI for set of UEs is created/deleted, UDR notifies PCFs and then PCF will update PCC rules to the related SMF(s).

# * * * * End of changes * * * *