3GPP TSG-WG SA2 Meeting #154	S2-2210550

# Toulouse, France, November 14 – 18, 2022		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0077 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common EAS/DNAI selection by AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to agreed conclution of TR 23.700-48, AF can determine the common EAS/DNAI, e.g. based on candidate DNAI(s) obtained from SMF(s), which is to be decided in normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New procedure: Common EAS/DNAI selection by AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreed conclusion on solution to KI#4 of TR 23.700-48 will not be investigated in normative work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.501CR3789, TS23.502CR0777 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new text) * * * *

6.2.x	Common EAS/DNAI selection by AF

Figure 6.2.x-1: Common EAS/DNAI selection by AF

1.	AF sends Nnef_CandidateDNAI_Get Request to NEF, including the identification of the UE set, optionally including the available DNAI list where EAS instances are deployed.

2.	NEF invokes Nbsf_Management_Discovery message to discover PCF.

Step 3-7 are performed for each UE of the UE set.

3.	NEF sends Npcf_Information_Get Request message to the PCF to query the candidate DNAI(s). The message may also include the available DNAI list where EAS are deployed if provided in AF request.

For trusted AF, step 1 is skipped and AF sends Npcf_Information_Get Request message to PCF directly.

4.	PCF invokes Npcf_SMPolicyControl_UpdateNotify Request service to query the candidate DNAI(s), including information received in step 3.

5.	SMF selects the candidate DNAI(s). The SMF may consider the UE location and available DNAI list provided by AF to select the closest available DNAI(s) as candidate DNAI(s). The SMF may also provide the DNAI(s) in a prioritized order.

6.	SMF sends candidate DNAI(s) of the UE to PCF.

7.	PCF sends candidate DNAI(s) of the UE to NEF.

For trusted AF, PCF sends Npcf_Information_Get Response message to the AF directly, and step 8 is skipped.

8.	NEF sends candidate DNAI(s) of the UE set based on candidate DNAI(s) of each UE and sends it to AF.

9.	After receiving the candidate DNAI(s) of the UE set or each UE in the UE set, the AF selects a proper DNAI as the common DNAI according to the candidate DNAI(s). If common EAS is required for the UE set, AF selects common EAS based on the common DNAI.

# * * * * End of changes * * * *