3GPP TSG-SA2 Meeting #154	S2-2210557

# Toulouse, France, Nov 14 – 18, 2022																(was xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0574 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8: Addition of Nnwdaf_MLModelTraining service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was concluded that federated learning for NWDAF containing MTLF is supported with Principle 7: “The services to enable the FL based ML model training should be generic enough for all ML model training mechanisms which require service provider trains the ML model provided by the service consumer.” However, the service to satisfy Pinciple 7 does not exist in TS 23.288. So this CR propose to define the service and procedure which satisfy the principle 7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR propses to add a new NWDAF services, i.e., Nnwdaf_MLModelTraining and the procedures to support Principle 7 in key issue #8. In the procedure, service consumer (e.g., FL server) provides (initial) ML model to service provider (e.g., FL client) and the service provider notifies model update results to the service consumer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 TR conclusion for key issue #8 is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2x.1 (new), 6.2x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_eNA_Ph3 and the WI code for the work is not assigned yet & will replace WI code DUMMY when available. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * The 1st change (all new text) * * * *

6.2x.1	ML Model Training Subscribe/Unsubscribe

The procedure in Figure 6.2x.1-1 is used by an NWDAF service consumer, i.e., an NWDAF containing MTLF to subscribe/unsubscribe at another NWDAF, i.e., an NWDAF containing MTLF, to be notified ML Model training results when becomes available at the service producer NWDAF, using Nnwdaf_MLTraining services as defined in clause 7.x. The service can be used by an NWDAF containing MTLF to enable Federated Learning or to update ML model when trained data is not enough. The service is also used by an NWDAF to modify existing ML Model training subscription(s). An NWDAF containing MTLF can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2x.1-1: ML Model for Training subscribe/unsubscribe

1.	The NWDAF service consumer (i.e. an NWDAF containing MTLF) subscribes to, modifies, or cancels subscription for training a (set of) ML Model(s) by invoking the Nnwdaf_MLModelTraining_Subscribe / Nnwdaf_MLModelTraining_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2x.2.

In order to enable Federated Learning, NWDAF Service consumer act as Federated Learning server can subscribe to multiple NWDAFs containing MTLF act as Federated Learning clients to train an ML Model based on ML model requirement (i.e., analytics ID, Service Area/DNAI, or availability of data).

When NWDAF service consumer determine to further update the ML model, NWDAF service consumer modifies the subscription by invoking Nnwdaf_MLModelTraining_Subscribe service operation including Subscription Correlation ID and new ML Model Information.

2.	The NWDAF containing MTLF trains ML model provided at step 1 by collecting new data or re-use the data that it owns.

3.	When the NWDAF containing MTLF completes ML model training, the NWDAF containing MTLF notifies the NWDAF service consumer with the updated ML Model Information (containing a file address of updated results) by invoking Nnwdaf_MLModelTraining_Notify service operation. The parameters that can be provided by the NWDAF containing MTLF is specified in clause 6.2x.2.

When the step 1 is for a subscription modification (i.e., including Subscription Correlation ID) and include new ML Model Information, the NWDAF containing MTLF may reject the subscription.

6.2x.2	Contents of ML Model Training

The consumers of the ML model training services (i.e., an NWDAF containing MTLF) may provide the input parameters as listed below:

-	Information for training an ML model including:

-	A list of Analytics ID(s): identifies the analytics for the provided ML model is used.

-	A list of ML Model ID(s): identifies the provided ML model.

-	ML Model Information, which includes the ML model file address(es) (e.g., URL or FQDN).

-	Interoperability token, which indicates interoperable model information, e.g., file format, platform, recording method for training results, etc.

NOTE:	The vendor list indicated by the Interoperability indicator is determined by the NWDAF vendor and is configured in the NWDAF.

-	[OPTIONAL] Training Filter Information: enables to select which data for the ML model training is requested, e.g., S-NSSAI, Area of Interest. Parameter types in the Training Filter Information are the same as or subset of parameter types in the ML Model Filter Information which are defined in procedure 6.2A.1.

-	[OPTIONAL] Target of Training Reporting: indicates the object(s) for which data for ML model training is requested, e.g., specific UEs, a group of UE(s) or any UE (i.e., all UEs).

-	Training Reporting Information with the following parameters:

-	Maximum response time: indicates maximum time for waiting notifications (i.e., training results).

-	The number of required notifications: indicates the number to be notified of training results for an ML model.

Editor's note:	It is FFS whether the number of required notifications is needed.

- 	Expiry time.

-	A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the NWDAF containing MTLF with this subscription.

The NWDAF containing MTLF provides to the consumer of the ML model training service operations as described in clause 7.x, the output information as listed below:

-	The Notification Correlation Information.

-	ML Model ID(s): identifies the provisioned ML model(s).

-	Updated ML Model Information, which includes a file address of updated results (e.g., URL or FQDN) for ML Model ID(s)

# * * * * End of changes * * * *