3GPP TSG-SA2 Meeting #154	S2-2210568

Toulouse, France, Nov 14 - 18, 2022	(revision of S2-xxxxxxx)

Source:	Tencent, Tencent Cloud

Title:	KI#4 and 5, Conclusion update to solve the ENs

Document for:	Approval

Agenda Item:	9.19.1

Work Item / Release:	FS_XRM /Rel-18

Discussion:

This contribution proposes to update the conclusions for KI#4 “PDU Set integrated packet handling” and KI#5: “Differentiated PDU Set Handling” in TR 23.700-60.

1. Regards to the following EN,

Editor's note:	the criteria for determining whether a PDU Set is successfully delivered or not are FFS

Whether the PDU Set is considered as successfully delivered is depending on the network configuration and policy taking into consideration of the application request, i.e. the application may provide the ratio on the necessary PDUs within the PDU Set. If the measured ratio of the successful delivered PDUs within the PDU set is below the ratio provided by application, the PDU Set is considered as error or lost. If the ratio on the necessary PDUs within the PDU Set is not provided by the application, the network may consider the PDU Set as error or lost when any PDU within the PDU set is not successfully delivered. For GBR QoS Flows with Delay-critical GBR resource type, the PDU Set which is delayed more than PSDB is counted as lost, and included in the PSER unless the data burst is exceeding the MDBV within the period of PSDB or the QoS Flow is exceeding the GFBR.

In SA4 LS S2- 2210181, it replies that “although some FEC codes allow for static redundancy ratio, the K/N ratio is not always static during a media delivery session. For example, Video usually relies on Flex-FEC configurations. In such a case, the application is expected to update the 5GS with any configuration change.” So generally the ratio of the necessary PDUs within the PDU set provided by the application could be based on the FEC codes or other information, this ratio could be changed and provided by the application to the network if this information is needed.

Proposal 1: remove the EN and add the above descriptions in the PSER part.

2. Regards to the following EN,

Editor's note:	Whether support PDU Set identification information in new RTP is pending to SA WG4 5G_RTP WI.

Basically, we agree that the new RTP header could be considered, however, considering that the related work in SA4 has not concluded yet, we propose to add the following note in the conclusion.

NOTE:	The PDU Set identification information in new RTP can be considered in normative phase if SA WG4 5G_RTP WI is concluded in the R18 timeline.

Proposal 2: remove the EN and add the above NOTE in the conclusion.

3. Regards to the following EN,

Editor's note:	Which above PDU Set information parameters is optional is FFS.

According to SA4 LS S2- 2210181, since “PDU set size” is not explicitly signaled in the bitstream in the current RTP header, it is proposed to add it as optional parameters. “Start PDU and End PDU of the PDU Set” could be supported and used as the mandatory parameter.

Proposal 3: “Start PDU and End PDU of the PDU Set” should be mandatory in the PDU set information.

4. Regards to the following EN,

Editor's note:	Whether PDU Set importance is used for mapping different QoS Flows, sub-QoS Flows, or included in GTP-U header is FFS. (Potential SoH).

Within the service data flow, there exist different kinds of PDU sets which may require different QoS handling. If different PDU sets are separated into different QoS flows, these multiple QoS flows also need to be coordinated handling, e.g. when one flow fails to be established, how to handle other QoS flows. Considering the complexity of the handling of multiple QoS flows, it proposes to consider the single QoS flow enhancements. It is proposed to resolve the EN by adding the following Note in the conclusion.

NOTE:	PDU Set importance is included in GTP-U header and helps the RAN handling for different kinds of PDU Sets.

Proposal 4:  Add the above NOTE in clause 8.4.2.3.

1. Proposal

It is proposed to solve the ENs in conclusions for KI#4 “PDU Set integrated packet handling” and KI#5: “Differentiated PDU Set Handling” in TR 23.700-60.

*********** First Change***********

8.4	Conclusions for KI#4 and KI#5

The following aspects are concluded as principles for the normative work to support the following two key issues:

-	Key Issue #4: PDU Set integrated packet handling.

-	Key Issue #5: Differentiated PDU Set Handling.

NOTE:	Further PDU Set handling for Uplink will be studied and led by RAN WGs. SA WG2 can align with RAN's progress and decision for Uplink, if any.

Editor's note:	Whether and how to address the charging offset issue of DL PDU set eligible dropping by the NG-RAN is FFS.

8.4.1	Control plane enhancements for supporting PDU Set in downlink

8.4.1.1	PDU Set QoS Parameters

PDU Set QoS treatment is determined using dynamic or non-dynamic PCC.

The following PDU Set QoS parameters are defined to support PDU Set handling:

-	PDU Set Error Rate: The PSER defines an upper bound for the ratio between the number of PDU Sets not successfully received and the total number of PDU Sets sent towards a recipient measured over a measurement window. Whether the PDU Set is considered as successfully delivered is depending on the network configuration and policy based on the application request, i.e. the application can provide the ratio of the necessary PDUs to the size of the PDU Set for performing the correct decoding. If the measured ratio of the lost PDUs within the PDU set is below certain threshold, the PDU set is considered as not successfully delivered.

For GBR QoS Flows with Delay-critical GBR resource type, the PDU Set which is delayed more than PSDB is counted as lost, and included in the PSER unless the data burst is exceeding the MDBV within the period of PSDB or the QoS Flow is exceeding the GFBR.

-	PDU Set Delay Budget.

Editor's note:	The definitions of PSER and PSDB are FFS. For PSDB, it needs further study the impact due to N6 jitter.

-	Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication).

Editor's note:	It is FFS "Whether a PDU Set is still valid in case PSDB is exceeded" is needed. It should be discussed together with the definition of PSDB, specially about the boundary of PSDB.

If PDU Set based QoS handling is used, PCF determines the above PDU Set QoS Parameters based on information provided by AF (described in 8.4.2) and/or local configuration. The PDU Set QoS parameters are sent to SMF as part of PCC rule, then SMF sends them to RAN.

8.4.1.2	AF Information Provisioning

PDU Set related assistance information provisioning by AF is supported for dynamic PCC. AF may provision one or more of the following PDU Set related assistance information to NEF/PCF during AF QoS request procedure:

-	PDU Set QoS parameters listed in clause 8.4.1.1.

-	Burst periodicity.

8.4.2	User plane enhancements for supporting PDU Set in downlink

8.4.2.1	PDU Set Information

The following PDU Set related information may be identified by UPF to support PDU Set based handling:

-	PDU Set Identifier.

NOTE 1:	Neighbor PDU Sets in sequence will use different PDU Set identifiers.

-	Start PDU and End PDU of the PDU Set.

-	PDU SN within a PDU Set.

-	Optional, PDU Set Size.

NOTE 2:	Either PDU Set Size expressed in bytes or PDU Set Size expressed as number of PDUs, needs further determined.

NOTE 3:	Either one among Start/End PDU of the PDU Set and Number of PDUs within a PDU Set needs to be supported.

-	PDU Set Importance.

8.4.2.2	PDU Set Information identification on UPF and supported N6 protocols

The detection and marking of the DL PDU Sets sent to the NG-RAN shall be done by the PSA UPF.

PSA UPF may identify the PDU Set based on instruction from SMF and packet header of N6 protocols:

-	by matching RTP/SRTP header and payload (RFC 3550/3711/6184/7798/draft-ietf-avtcore-rtp-vvc/draft-ietf-avtext-framemarking are supported).

NOTE:	The PDU Set identification information in new RTP can be considered in normative phase if SA WG4 5G_RTP WI is concluded in the R18 timeline.

NOTE:	In above cases, it is assumed that the RTP/SRTP header and/or payload necessary for the identification of PDU Set Information is not encrypted.

-	by UPF implementation, e.g. PDU Set detection based on traffic characteristics. IP header parameters DSCP/TOS, IP port, IPv6 flow label may be used to detect PDU set, however detailed mechanisms in UPF for PDU Set information identification will not be standardized.

Editor's note:	Other N6 protocols, i.e. HTTP/MASQUE, GTP-U, IP/TCP/UDP/QUIC options, carrying PDU Set information are FFS. (Potential SoH).

8.4.2.3	Delivering PDU Set Information to RAN

PDU Set Information (listed in clause 8.4.2.1) are informed by UPF to RAN via GTP-U header of user plane packet.

NOTE:	PDU Set importance is included in GTP-U header and helps the RAN handling for different kinds of PDU Sets.

8.4.3	PDU Set based QoS handling

RAN performs PDU Set based QoS handling based on received PDU Set QoS Parameters via control plane, and PDU Set Information received via user plane. The details of RAN behaviours are defined in RAN WGs.

* * * * End of Changes * * * *