3GPP TSG-WG SA2 Meeting #154  	S2-2210602

Toulouse, France, November 14 – 18, 2022	(revision of S2-220xxxx)

Title:	[DRAFT] LS reply on Indication of Network Assisted Positioning method

Response to:	LS (S2-S2-2210172/C4-224626) on Indication of Network Assisted Positioning method from CT4

Release:	Rel-17

Work Item:	ID_UAS, 5G_eLCS_ph2

Source:	[Huawei to be] SA2

To:	SA3, CT4

Cc:

Contact Person:

Name:	Guanglei Li

Tel. Number:

E-mail Address:	liguanglei3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.273 CR0243, 23.256 CR0069

1. Overall Description:

SA2 thanks CT4 for the LS S2-2210172/C4-224626.

SA2 understands the high reliability requirement defined by SA3 for UAV tracking:

CT4 proposes to add the indication for GMLC/LMF to indicate 5GS to obtain reliable UE location information and implicitly indicate LMF to select Network Assisted Positioning method.

SA2 confirms that a reliability requirement indication is acceptable in the UAV tracking scenario and SA2 has aligned with SA3 to support the ‘reliability requirement’ in all three UAV tracking modes, see attached CRs.

However, SA2 would like to remind that network-assisted (i.e., RAN measurements) mode may not always satisfy QoS requirement. LMF may still select UE-assisted positioning modes and provide the selected positioning method in output and it is up to USS/UTM to determine the ‘reliability’. This is to align with the principle in previous Ls s3i210282 from SA3-LI for UE location verification under the 5GSAT_ARCH topic:

Separately from this discussion, and for the avoidance of doubt, LI generally requires the ability to report any location information available to the network (whether considered reliable or not), together with an indication of how the location was obtained so that the "reliability" of the location can be determined by Law Enforcement.

2. Actions:

To SA3 and CT4 group.

ACTION: 	SA2 kindly asks SA3 and CT4 group to take above information into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #154 Ad Hoc	Jan 16 – 20, 2023				eMeeting

TSG-SA2 Meeting #155		Feb 20 – 24, 2023				Athens, GR