3GPP TSG-WG SA2 Meeting #154	S2-2210606

# Toulouse, France, November 14 – 18, 2022		(revision of S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0247 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of GNSS assistance data collection from AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 153-E meeting, the following conclusions have been achieved for GNSS assistance data collection: LMF reuses the AF event exposure service to collect nearby GNSS assistance data. The eLCS_Ph3 will be commenced in 154 meeting, to align with the conclusion, it is proposed to add the procedures to support the GNSS assistance data collection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update LMF functionality and add a high level feature to support GNSS assistance data collection. - Add GNSS assistance data collection procedures for trusted and untrusted AFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LMF does not support to collect GNSS assistance data from AFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.8, 5.X(new), 6.X (new), 6.X.1(new), 6.X.2(new), 6.X.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.3.8	Location Management Function, LMF

The LMF manages the overall co-ordination and scheduling of resources required for the location of a UE that is registered with or accessing 5GCN. It also calculates or verifies a final location and any velocity estimate and may estimate the achieved accuracy. The LMF receives location requests for a target UE from the serving AMF using the Nlmf interface. The LMF interacts with the UE in order to exchange location information applicable to UE assisted and UE based position methods and interacts with the NG-RAN, N3IWF or TNAN in order to obtain location information.

The LMF shall determine the result of the positioning in geographical co-ordinates as defined in TS 23.032 [8] and/or in local co-ordinates as defined in TS 23.032 [8]. If requested and if available, the positioning result may also include the velocity of the UE. The coordinate type(s) is determined by LMF when receiving a location request, based on LCS Client type and supported GAD shapes. If the location request indicates regulatory LCS Client type the LMF shall determine a geographical location and optionally a location in local coordinates. For location request indicates a value added LCS Client type, the LMF may determine the UE location in local coordinates or geographical co-ordinates or both. If the supported GAD shapes is not received or Local Co-ordinates is not included in the supported GAD shapes, the LMF shall determine a geographical location.

NOTE 1:	Some RAT independent position methods (e.g. GNSS based position methods) can only determine a UE location in geographical co-ordinates. In such a case, the LMF may translate a UE location in geographical co-ordinates into a location in local co-ordinates when an origin for the local co-ordinates has known global coordinates. When an origin for the local co-ordinates does not have known global coordinates, position methods that can only determine a UE location in geographical co-ordinates cannot be used to determine a UE location in local co-ordinates.

Additional functions which may be performed by an LMF to support location services include the following.

-	Support a request for a single location received from a serving AMF for a target UE.

-	Support a request for periodic or triggered location received from a serving AMF for a target UE.

-	Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type.

-	Report UE location estimates directly to a GMLC for periodic or triggered location of a target UE.

-	Support cancelation of periodic or triggered location for a target UE.

-	Support the provision of broadcast assistance data to UEs via NG-RAN in ciphered or unciphered form and forward any ciphering keys to subscribed UEs via the AMF.

-	Support change of a serving LMF for periodic or triggered location reporting for a target UE.

-	Support of receiving stored UE Positioning Capability from AMF and support of providing updated UE Positioning Capability to AMF.

-	Map the UE location to a geographical area where the PLMN is or is not allowed to operate based on the request from AMF.

-	Support determination of a UE location at a scheduled location time.

-    Support collection of GNSS assistance data from AFs.

NOTE 2:	Country, area within a country, or an international area can be supported as different types of geographical area.

# * * * * Second change * * * *

5 High Level Features

5.X Collection of GNSS assistance data

LMF can collect GNSS assistance data by reusing Data Collection from AF as described in clasue 5.20a of TS 23.501 [18] and Naf_EventExposure service described in clause 5.2.19.2 of TS 23.502 [19].  The collection may be performed on a periodic basis.

After the collection, LMF may send the collected GNSS assistance data to UEs via the procedures described in clauses 6.14.1 (Broadcast of Assistance Data by an LMF) or 6.11.1 (UE Assisted and UE Based Positioning Procedure) during a positioning session.

LMFs collected GNSS assistance data may update its NF profile in NRF by indicating the TAs of which corresponding GNSS assistance data exists, then the LMF can be discovered and its collected GNSS assistance data can be used by other LMFs not having corresponding GNSS assistance data.

# * * * * Third change * * * *

6.X	Procedures for GNSS assistance data Collection

6.X.1	GNSS assistance data collection from untrusted AF via NEF

The following procedure is used by LMF to collect GNSS assistance information from AFs via the NEF.

Figure 6.X.1-1: GNSS assistance data collection from untrusted AF

1a-1c.	Reuse the steps 1a-1c in figure 6.2.2.3-1 of TS 23.288 [x] with following difference:

- The AF should also provide its serving area to NEF. NEF should map the serving area into TA(s) if it is presented as geographical location and update the NEF profile in NRF including the serving area information.

1d-1e.	Reuse the steps 1d-1e in figure 6.2.2.3-1 of TS 23.288 [x] with following difference:

- When LMF needs to discovery the available data from AFs and the appropriated NEF to collect GNSS assistance data, LMF additionally provides TAs in the Nnrf_NFDiscovery_Request_request service operation.

2-5. Reuse the steps 2-5 in figure 6.2.2.3-1 of TS 23.288 [x] with following difference:

- Besides available GNSS assistance data, AF also provides associated serving area of the GNSS assistance data and optional precise global coordinates of the source of the GNSS assistance data. NEF may map the serving area into a TA or TA list.

6. Afthe LMF gets the GNSS assistance data, it can send the assistance data to UE via the procedures described in clauses 6.14 or 6.11.1 during a positioning session.

# * * * * Third change * * * *

6.X.2	GNSS assistance data collection from trusted AF

For trusted AFs, LMF collects GNSS assistance data through AFs directly. LMF discovers trusted AFs covering LMF serving area through NRF and subscribes to GNSS assistance data exposure from discovered trusted AFs.

Figure 6.X.2-1: GNSS assistance data collection from trusted AF

1a-1c. 	AF invokes Nnrf_NFManagement_NFUpdate_request service operation to register its GNSS releated information including Event IDs, and associated AF identification, Application ID(s) and its serving area (TAs). NRF stores the received AF update information and sends Nnrf_NFManagement_NFUpdate_response message to AF.

1d-1e.	When LMF needs to discovery the available GNSS assistance data from AFs, LMF provide TAs to NRF through Nnrf_NFDiscovery service to discover related AFs.

2. 	LMF subscribes to or cancels subscription to GNSS assistance data in AF by invoking the Naf_EventExposure_Subscribe/ Naf_EventExposure_Unsubscribe service operation.

3. 	AF notifies the LMF with the available GNSS assistance data by invoking Naf_EventExposure_Notify service operation. AF also provides associated serving area (TAs) of the GNSS assistance data and optional precise global coordinates of the source of the GNSS assistance data. The AF notifies the LMF with the data by invoking Nnef_EventExposure_Notify service operation.

4. 	Afthe LMF gets the GNSS assistance data, it can send the data to UE via the procedures described in clauses 6.14 or 6.11.1 during a positioning session.

# * * * * Fouth change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.071: "Technical Specification Group Systems Aspects; Location Services (LCS)".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[5]	3GPP TS 43.059: "Functional Stage 2 description of Location Services in GERAN".

[6]	Void.

[7]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[8]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[9]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[10]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[11]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[12]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[13]	OMA MLP TS: "Mobile Location Protocol", [http://www.openmobilealliance.org].

[14]	Void.

[15]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[16]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[17]	3GPP TS 25.305: "Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[18]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[19]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[20]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[21]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[22]	Void.

[23]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[24]	3GPP TS 23.222: "Common Application Programming Interface (API) framework for 3GPP northbound APIs".

[25]	RFC 2396: "Uniform Resource Identifiers".

[26]	RFC 3261: "SIP: Session Initiation Protocol".

[27]	3GPP TS 23.228: "IP multimedia subsystem (IMS)".

[28]	3GPP TS 23.003: "Numbering, addressing and identification".

[29]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[30]	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

[31]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[32]	Void.

[33]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[34]	Void.

[35]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[36]	3GPP TS 24.571: "5G System (5GS); Control plane Location Services (LCS) procedures; Stage 3".

[x]	3GPP TS 23.288: " Architecture enhancements for 5G System (5GS) to support network data analytics services".

# * * * * End of changes * * * *