SA WG2 Meeting #SA2-154 	S2-2210611

Nov 14 – 18, 2022,

Source:	China Mobile,

Title:	Conclusion update for KI#2

Document for:	Approval

Agenda Item: 	9.19.1

Work Item / Release:	FS_XRM

Abstract of the contribution: This document proposes conclusion update for KI#2

- Discussion

Generally, to support the load balance in real network, the PDU sessions which belong to multiple UEs may select difference PCFs. And these PCFs are supposed to access the same UDR. If the PCFs not belong to single UDR, which imply that the PCFs are deployed between a larger service area and the coordination policy is useless.

So it is suggested to reuse the interaction between PCF and UDR, and UDR is responsible to coordinate the policy parameters and then notify these update to corresponding PCF.

- Proposal

It is proposed to capture the following aspects in TR 23.700-60.

# * * * * Start of change * * * *

8.2	Conclusions for Key Issue#2

The following aspects are concluded as principles for the normative work:

-	For multi-modality flows transmitted by multiple UEs, the AF manages the flows in the following ways:

-	The AF procedure is extended with QoS requirements for individual service flows and treatment requirements and policies according to the conclusions for Key Issue #1.

-	In case multiple UEs are involved, AF provides the information related to all UEs to NEF either using a single procedure or using multiple UE specific procedures.

-	In either case, the AF may indicate that specific service data flows belong to the same multimodal service by associating them with a common ID. PCF uses the individual service data flow QoS requirements and requirements and policies in the same way as concluded for Key Issue #1.

-	Multiple PCFs may be involved for multiple UEs. If the policy coordination is required, the PCFs can update the policy based on UDR.

# * * * *end of change * * * *