3GPP TSG-WG SA2 Meeting #154	S2-2210616

# Toulouse, France, November 14 – 18, 2022		(revision of S2-221xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0144 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe UE-to-UE Relay reference architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR adds 5G ProSe UE-to-UE Relay reference architecture based on the KI#1 conclusion in TR 23.700-33. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5G ProSe UE-to-UE Relay reference architecture is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe UE-to-UE Relay reference architecture is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x (new), 4.2.x.1 (new), 4.2.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.2.X	5G ProSe UE-to-UE Relay reference architecture

#### 4.2.X.1	Non-roaming reference architecture

The following figure 4.2.X.1-1 shows the high level non-roaming reference architecture for 5G ProSe UE-to-UE Relay. In this figure, 5G ProSe End UE A, 5G ProSe End UE B and 5G ProSe UE-to-UE Relay use a subscription of the same PLMN.

This reference architecture applies to both 5G ProSe Layer-2 UE-to-UE Relay and 5G ProSe Layer-3 UE-to-UE Relay.

Figure 4.2.X.1-1: Non-roaming reference architecture for 5G ProSe UE-to-UE Relay

#### 4.2.X.2	Roaming reference architecture

The following figure 4.2.X.2-1 shows the high level roaming reference architecture for 5G ProSe UE-to-UE Relay. In this figure 5G ProSe UE A uses a subscription of HPLMN and 5G ProSe End UE B and 5G ProSe UE-to-UE Relay use a subscription of VPLMN.

This reference architecture applies to both 5G ProSe Layer-2 UE-to-UE Relay and 5G ProSe Layer-3 UE-to-UE Relay.

Figure 4.2.X.2-1: Roaming reference architecture for 5G ProSe UE-to-UE Relay

# * * * * End of changes * * * *