3GPP TSG-SA2 Meeting #154 	S2-2210630

# Toulouse, France, Nov 14 – 18, 2022													  (was S2-220xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0578 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for for analytics collection from MDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Implement the conclusion for KI#10: Interactions with MDAS/MDAF in clause 8.10 of TR 23.700-81. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Focus on the procedure for for analytics collection from MDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure is unknown on how the NWDAF consumes analytics from the MDAS/MDAF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.X (new), 6.2.X.1 (new), 6.2.X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is part of outcome of SI FS_DUMMY and the WI code for the work is not assigned yet & will replace WI code DUMMY when available |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[6]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[7]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[8]	ITU-T Y.1540: "Internet protocol data communication service - IP packet transfer and availability performance parameters".

[9]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[10]	3GPP TS 23.003: "Numbering, addressing and identification".

[11]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[12]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[13]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)".

[14]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[15]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[16]	3GPP TS 38.314: "NR; Layer 2 Measurements".

[17]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[18]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[19]	3GPP TS 23.008: "Organization of subscriber data".

[20]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[21]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[23]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[24]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[25]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[26]	3GPP TS 28.532: " Management and orchestration; Generic management services;".

[27]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[28]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[29]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[xx]	3GPP TS 28.104: " Management and orchestration; Generic management services;".

* * * * Next Change * * * *

6.2.X	Analytics Collection from MDAF

6.2.X.1	General

The MDA functional overview and service framework in Figure 5.1-1 as defined in TS 28.104 [xx] is used by NWDAF to trigger the MDA MnS to request analytics from the MDA Management Function.

Before NWDAF requests analytics from the MDA Management Function, the NWDAF firstly discover the MDA Management Function via the MnS discovery service producer as defined in clause 5 of TS 28.537 [28].

6.2.X.2	Procedure for analytics collection from MDAF

Figure 6.2.X.2: Procedure for analytics collection from MDAF

1.	NWDAF discovers the MDA Management Function from the MnS discovery service producer by sending a MnS producer discovery service operation. The service operation may include the following parameters:

-	requestedMDAType: indicate a specific MDA capability such as Slice coverage analysis, Mobility performance analysis as defined in clause 7.2 of TS 28.104 [xx];

-	Area of Interest;

-	Network Slice information (i.e. NetworkSliceInfo including a DN (Distinguished Name) of the NetworkSlice managed object relating to the network slice instance associated to the S-NSSAI and NSI ID if available as defined in TS 28.541 [29]).

2.	The MnS discovery service producer feedback the address of the MDA Management Function.

3.	NWDAF requests analytics from the MDA Management Function by triggering a MDARequest service operation as defined in clause 9.3.2 of TS 28.104 [xx]. The service operation may include the following parameters:

-	requestedMDAOutputs: MDAOutputPerMDAType;

-	reportingMethod: periodic reporting mode, the periodicity or specified time of the report, etc.;

-	analyticsScope: Area of Interest, Network Slice information, etc.;

-	startTime;

-	stopTime;

-	analyticsWindow.

4.	The MDA Management Function provides the analytics to the NWDAF by triggering an MDAReporting service operation.

NOTE:	NWDAF and MDA Management Function/MnS discovery service producer, if from different operators, may interact via the Exposure Governance Management Function as defined in clause A.3 of TS 28.533 [27].

* * * * End of Change * * * *