3GPP SA WG2 Meeting #154  			   	   S2-2210636

Toulouse, France, November 14-18, 2022

Source:	Samsung, OPPO, Oracle, LG Electronics, ETRI, Tencent

Title:	KI#7 Conclusion: Assistance to Member Selection

Document for:	Approval

Agenda Item:	9.21

Work Item / Release:	FS_AIMLsys

Abstract of the contribution: This contribution provides a conclusion proposal to KI#7.

# 1.	Discussion

This contribution provides a conclusion proposal to KI#7 on assistance to member selection

# 2 Proposal

It is proposed to include the below changes to TR 23.700-80.

FIRST CHANGE

## 8.7	Key Issue #7: 5GS Assistance to Federated Learning Operation

### 8.7.X	Assistance to Member Selection

It is concluded that an AI/ML AF performing FL operation may request the 5GC to provide assistance with the selection of the UEs participating in the FL operation by triggering the FL member selection assistance functionality in the 5GC.

FL member selection assistance is defined as a functionality offered by the 5GC upon receiving a request from the AI/ML AF to provide a list of candidate UEs and additional information to enable 5GC to assist the AI/ML AF to perform FL operation. After receiving the AI/ML AF request, a 5GC NF may then trigger corresponding 5GC procedures to derive the candidate UE list as part of the assistance information to be reported back to the AIML AF.

A new 5GC service is required to provide FL member selection assistance to the AF. The following characteristics apply to the proposed service:

-	The AF uses a request or subscription service operation to trigger the FL member selection assistance functionality. The 5GC provides the assistance to the AF by using a notify service operation.

-	The AF may include the following parameters as part of the request or subscription service operation to the 5GC:

-	Initial UE list (a UE or a UE list or any UE)

-	Optionally, the following additional parameters can be provided: UE(s) consent, UE(s) connection status (e.g. S-NSSA, DNN, MA configuration, access type), UE(s) location, target area(s), QoS requirements, time validity, etc.

-	The 5GC may provide the following parameters as part of the notify service operation:

-	List(s) with candidate UEs to be selected by the AF for the FL operation.

-	Recommended time window(s) per candidate UE to be allocated by the AF for the FL operation.

-	Optionally, additional information per candidate UE per time window may be provided: UE(s) consent, UE(s) connection status (e.g. S-NSSA, DNN, MA configuration, access type), UE(s) location, QoS, etc.

NOTE 1:	The 5GC service providing FL member selection assistance to the AF shall comply with the list of General Architecture Principles contained in clause 8.0.

NOTE 2:	The complete list of parameters provided in each service operation may be finalized during normative work.

The new 5GC service providing FL member selection assistance can be hosted in a new NF, NWDAF, or NEF. If needed, the entity providing such service may collect additional information from other 5GC NF to respond to the AF request or subscription.

In Release 18, the above new 5GC service may be also leveraged to provide UE selection assistance to the AI/ML AF for AI/ML operations other than FL defined by SA1 in TS 22.261 [2], clause 6.40.

END OF CHANGES