3GPP TSG-WG SA2 Meeting #154 e-meeting 	S2-2210697

Toulouse, November 14th -18th, 2022	(revision of S2-220xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#1 Updated evaluations and conclusions

Document for:	Approval

Agenda Item:	9.14

Work Item / Release:	FS_eNS_Ph3/ Rel-18

Abstract:  This paper provides update to KI#1 evaluations and conclusions related to the aspects of bullets 2,3 in the conclusions.

# 1. Introduction/Discussion

At SA2#153 initial conclusions were drawn for KI#1.

There are several open issues needing resolution.

1) Do we need Change of NSI?

If it is to face C-plane load, the proposed solution increases load in the current overloaded NSAI, hence it is not a good approach. if it is to address UP load, it is unclear why the UP load becomes unevenly balanced in the NSUI and the others are not suffering from the same peak traffic. it looks like that these short-term events should not be addressed in this manner and in any case the even load balancing should be pursued at PDU session establishment. We propose not to proceed with bullet 2 in the conclusions

2) Do we need to change S-NSSAI to use resources in same SMF'/UPF?

Like for the case of resource shortage in the RAN, we propose that when the functions are not changed, we use resource repartitioning approach. We propose not to proceed with bullet 2 in the conclusions

3) What services are used between AMF and PCF to trigger temporary S-NSSAI replacement?

We propose this is contribution driven in normative phase.

4) How to address the case of UEs not yet updated with the new replaced S-NSSAI?

We propose the PDU session is established as normal with the old S-NSSAI value, then the replacement steps are triggered as for the other established PDU sessions for the old S-NSSAI.

5) Should we use the mapped value defined in rel-15 or explicitly indicate the replacement S-NSSAI?

Since the replacement S-NSSAI may be in the subscription of the UE, this is not well tailored to reusing the recently removed behaviour in the HPLMN. Hence, we propose that an explicit indication of replacement is provided to the UE.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-41

# * * * * First change * * * *

## 7.1	Evaluation for KI#1

Solutions #1, #2, #3, #4, #5, #15, #32, #40, #41, #42 and #43 are documented to address all or some of the scenarios 1b), 1c) and 2d) from the KI#1 description.

The Table 7.1-1 shows the evaluation of the solutions.

Table 7.1-1: Evaluation of solutions for Key Issue 1

| Solution #1 Additional S-NSSAI associated with the PDU session | The main principle is that a PDU session can be associated with multiple S-NSSAIs in the network (i.e. NG-RAN, AMF, SMF, UPF). During PDU session establishment or HO procedure, the AMF notifies the new S-NSSAI to SMF. The SMF provides the new S-NSSAI to the NG-RAN. The network does not notify the change of S-NSSAI in the UE while NG-RAN, AMF, SMF and UPF update the S-NSSAI of the PDU Session. The solution applies to certain deployments where the old S-NSSAI and the new S-NSSAI are associated with the same NSI (e.g. the new S-NSSAI uses shared resource of the NSI, whereas the old S-NSSAI uses dedicated resource of the NSI). One benefit is that the network can apply the procedure of PDU Session transfer to the new S-NSSAI for legacy UEs, as there are no UE impacts. This solution requires that the new S-NSSAI is part of the Allowed NSSAI. The solution assumes that all the S-NSSAIs supporting slice remapping establish end-to-end availability in the combination of OAM and signalling among network functions, i.e. the SMF and the UPF shall always support both original S-NSSAI and new S-NSSAI. This solution uses different S-NSSAI towards the UE compared to S-NSSAI used in the network. It also assumes the same SMF and UPF is used, and IP address is not changed! it seems then that since UP and CP resources in same functions are used, like for the case of Resources repartitioning used to address the resource shortage use case in RAN3, we should apply the same concept here in SMF+UPF. |
| --- | --- |
| Solution #2 Slice Re-mapping Capabilities for Network Slice Service Continuity | The main principle is that the AMF derive a new/alternative S-NSSAI for a congested S-NSSAI optionally requesting the AM PCF (which specifically supports slice re-mapping), where the new/alternative S-NSSAI has to support the same DNN, DNAI and selected within Allowed NSSAI. Then, the AMF requests the current SMF to change the PDU Session to the new S-NSSAI. The SMF triggers the PDU session transfer to the new S-NSSAI without UPF relocation (i.e. Option 1, re-use the existing PDU session) and with UPF relocation (i.e., Option 2, new PDU session established similar to SSC mode 3) based on the SSC mode of the PDU session and sends to the UE a PDU Session Modification Request including the corresponding cause value and the new S-NSSAI. The solution also proposes to update the URSP rules of the UE by means of UE Policy PCF trigger of slice re-mapping event. Further, in the solution, AM PCF based new S-NSSAI selection requires that the AM PCF has knowledge about the S-NSSAIs configuration. It is not specified when the AMF triggers the slice remapping procedure. Besides, the UE may not use updated URSP immediately. It is unclear how the service continuity is supported in this case. |
| Solution #3 Support of Network Slice Service continuity using a SSC mode 3 type of Service continuity | The main principle is that the AMF determines the need to exchange old/original S-NSSAI 1 with a new/alternative S-NSSAI 2 (and in case of mobility to T-RAN, the T-RAN temporarily accepts the PDU Session of S-NSSAI 1 and indicates in the Path Switch Request to the AMF an alternative S-NSSAI 2). The AMF triggers UCU procedure to the UE to include both S-NSSAI 1 and S-NSSAI 2 in the Allowed NSSAI, and afterwards the AMF notifies the SMF about the end of usage of S-NSSAI 1 and to use the new S-NSSAI 2. The solution proposes to notify the UE of slice re-mapping by enhancing the Allowed NSSAI format and using the NAS MM signalling, and in addition, the solution proposes to also use SM signalling from the SMF (SSC mode 3 like procedure) to update the UE about the S-NSSAI change/remapping. The question is whether a single NAS signalling is not enough to reconfigure the UE about the S-NSSAI change, i.e. either using NAS MM or NAS SM signalling may be sufficient. As the network slice configuration is performed in the NAS MM-sublayer, it would aligned with the existing network slice configuration to use the NAS MM signalling from the AMF to the UE. Additionally, it is not clear whether the impacts to the NG-RAN (notification about slice re-mapping) are required, since there is no NG-RAN specific behaviour to support slice re-mapping. The AMF itself can determine the alternative S-NSSAI 2 as in other solutions (e.g. solution #1, 32, 42, etc.). How to do slice remapping for N2 based handover is FFS. |
| Solution #4 PDU Session on compatible network slice | [Non-mobility use case] During new PDU Session Establishment, if there are further RSDs for the matching URSP rule, the UE includes in the NAS message to the AMF an alternative S-NSSAI (which is stored in the UE's context in the AMF). If the original S-NSSAI is congested, the AMF may use the alternative S-NSSAI to select an SMF and continue with the PDU Session establishment on the alternative S-NSSAI. The solution covers the use case of a new PDU Session establishment. The following analysis is provided: - If the alternative S-NSSAI (sent from the UE to the AMF) is part of the Rejected NSSAI or not available in the current TA, the AMF cannot use such UE-indicated alternative S-NSSAI. Therefore, the AMF itself is required to have the ability to determine an appropriate alternative S-NSSAI. As result, the indication of the alternative S-NSSAI from the UE to the AMF applies to limited scenarios and it is not necessarily needed. - It is proposed that for a new PDU Session establishment on the old/current S-NSSAI, the AMF determines to apply the new/alternative S-NSSAI and select an SMF accordingly. It is not clear what is the UE behaviour when the UE receives the PDU Session Establishment Accept message with an S-NSSAI different from the requested S-NSSAI. Also, the UE may have applied cell selection using NSAG feature for the old S-NSSAI, however, the new S-NSSAI may be served preferably on another cell/frequency layer. Therefore, it may be more efficient if the AMF rejects the new PDU Session establishment request with an indication to the UE to use the new/alternative S-NSSAI replacing the old S-NSSAI; and then the UE would apply the new S-NSSAI for the PDU Session establishment appropriately. This solution is not applicable when the matching URSP rule, which triggers the PDU Session establishment, is associated with a single S-NSSAI in the URSP rule. |
| Solution #5 PDU session handover to a target CN with an alternative S-NSSAI support | [Mobility use case, scenario 2d] This solution is similar to solution #4 with the addition that the T-AMF may select a different SMF based on the alternative S-NSSAI, if the PDU Session switches to an alternative S-NSSAI. The old SMF triggers PDU Session modification procedure. It is unclear how (e.g. in step 7) the T-AMF selects a new SMF and in parallel the old SMF triggers PDU Session modification procedure, i.e. it seems there is concurrent SM signalling for the same PDU Session. Also, during the Registration procedure with the T-AMF, it is not clear whether and how the UE includes the alternative S-NSSAI in the Requested NSSAI. This solution is not applicable when the matching URSP rule, which triggers the PDU Session establishment, is associated with a single S-NSSAI in the URSP rule. |
| Solution #15 Service continuity in case of Network Slice instance overload | It is assumed that multiple NSIs are deployed for the same S-NSSAI. The current NSI selection is done without being able to consider load balancing after the UE registration/PDU Session Establishment. The solution describes how the NSI can be changed. When the existing PDU session is decided as to be migrated to another Network Slice instance, the SMF performs PDU Session re-establishment by using the mechanism of SSC mode#2 or SSC mode#3. Then the PDU session establishment request arrives at the AMF, the AMF removes from the UE context the old NSI ID and the AMF can select another NSI. This solution is not applicable for SSC mode#1. The service continuity is ensured at application layer. The issue with this solution is that if this is to address C-plane load in the NSI, it causes MORE C-plane load in the NSI as it triggers signalling for settled PDU sessions. If it is to handle load in the U-plane, it is questionable that the solution is to move sessions from one NSI to another rather than changing the weight factors. also it seems to assume that due to some "unlucky" event the load affect disproportionately one NSI and this is statistically unlikely as the load balancing at session setup aims at evenly loading the NSIs. it also assumes the traffic is not subject to any application layer flow control that is unlikely. The right approach for operators would be to ensure that the AMF can be on NSIs where the expected SMFs serving the S-NSSAIs it supports are and that OAM modifies at run time the load balancing weights as it is already possible today. So if the network is configured this way, there is no impact to standards specifications, and all works well since rel-15 by ensuring even load balancing across NSIs. |
| Solution #32 Solution for Network Control for UE Slice Use | Three options are described in this solution: - Option 1: the UE-initiated procedure, i.e. when the UE establishes a new PDU Session the UE is configured to use the new S-NSSAI. The new S-NSSAI is provided to the UE by the AMF during enhanced UCU procedure indicating that PDU session transfer is requested from an old S-NSSAI (i.e. to be removed) to the new S-NSSAI. - Option 2: AMF-initiated procedure towards SMF, i.e. the AMF notifies the SMF to initiate SSC mode 3 like procedure so that the UE triggers PDU Session Establishment by using target slice. This option is similar to solutions # 2, 3, 4, 5. - Option 3: PCF-initiated procedure, i.e. the PCF notifies the SMF that an alternative S-NSSAI is to be used. As in Option 2, the SMF uses enhanced NAS SM procedure to notify the UE to initiate a new PDU session towards the alternative S-NSSAI. For the UE-initiated option, it is not clear how to apply the procedure for existing PDU Sessions. If Option 1 and Option 2 are combines, then the result would be similar to Solution #3. Please refer to the evaluation of solution #3. Option 3 is similar to and solution #40. Therefore similar evaluation would apply as to solution #2 and solution #40. The impact of overriding of the SSC mode is FFS. |
| Solution #40 S-NSSAI change decided by PCF | The main principle is that the PCF determines (e.g. triggered by a SMF or NWDAF) whether the S-NSSAI associated to an ongoing PDU session needs to be changed and which is the alternative/replacement S-NSSAI. Then, the PCF provides the suggested new S-NSSAI to the SMF, and the SMF initiates the modification of the PDU session and notifies the AMF. The solution addresses scenarios 1b) and 1c). The PCF for a Session may only serve the original S-NSSAI. It is not clear how the PCF can determine the new S-NSSAI in the roaming case and how the PCF can decide the mapped new S-NSSAI. |
| Solution #41 Network Slice change without service interruption | The main principle is that the AMF determines the new/alternative S-NSSAI to be used to replace the congested S-NSSAI and: - (option 1) if the current SMF (PSA) supports the new S-NSSAI and the capability to change S-NSSAI, then AMF requests SMF to change to the new S-NSSAI. The SMF initiates SM procedure towards the UE to notify the UE that the S-NSSAI has been changed. This case is similar to solution #2, but solution #41 informs the UE about the S-NSSAI change. - (option 2) if the current SMF (PSA) does not support the new S-NSSAI and/or the capability to change S-NSSAI, the AMF invokes procedure to initiate PDU Session transfer using any SSC mode as described by other solutions (e.g. solutions 3, 4). The solution addresses scenarios 1b, 1c, 2d. In option 1, similar as Solution #2, it is not clear what is the behaviour of the UE after receiving the PDU Session Modification request from the SMF indicating change of the S-NSSAI. |
| Solution #42 Network controlled change to an alternative S-NSSAI | The main principle is that the AMF (or together with the NSSF or OAM system) determines an alternative S-NSSAI (e.g. S-NSSAI-2) which is available in the current TA and can be used by the UE (e.g. S-NSSAI is not part of the Rejected NSSAI, or NSSAA has failed, etc.). If the NSSF is used to determine the alternative S-NSSAI, the AMF may include a new indication to the NSSF that an alternative S-NSSAI to S-NSSAI-1 is required (this is similar to solution #43). Two options are proposed: - (Option 1) If the S-NSSAI-2 is part of the UE's subscribed S-NSSAIs, it is assumed that the S-NSSAI-2 is part of further RSD the same URSP rule which is used for established PDU Session. The AMF triggers SM procedure towards the SMF to indicate that the PDU Session on S-NSSAI-1 should be released and alternative S-NSSAI may be used. The SMF releases the PDU Session with an indication to the UE to establish the PDU Session on an alternative S-NSSAI. - (Option 2) If the S-NSSAI-2 is not part of the UE's Subscribed S-NSSAIs, the AMF triggers UE network slice reconfiguration procedure (e.g. UCU procedure) in order to send a new Configured NSSAI with a corresponding Mapping of the Configured NSSAI information containing the mapping of S-NSSAI-2 to S-NSSAI-1 (and Mapping Of the Allowed NSSAI). The AMF may wait with the UE reconfiguration when the PDU Session(s) on the old S-NSSAI are inactive. In Option 1, it is not assured that the URSP rule includes an RSD with S-NSSAI-2 and also, the Option 1 works only when the RSD with S-NSSAI-2 in the URSP rule is configured to match the traffic to be moved in S-NSSAI-1. Therefore, it may be more deterministic if the option 2 is always performed, i.e. the UE should be updated to include in the Allowed NSSAI the S-NSSAI-2 and the mapping of S-NSSAI-2 to S-NSSAI-1. There can be another alternative that S-NSSNI-2 is part of Configured S-NSSAI then legacy behaviour can be used. In Option 2, in addition to the MM reconfiguration in the UE, in case of existing PDU Session(s) on the old S-NSSAI-1, the AMF may indicate to the SMF that an explicit PDU Session release is required and the SMF releases the current PDU Session with re-establishment indication. This results in more deterministic behaviour that the UE riggers a new PDU Session establishment to the old S-NSSAI with a mapped new S-NSSAI value, i.e. not relying on the UE implicit trigger of PDU Session re-establishment after receiving the new Allowed NSSAI. Option 2 describes that the S-NSSAI-2 is provided to the UE in the Mapping Of Configured/Allowed NSSAI" information when the S-NSSAI-2 is not within the Subscribed S-NSSAI to UE. However, if the S-NSSAI-2 is part of the Subscribed S-NSSAIs, the Mapping of the Allowed NSSAI information can be also provided to the UE, similar to solutions #3 and #32. The S-NSSAI-2 may not be part of the Subscribed S-NSSAIs in the scenarios where the UE has a single Subscribed S-NSSAI, namely S-NSSAI-1. |
| Solution #43 Allowed NSSAI Determination in Initial Registration to Support Network Slice Service Continuity | The AMF queries the NSSF to obtain alternative S-NSSAI for each S-NSSAI of the Allowed NSSAI. The Allowed NSSAI, which is sent to the UE, contains two parts: (1) S-NSSAIs of network slices which UE can access and (2) alternative S-NSSAIs to support slice continuity. The USRP provided to UE contains only the part (1). It may be inefficient to always query the NSSF about the alternative S-NSSAI for each S-NSSAI from the Allowed NSSAI, since the cases to use the alternative S-NSSAI are rather rare, and therefore, it is considered more efficient to query the alternative S-NSSAI from the NSSF on demand. This solution does not describe how the UE uses this secondary S-NSSAI in the allowed NSSAI, i.e. whether the existing " Mapping Of Allowed NSSAI" information is used. |

Regarding the Network Function, which determines the alternative S-NSSAI to replace the old S-NSSAI, the solutions can be categorized into 2 groups:

-	The AMF determines the alternative S-NSSAI (e.g. solutions #1, #2, #3, #4, #5, #32, #42, #43, etc.). The AMF may optionally use the NSSF services to determine the alternative S-NSSAI (e.g. solutions #42, #43). The AMF may optionally use the PCF services to determine the alternative S-NSSAI (e.g. solutions #2, #41). The AMF may be triggered by the RAN (solution #3) or by the NWDAF.-	The PCF determines the alternative S-NSSAI (e.g. solution #40). As documented in the#40 in Table 7.X-1, it is unclear how the PCF knows whether the alternative S-NSSAI is supported in the current TA and the RAN node.

Regarding the impacts to the UE, the solutions can be categorized into the following groups:

-	Solutions without impact to the UE:

-	For scenarios where the same S-NSSAI is deployed on multiple NSIs and NSI change is performed (e.g. solution #15): the NSI change is performed seamlessly to the UE, although there is signalling to the UE, but it uses legacy signalling.

-	For scenarios where the old and alternative S-NSSAIs are deployed on the same NSI (e.g. solution #1): the principle is that the change of the old S-NSSAI to alternative S-NSSAI is handled within the 5GC and NG-RAN and transparent to the UE.

-	Solutions with impact to the UE (e.g. solutions #2, #3, #4, #5, #32, #41, #42, etc.): the UE is informed either by using NAS MM signalling from AMF and/or by using NAS SM signalling from the SMF that the old S-NSSAI is to be replaced by an alternative S-NSSAI. It is beneficial if only a single configuration is performed to the UE. Usually, the NAS SM signalling is meant to influence a single PDU. It appears beneficial to use both:

-	NAS MM signalling to include the alternative S-NSSAI in the Allowed NSSAI, the alternative S-NSSAI replaces the old S-NSSAI.

-	NAS SM signalling is used to release the PDU Session with an indication to use an alternative S-NSSAI (e.g. as per NAS MM configuration).

In solutions #4, #5 and #32, it is described that the AMF may provide back off timer for the old S-NSSAI and/or includes the old S-NSSAI in the Rejected NSSAI to prevent the UE requesting the old S-NSSAI due to URSP rule re-evaluation. However, if the old S-NSSAI is included in the Rejected NSSAI or back-off timer is configured, and the URSP rules have only the old S-NSSAI in the RSDs, the UE will not be able to establish a new PDU Session for the matching URSP rule, as the RSDs will be determined as invalid. Such solution is only applicable if the URSP rule(s) or local configuration contains multiple RSDs including the old S-NSSAI and the alternative S-NSSAI.

NOTE:	Given the limitation of this solution (i.e. include the old S-NSSAI in the Rejected NSSAI or using of BOT), it would be beneficial if the old S-NSSAI is excluded from the Allowed NSSAI but included as replaced (or mapped) S-NSSAI value in the Mapping OF Allowed NSSAI (or a new IE), i.e. similar to the mapped S-NSSAI value sent in the existing mapping of Allowed NSSAI.

# * * * * next change * * * *

## 8.1	Conclusions for KI#1

The following principles are concluded for the normative work.

NOTE:	The IE names for the alternative S-NSSAI and other new information in the conclusion is to be determined during normative phase.

0)	The trigger to replace a currently used S-NSSAI with an alternative S-NSSAI is either based on local configuration (e.g. based on trigger from OAM) or based on PCF notification to AMF. The service used for PCF-AMF interaction will be defined in normative phase.

1)	The AMF determines the alternative S-NSSAI to be used to replace the old S-NSSAI. The AMF may interact with the PCF and/or with the NSSF to determine the alternative S-NSSAI. The alternative S-NSSAI may or may not be part of the Subscribed S-NSSAIs of the UE. The Subscribed S-NSSAIs of the UE are not updated for the purpose of KI#1. Since it cannot be assumed that the replacement S-NSSAI is outside the UE subscribed S-NSSAIs in HPLMN (which is why the mapped value was introduced in HPLMN) the replacement S-NSSAI for the short-term issues identified in this KI shall be indicated explicitly to the UE

2)	In cases where the old S-NSSAI is replaced by an alternative S-NSSAI deployed on a different NSIs, the following principles apply:

a)	If the UE has indicated support of the transfer of PDU sessions between network slices, The AMF performs UE MM configuration update (e.g. UCU procedure) to include the alternative S-NSSAI in the Allowed NSSAI and/or in the Configured NSSAI, if not included yet. The AMF provides information that the alternative S-NSSAI is associated with a replaced/mapped value to the old S-NSSAI in both roaming or non-roaming case; and independent whether the alternative S-NSSAI is part or not part of the Subscribed S-NSSAIs.

-

b)	For a new PDU Session establishment request from a UE that is not yet updated with the mapping on the old S-NSSAI:

the AMF proceeds with the PDU Session establishment to the SMF as usual, then the AMF indicates the alternative S-NSSAI to the UE in UE configuration update and then requests the SMF to proceeds with the steps in bullet c) here below.

c)	For an existing PDU Session transfer from the old S-NSSAI to an alternative S-NSSAI:

-	If the UE has indicated support of the transfer of PDU sessions between network slices, the AMF notifies the current SMF, e.g. by triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be relocated to an alternative S-NSSAI and optionally indicates the alternative S-NSSAI. In Home routed case the S-NSSAIs mapping to HPLMN values for the existing and replacement S-NSSAI are provided to the H-SMF.

-	The SMF sends to the UE either PDU Session Modification Commandand indicates to the UE that the PDU Session can be re-established on another S-NSSAI. If AMF indicated the alternative S-NSSAI, the SMF provides the alternative S-NSSAI to the UE.

-	the UE triggers a new PDU Session Establishment procedure to establish a PDU Session as follows:

-

-	The PDU Session establishment request includes the alternative S-NSSAI provided by the AMF together with the old S-NSSAI (similar as to use the mapped S-NSSAI value of the VPLMN when applying the RSD matched to the HPLMN S-NSSAI).

-	(alternative to the previous bullets) If the UE didn’t receive an alternative S-NSSAI from the SMF, the UE uses the URSP to determine the S-NSSAI to use and in the process the UE does not use URSP rules with the old S-NSSAI.

# * * * * End of changes * * * *