3GPP TSG-WG SA2 Meeting #154	S2-2210715

# Toulouse, France, Nov 14th – 18th

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0079 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for HR PDU Session supporting SBO in VPLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion as in the cluase 8.1.2 of TR 23.700-48, the key issue of the is concluded as follows: AMF can get SMF Selection Subscription data with indication for HR-local traffic routing allowed during registration procedure, in order to help AMF select the proper SMF. V-SMF sends the HR PDU session establishment request to H-SMF, H-SMF authorizes the local traffic routing request for this PDU Session based on subscription data and provides HR-local traffic offloading authorization information (e.g. allowed FQDN ranges) to V-SMF. The V-SMF configures V-EASDF taking into account this authorization information. H-PCF may send roaming offload policy including traffic description information, e.g. FQDN(s), EAS IP(s), to indicate traffic which is authorized to perform local traffic routing in VPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to describe a high level procedure for the authorization of the HR PDU Session supporting the Session Breakout in VPLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusion of the study is not reflected in the normative specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ….CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

************** 1st changes (all new text) ****************

6.X	Support of the local traffic routing in VPLMN for Home Routed PDU Sesion for roaming

6.X.1	General

When roaming, the UE establishes a Home Routed Session that is capable of supporting session breakout in V-PLMN based on the subscription. In this scenario, the Home PLMN and Visited PLMN have an agreement on the support of the local traffic routing (i.e. session breakout performed by V-SMF) in VPLMN for the home routed session.

1)	During the establishment of Home Routed PDU Session, Home PLMN allows the roaming UE to access the local part of DN identified by DNN based on its HPLMN subscription.

2)	V-SMF in V-PLMN performs the session breakout procedure for the UE to access the local part of DN where the EAS in EHE is located in VPLMN. This can be triggered by the EAS discovery procedure using V-EASDF.

3)	Through a Home Routed Session, the UE can access EAS deployed in EHE in VPLMN while the UE can also access the data network in the Home PLMN.

6.X.2	Procedure

6.X.2.1	General

This procedure describe the abstract procedure for the authorization of the local traffic offloding using HR PDU Session.

6.X.2.2	HR PDU Session for supporting SBO in VPLMN

Figure 6.X.2.2-1. Procedure for HR PDU Session supporting Session BreakOut (SBO) in VPLMN

- During the registration procedure, the AMF receives the HR-SBO allowed indication per DNN/S-NSNAI from the UDM in the step 14b of the procedure in the clause 4.2.2.2.2 of TS 23.502 [3].

- During the PDU Session establishment procedure for Home-routed roaming as in clause 4.3.2.2.2 of TS 23.502 [3], if the AMF receives the HR-SBO allowed indication for requested DNN/S-NSSAI in the step 1, the AMF selects V-SMF supporting ULCL/BP with V-EASDF interaction and the AMF sends the local traffic offloading allowed indication to the V-SMF. If the V-SMF determines to request of session breakout for home routed PDU Session, V-SMF selects the V-EASDF and V-SMF sends the request for local traffic routing with the V-EASDF address to H-SMF, the H-SMF authorizes the request based on subscription data and provides VPLMN offloading policy (e.g. Session AMBR for the local DN, FQDN range, IP range) and the DNS server address of HPLMN to V-SMF.

- Based on the information received from the H-SMF, the V-SMF configures the V-EASDF with the DNS handling rules. The V-SMF configures the V-EASDF considering the offloading policy and the DNS server address of HPLMN.

- EAS (re)-discovery procedure for HR roaming scenario is perfomed.

******************** End changes ********************