3GPP TSG-SA2 Meeting #154	S2-2210766

Toulouse, France, Nov 14 - 18, 2022	(revision of S2-xxxxxxx)

Source:	Tencent, Tencent Cloud

Title:	KI#3, Evaluation of solutions

Document for:	Approval

Agenda Item:	9.19.1

Work Item / Release:	FS_XRM /Rel-18

Abstract of the contribution:

This contribution evaluates solutions for KI#3 "5GS information exposure for XR/media Enhancements" in TR 23.700-60.

1. Proposal

It is proposed to evaluate solutions for KI#3 "5GS information exposure for XR/media Enhancements" in TR 23.700-60.

*********** First Change (all text new)***********

7.x	Evaluation of Solutions for Key Issue #3

Key Issue #3 is for 5GS information exposure for XR/media Enhancements. Currently, there are 11 solutions to solve this KI. All the solutions use the same subscribing method. The summary of the solutions are listed in Table 7.x-1.

Table 7.x-1 Candidate Solutions

| Solution | Exposed information | Exposure path |
| --- | --- | --- |
| #5: Exposure of the UE data rate, normal data transmission interruption event and congestion information | Normal Data Transmission interruption event | via RAN notification: RAN->SMF->PCF->AF; or via GTP-U from RAN to UPF: RAN->UPF(API)->AF |
|  | radio congestion information |  |
|  | data rate | measured and exposure by UPF: UPF(API)->AF |
| #6: Mean bit rate change report | mean bitrate for non-GBR QoS flow | via RAN notification: RAN->SMF->PCF->AF |
|  | requested Alternative QoS Parameter Sets | from AF to NEF. |
| #41: Use of ECN bits for L4S to enable codec/rate adaptation to meet requirements for services | radio congestion information | ECN bits marking in RAN for L4S |
| #42: Exposure of round-trip delay to support XR/media enhancements | round-trip delay | QoS monitoring via UPF. |
| #43: Information Exposure to AF for XR/media Enhancements | radio congestion information | Uplink: RAN(GTP-U)->UPF(API)->AF Downlink: RAN(GTP-U)->UPF(ECN)->UE |
| #44: XRM AF driven Quick QoS Notification Control | requested AW | AF->NEF |
| #45: Information Exposure to AF for XR/media Enhancements | estimated QoS | OAM->NWDAF->NEF |
| #46: Use of ECN marking for L4S for scalable congestion control and meet requirements for services | radio congestion information | Uplink: UE or RAN or UPF supports ECN marking; Downlink: RAN or UE supports ECN marking. ECN marking in different layers in RAN may be supported. |
| #47: Delay Difference and Delay Notifications and Reports | delay difference | QoS monitoring via UPF |
|  | two way delay |  |
| #48: Enhanced Notification of Rata Adaptation Request | rate adaptation suggestion | via RAN notification: RAN->SMF->PCF->AF |
| #67: Supporting ECN mark for L4S in 5GS for XR/media Enhancements | radio congestion information | ECN marking in RAN or UPF |

In Table 7.x-1, the information exposed to AF could help the XR/media application server to enable application codec/rate adaptation.

- Non-ECN related solutions (Solutions #5, #6, #42, #44, #45, #47,and #48)

Solution#5 proposes to expose Normal Data Transmission interruption event, congestion information and data rate to the application. Regards to Normal Data Transmission interruption event and congestion information, these two information are exposed by RAN through RAN notification or GTP-U. Regards to data rate, it is measured and exposed by PSA UPF.

Solution#6 proposes to expose mean bitrate for non-GBR QoS flow through QoS notification. This solution also proposes that the AF could request Alternative QoS Parameter Sets to enable the network to provide alternative QoS parameters as references for the currently supported QoS level for the XR traffic.

Solution #42 proposes that the round trip delay can be measured and exposed by the PSA UPF. The RAN part of UL/DL packet delay measurement is as defined in clause 5.33.3 of TS 23.501. Two cases are considered. One is that the XR traffic is transmitted within a single QoS flow; The other one is that the XR traffic is transmitted within multiple QoS flows, e.g. the main UL and DL of XR traffic are separated into two flows.

Solution #44 proposes that the AF could request the average window (AW) to the NEF. The AW can determine the measurement periodicity (i.e. the rate) of the GFBR/MFBR.

Solution#45 proposes to support estimated QoS exposure to the AF. It reuses the procedure in Clause 6.2.3.2 of TS 23.288 for the NWDAF to get the information from OAM. Then the NWDAF predicts the average estimated QoS based on the information.

Solution #47 proposes that delay difference and round trip delay is measured by UPF and RAN. Regards to delay difference, AF may request to be notified when the delay difference between two QoS Flows exceeds a threshold. The delay measurement for individual QoS Flows is based on QoS monitoring in clause 5.33.3 of TS 23.501 [2].

Solution #48 proposes that the RAN may provide rate adaptation suggestion to the AF. The RAN could expose the information through RAN notification.

- ECN related solutions (Solutions #41, #43, #46 and #67)

Solution #41 and #46 exposes the radio congestion information to the application server through ECN bits marking for the purpose of L4S in RAN; while Solution #43 exposes the radio congestion information to the UE through ECN bits marking for L4S in UPF. Solution #67 could be considered as the complementary solution for ECN marking for L4S. The L4S marking is according to [37] and [62].

Solution #41 requires the RAN to support ECN bits marking for L4S of payload packets for the uplink and downlink. A QoS flow used for L4S enabled traffic is established. The criteria for the RAN to perform marking is up to RAN implementation.

Solution #46 provides different options for supporting ECN marking of payload packets for both uplink and downlink. For the uplink, either the RAN or the UE needs to support the ECN marking for L4S. The ECN marking for L4S could be set in different layers. If the ECN marking is set in PDU IP header, there will be no impact to the UPF. Other options require the UPF to transfer the ECN marks to the PDU IP header. For the downlink, the RAN needs to support ECN marking for L4S. If the ECN marking is set in PDU IP header, there will be no impact to the UE. Other options require the UE to support the ECN marking for L4S.

Solution #43 proposes that the RAN sends the congestion information to the UPF via GTP-U, and then the UPF could send the congestion information to the AF for the uplink, and send the congestion information to the UE via ECN marking in the layer 4 protocol for the downlink.

Solution #67 proposes that the AF could provide ECN marking for L4S instruction information. Based on the AF information, the PCF generates the corresponding policy, and sends it to the SMF, RAN or UPF to enable the ECN marking for L4S.

* * * * End of Changes * * * *