3GPP WG SA WG2 Meeting #154	S2-2210793

Toulouse, France, Nov 14 – 18, 2022	(revision of S2-220xxxx)

Source:	China Mobile

Title:	KI#7 Update to Conclusion

Document for:	Approval

Agenda Item:	9.21.1

Work Item / Release:	FS_AIMLsys / Rel-18

Abstract of the contribution: This contribution proposes conclusion for KI#7 on FL member selection aspect.

1.	Proposal

It is proposed to adopt the following change in the TR 23.700-80.

* * * * First Change * * * *

8	Conclusion

8.7	Key Issue #7: 5GS Assistance to Federated Learning Operation

For FL member selection aspect:

- AF should be responsible for final FL member selection. Before that, the NWDAF may generate the candidate member list and send it to AF.

- The AF can directly collect the data for FL member selection (possibly via NEF) from 5GC NFs, based on that, the FL members can be determined by the AF.

- The list of Member Selection Criteria should at least consist of user consent subscription data from UDM, UE location from AMF (optional), delay time obtained by reusing QoS Monitoring, Throughput UL/DL, Packet transmission, Packet retransmission from UPF.

* * * * End of change * * * *