SA WG2 Meeting #S2-154	S2-2210809

Toulouse, France, Nov 14 - Nov 18, 2022

Source: 	Ericsson

Title: 	KI#2: Conclusion update, addressing remaining Editor's note

Document for: 	Agreement

Agenda Item: 	9.4.1

Work Item / Release:	FS_eNPN_Ph2 / Rel-18

Abstract of the contribution: This paper proposes a conclusion update for Key Issue #2, addressing remaining Editor's note.

# Discussion

## 1.	Introduction

There is one remaining EN for KI#2:

Editor's note:	Additional conclusions related to the use of the same credentials for access to SNPN with NG-RAN and to WLAN Access Network as proposed in Solution #16 can be considered after progress in SA WG3.

An LS has been sent to SA3, asking SA3 to look into it. It is therefore not expected that SA2 will need to spend more TR study time and therefore SA2 can handle any potential need to update SA2 specifications based on any SA3 feedback.

# Proposal

Add the following conclusion to TR 23.700-08.

*** BEGIN CHANGES ***

## 8.2	Key Issue #2: Support of Non-3GPP access for SNPN

The following conclusions are agreed for normative work:

-	Access to SNPN services via Untrusted non-3GPP access network includes support for Credential Holder, UE Onboarding and Emergency services. Specifically:

-	When UE registers to SNPN with credentials owned by the SNPN, UE uses the same N3IWF selection procedure as specified for access to stand-alone non-public network services via PLMN in clause 6.3.6.2a of TS 23.501 [3].

-	UE initiates N3IWF selection for emergency services when the UE detects a user request for emergency session and determines that Untrusted non-3GPP access is to be used for the emergency access. The UE with SNPN subscription performs the following:

-	If the UE determines that it is located in the same country as the subscribed SNPN, the UE uses the configured N3IWF FQDN for N3IWF selection.

-	Otherwise, the UE follows the N3IWF selection procedure for Emergency services for UE not equipped with UICC, as defined in clause 6.3.6.4.2 of TS 23.501 [3].

-	When the UE is registering to SNPN over Untrusted N3GPP access for UE Onboarding, UE may select a N3IWF in the SNPN which supports UE Onboarding by using the pre-configured N3IWF FQDN used for onboarding.

NOTE 1:	The GIN is not used to construct a N3IWF FQDN if a UE is registering to SNPN over Untrusted N3GPP access.

NOTE 2:	The format of FQDN will be specified by CT WG4.

-	If the PVS is reachable from the local Untrusted non-3GPP access network (e.g. via the Internet) using the local IP connectivity, UE may connect directly (i.e. without connected to a N3WIF) with a PVS to obtain the SNPN credentials.

-	Access to SNPN services via Trusted non-3GPP access network is to be specified according to the principles described in Solution #3, including support for credential Holder, Emergency services and UE onboarding. Specifically:

-	ANQP is to be extended to support advertising of:

-	SNPN IDs and GINs corresponding to SNPNs with which 5G connectivity is supported and related parameters as described in clause 6.3.3.1.

-	Support for Emergency services.

-	Onboarding enabled indication.

NOTE 3:	The work on additional parameters in ANQP is to be kept internal to 3GPP.

NOTE 4:	If the UE tries to register with an SNPN via TNAN X in the case of SNPN ID with self-assigned NID, and the UE is rejected by the AMF with a cause code that temporarily prevents the UE from registering with this SNPN, the UE does temporarily not attempt to register with the same SNPN, even if the same SNPN ID is advertised via another TNAN Y.

-	When accessing SNPN services via Trusted or Untrusted non-3GPP access the UE needs to be able to construct a prioritized list of WLAN access networks by using enhanced WLAN Selection Policy (WLANSP) rules from ANDSP (currently supported only for PLMN in TS 23.402 [9]) or based on local configuration.

-	N3IWF and TNGF needs to be able to include the "selected NID" in the [NGAP] INITIAL UE MESSAGE, which is up to RAN3 to define.

-	Access to SNPN services via wireline access network and use of Credential Holder is to be supported by defining a new GCI including a "NID"

-	The NSWO procedure is to be extended to support UE authentication using SNPN credentials (applies both to SIM-based and non-SIM based credentials). It is expected that the impact is limited to the use of a SUCI format whose "realm" part enables routing of SWa requests from the WLAN AN to the NSWO in the SNPN's 5GC, which is already supported.

NOTE 3:	Additional conclusions for wireline access related to UE behind 5G-RG depends on the progress on the FS_5WWC_Ph2 study.

NOTE 4:		Functionality related to the use of the same credentials for access to SNPN with NG-RAN and to WLAN Access Network as proposed in Solution #16 can be considered during normative work based on any feedback from SA WG3.

*** END CHANGES ***